3GPP TSG-RAN WG4 Meeting#101      	R4-2117583

E-meeting, 1st – 12th Nov, 2021

Title:	TP to TR 38.717-01-01: CA_n77B

Source:	Nokia, US Cellular

Agenda item:	7.6.2

Document for:	Approval

# 1	Introduction

This contribution introduces CA_n77B into TR 38.717-01-01.

************************ Start of TP **********************************************

## 5.X	CA_2DL_n77B_1UL_n77A

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n77 | n77 |

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n77B | - | 20 | 25, 30, 40 |  |  |  | 60 | 0 |
|  |  | 25 | 30 |  |  |  |  |  |

### 5.X.2	UE maximum output power for Intra-band contiguous CA

Not needed as uplink is single CC.

### 5.X.3	UE additional maximum output power reduction for CA

Not needed as uplink is single CC.

### 5.X.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Not needed as uplink is single CC.

### 5.X.5	Reference sensitivity power level for Intra-band contiguous CA

Not needed as PCC REFSENS is same as single carrier REFSENS and DL SCC is further away from UL than DL PCC.

### 5.X.6	In-band blocking

Already specified for n77.

### 5.X.7	Out-of-band blocking

Already specified for n77.

### 5.X.8	Narrow band blocking

Not needed for n77.

************************ End of TP **********************************************