3GPP TSG-RAN WG4 Meeting # 101-e                                						R4-2117528

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-11: DC_n3A_1A

Agenda Item:	7.13

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR band n3 and LTE band1 for TR 37.717-11-11[1].

# Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.4.0

# Text Proposal

----- Start of TP -----

### 6.1.x	DC_n3_1

#### 6.1.x.1	Configuration for DC

Table 6.1.x.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n3A_1A | DC_n3A_1A | DC_n3_1 |

#### 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n3A_1A | 23 | +2/-3 |

#### 6.1.x.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n3A_1A are the same as DC_1A _n3A, which have been already defined in the TS38.101-3 spec.

#### 6.1.x.4	MSD analysis for DC

The MSD analysis for DC_n3A_1A are the same as DC_1A _n3A, as specified in 6.1.37.5 of3GPP TR 37.716-11-11 V2.0.0.

#### 6.1.x.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n3A_1A are the same as DC_1A _n3A, which have already been defined in the TS38.101-3 spec.

#### 6.1.x.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, The MSD test points for DC_n3A_1A are the same as DC_1A _n3A, as specified in Table 7.3B.2.3.5.1-1 of TS38.101-3 spec.

#### 6.1.x.7	OOB blocking exception requirements

No OOB blocking exception for DC_ DC_n3A_1A.

----- End of TP -----