3GPP TSG-RAN WG4 Meeting #101-e              	R4-2117594

Electrical Meeting, 1st Nov-12th Nov, 2021

Agenda item:	8.9.5.3.1

Source:	Samsung

Title:	Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters on PUSCH requirement for Rel-17 FR2 HST were further discussed. The related agreement was captured in the WF [1] as

- PUSCH requirement for Uni/Bi-directional RRH scenarios in scenario A and B - No dedicated PUSCH requirement in Bi-directional for Scenario A - Introduce PUSCH requirement in Uni-directional for Scenario A if the feasibility of Uni-directional deployment is confirmed - Introduce PUSCH requirement in Uni-directional and Bi-directional for Scenario B - Further discuss the following aspects - Introduce of test applicability rule if needed - Introduction of BS declaration for applicable test case if more than one will be introduced with different deployment scenario - FFS whether a single requirement/test case can be made to cover both Uni-directional and Bi-directional deployments of Scenario-B and even Scenario-A - Companies can provide performance comparison among Uni-directional and Bi-directional deployment - BS test setup feasibility for Bi-directional deployment with two panels - RS configuration - Option 1: 1 DMRS +PT-RS (L=1, K=2) and 2 DMRS+PT-RS (L=1, K=2) with test applicability rule, based on BS manufacturer declaration - Option 2: 2 DMRS +PT-RS (L=1, K=2) and 3 DMRS +PT-RS (L=1, K=2) with test applicability rule, based on BS manufacturer declaration - Option 3: 1DMRS +PT-RS (L=1.K=2) and 3 DMRS+PT-RS (L=1, K=2) with test applicability rule based on BS manufacturer declaration - Companies are encouraged to provide performance comparison between different RS configuration in the next meeting - MCS - Option 1: MCS16 - Option 2: MCS17 - Option 3: MCS2 - Encourage companies bring the simulation results for MCS16, MCS17 and MCS20 in the next meeting - Decide whether to define MCS16, MCS17 and MCS 20 based on the simulation results - Frequency offset compensation implementation - Option 1: Considering only pre-FFT frequency offset compensation for FR2 PUSCH requirement - Option 2: FOE method is up to BS implementation - Chose the worst case for requirement definition - Encourage companies to bring the simulation results for MCS16,MCS17 and MCS20 in the next meeting - Decide whether to define the worst case for requirement definition - CBW - Define 50MHz and 200MHz CBWs with test applicability rule that only one of them is tested based on BS manufacturer - Length of PUSCH data symbol: 10

In this contribution, the view on remaining issue are provided for PUSCH requirement. Meanwhile, the initial simulation results are provided to investigate the impact of different RS configuration, MCS level under high Doppler Frequency value.

# 2	Discussion

RS configuration

In FR2, to enable 350km/h maximum speed, the PTRS should be configured. Based on maximum frequency offset tracking capacity, PTRS has small distance between adjacent RS compared with DMRS. Therefore, the number of DMRS is not the bottleneck of frequency tracking for FR2.

Based on the agreement, only single tap channel is considered for FR2 HST WI. Different with fading channel, there is no necessary to configure more DMRS to overcome the fading impact.

Meanwhile, beside on the Doppler impact for HST in FR2, the phase noise impact should be considered. With configured more PTRS symbols can be beneficial for phase noise estimation, since the phase noise impact can change per symbol depending on the nonlinearity of PA in the real product.

Again, as mentioned in the WID, the train-mounted UE is a decided CPE for HST scenario in FR2. As agreed, RAN4 requirement can be defined based on the baseline of 1 CPE device per train. In that sense, less UE can be served by RRH, where it is not an interference limited scenario or resource limited scenario, different for FR1, more UE can access the network for uplink. Therefore, more resources can be allocated for uplink to improve the uplink throughput in FR2.  With more DMRS symbols, the overhead of RS is higher than 1 DMRS+PTRS. The following is overhead analysis based on different RS configurations.

Table 2-1: Overhead of RS with different RS configurations

| RS configuration | BW (MHz) | Position of DMRS | Overhead of DMRS (length of data is 10, number of CDM group =2 Mapping type B | Overhead of PTRS (length of data is 10, number of CDM group=2, mapping type B) | Total |
| --- | --- | --- | --- | --- | --- |
| 1 DMRS +PTRS (L=1,K=2) | 200 | {0} | 792/15840 | 594/15840 | 1386/15840 |
| 2DMRS + PTRS (L=1, K=2) | 200 | {0,8} | 1584/15840 | 528/15840 | 2112/15840 |
| 3DMRS+ PTRS (L=1, K=2) | 200 | {0,4,8} | 2376/15840 | 462/15840 | 2838/15840 |
| 1 DMRS +PTRS (L=1,K=2) | 50 | {0} | 192/3840 | 144/3840 | 336/3840 |
| 2DMRS + PTRS (L=1, K=2) | 50 | {0,8} | 384/3840 | 128/3840 | 512/3840 |
| 3DMRS+ PTRS (L=1, K=2) | 50 | {0,4,8} | 576/3840 | 112/3840 | 688/3840 |

Observation 1:  The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Also, based on simulation results as indicated section 3, with configured 1 DMRS+PTRS, the better throughput can be achieved.

Meanwhile, similar SNR can be achieved for 2DMRS and 3DMRS in terms of normalization throughput. Based on UL Link budget analysis, the achievable SNR is higher, so, it is not a practical scenario that 3 DMRS will be configured by Network.

In Rel-15 NR FR2, both 1 DMRS and 2 DMRS configuration were considered for PUSCH requirement. Meanwhile, the test applicability rule was defined to reduce the test effort based on the BS manufacturer declaration, in case that only one RS configuration can be supported by BS vendors. Based on Rel-15 discussion, it seems that all the interested infrastructure vendor can support at least 1 DMRS or 2 DMRS configurations.

As analyzed, from Doppler estimation accuracy and achieved throughput perspective, the benefit with more than 2 DMRS configuration is limited.

Regarding the RS configuration, RAN4 has several meeting to discuss it, there is no consensus among companies. To moving forward, we propose to define PUSCH requirement with both 1 DMRS and 2 DMRS configuration, with applying the same test applicability rule as Rel-15.

Proposal 1: Define PUSCH demodulation requirement with both 1 DMRS +PT-RS (L=1, K=2) and 2 DMRS +PT-RS (L=1, K=2) configurations. Apply the same test applicability rule for BS conformance test, based on BS manufacturer declaration.

FOE implementation

Regarding the FOE implementation, RAN4 has discussed two kinds of FO method, both pre- FFT frequency offset compensation and Post-FFT frequency offset compensation. Generally, we agree that both two kinds of FO method can be possible for BS implementation. While whether to apply pre or post-FFT frequency offset compensation is up to BS implementation.

Pre-FFT operation is benefit for the scenario, where less UE can access the network. Post-FFT is the typical implementation for UL, considering it is not feasible to do the pre-FFT frequency offset pre UE. This method will cause ICI with large Frequency offset value. Although the requirement is targeting for HST scenario, the common UEs around RRH can still access the BS in the practical deployment. In that sense, post-FFT frequency offset compensation should be the typical implementation for UL

In terms of RAN4 requirement, in our understanding, there is no restriction for them, similar as channel estimation, and TO/FO estimation, only the practical estimation is indicated in the simulation assumption for requirement. Therefore, we propose not to mandate the FOE implementation method, which is up to BS implementation. Meanwhile, RAN4 is to define the minimum requirement, the final requirement can be determined based on the worse cases, if large performance gap is existed.

Proposal 2:  FOE method is up to BS implementation. Choose the worst case for requirement definition.

MCS

Regarding MCS, in FR1 HST, MCS 16 is specified for PUSCH requirement. We can use it as a starting point. As company’s contribution mentioned, UL demodulation performance degradation due to post-FFT frequency offset compensation, defined two sets of requirements is suggested to distinguish different implementation.

For non-HST scenario, up to 64QAM are considered for PUSCH requirement. For HST scenario, as mentioned by company in the last meeting, considering the roof-mounted CPE is dedicated UE for HST network, the better signal strength can be available. Therefore, it may have some benefit with high modulation. On the other hand, for high MCS, the achievable SNR is high, it may be higher than 20 dB. We are open to further discuss whether high modulation requirement will be considered, such as MCS 17, if no OTA testability issue is identified.  If there is no consensus to define one sets of MCS for PUSCH, we are fine to define both MCS 16 and MCS17 for PUSCH requirement.

Proposal 3:  Define PUSCH requirement with MCS 17, if no OTA testability issue.

Proposal 4: Both MCS 16 and MCS 17 can be considered for FR2 HST PUSCH requirement.

In the section3, the initial simulation result with different MCS level are provided.

# 3	Initial Simulation Results

In this subsection, the initial simulation results for scenario B under bi-directional, and scenario A/B under Uni-directional are provided.  Also, the performance with different MCS level are investigated to check the impact on high modulation order due to the Doppler frequency.

Table 3-1 Simulation Assumption for PUSCH requirement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna layout |  | 1T2R, ULA Low Correlation |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Option1: 1+0 Option2: 1+1 Option3: 1+1+1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
|  | Bandwidth | 50MHz and 200MHz |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | KPT-RS=2 |
|  | Time density (LPT-RS) | LPT-RS = 1 |
| MCS |  | MCS16, MCS17 or MCS 20 (64 QAM table) |
| Propagation channel |  | Bi-directional with sceanro B Uni-directional with scenario A/B |

The following table is provided the initial results with MCS 16, MCS 17 and MCS20 are considered for different DMRS configuration

Table 3-2:  Initial results for different MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m) (To be updated)

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+0 | 9.7 |
| Case 2 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+1 (0,8) | 9.5 |
| Case 3 | 1T2R | 120KHz 200MHz | B | 10 | 16 | 1+1+1(0.4.8) | 9.5 |
| Case 4 | 1T2R | 120KHz 200MHz | B | 10 | 17 | 1+0 | 11.2 |
| Case 5 | 1T2R | 120KHz, 200MHz | B | 10 | 17 | 1+1 (0.8) | 10.7 |
| Case 6 | 1T2R | 120KHz, 200MHz | B | 10 | 17 | 1+1 (0,4,8) | 10.9 |
| Case 7 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+0 | 17.7 |
| Case 8 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1(0,8) | 17.2 |
| Case 9 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1+1(0,4,8) | 17.5 |

Table 3-3:  Initial results for different MCS level for scenario-A with Uni-directional deployment scenario (Ds=700m, Dmin=10m, Ds_offset=10m) (To be updated)

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case1 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+0 | 15.3 |
| Case2 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+1 (0,8) | 14.7 |
| Case3 | 1T2R | 120KHz 200MHz | B | 10 | 16 | 1+1+1(0,4,8) | 14.7 |
| Case4 | 1T2R | 120KHz, 200MHz | B | 10 | 17 | 1+0 | 23.7 |
| Case5 | 1T2R | 120KHz, 200MHz | B | 10 | 17 | 1+1 (0,8) | 19.5 |
| Case6 | 1T2R | 120KHz 200MHz | B | 10 | 17 | 1+1+1(0,4,8) | 21.7 |
| Case7 | 1T2R | 120KHz, 200MHz | B | 10 | 20 | 1+0 |  |
| Case8 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1 (0,8) |  |
| Case9 | 1T2R | 120KHz, 200MHz | B | 10 | 20 | 1+1+1(0,4,8) |  |

Table 3-4:  Initial results for different MCS level for scenario-B with Uni-directional deployment scenario (Ds=700m, Dmin=150m, Ds_offset=100m) (To be updated)

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case1 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+0 | 13.1 |
| Case2 | 1T2R | 120KHz, 200MHz | B | 10 | 16 | 1+1 (0,8) | 12.7 |
| Case3 | 1T2R | 120KHz 200MHz | B | 10 | 16 | 1+1+1(0,4,8) | 12.7 |
| Case4 | 1T2R | 120KHz, 200MHz | B | 10 | 17 | 1+0 | 16.2 |
| Case5 | 1T2R | 120KHz, 200MHz | B | 10 | 17 | 1+1 (0,8) | 15.1 |
| Case6 | 1T2R | 120KHz 200MHz | B | 10 | 17 | 1+1+1(0,4,8) | 15.7 |
| Case7 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+0 |  |
| Case8 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1(0,8) |  |
| Case9 | 1T2R | 120KHz 200MHz | B | 10 | 20 | 1+1+1(0,4,8) |  |

# 4	Conclusion

In this contribution, the view on the PUSCH requirement for Rel-17 FR2 HST is provided. Meanwhile, the initial simulation results with different MCS level are provided to check the feasibility of high modulation order in HST scenario.

Observation 1:  The overhead of 1DMRS +PTRS (L=1, K=2) configuration is the smallest compared with other RS configuration schemes.

Observation 2: similar performance can be achieved with 2 DMRS and 3 DMRS configuration

Observation 3: Better performance can be achieved for bi-directional scenario in scenario B compared with Uni-directional scenario

Proposal 1: Define PUSCH demodulation requirement with both 1 DMRS +PT-RS (L=1, K=2) and 2 DMRS +PT-RS (L=1, K=2) configurations. Apply the same test applicability rule for BS conformance test, based on BS manufacturer declaration.

Proposal 2:  FOE method is up to BS implementation. Choose the worst case for requirement definition.

Proposal 3:  Define PUSCH requirement with MCS 17, if no OTA testability issue.

Proposal 4: Both MCS 16 and MCS 17 can be considered for FR2 HST PUSCH requirement.

# References

R4-2115726, WF on FR2 HST Demodulation, Samsung