3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117623

# Electronic Meeting, November 01-12, 2021

Agenda item:	8.24.2

Source:	Qualcomm Inc.

Title:	On NR SL relay RRM Requirement Scope

Document for:	Discussion

# Introduction

In this contribution, we present our view on the open issues of R17 NR SL relay RRM requirement from [1].

# Discussion

## Relay (re)Selection Procedure

RAN2 made the following agreement in the previous meeting

- As a baseline, SL-RSRP of the serving relay is used as the SL measurement quantity for the case of path switch from indirect to direct path.​

- SD-RSRP is used as the SL measurement quantity for path switch from direct to indirect path.

The L1 SL-RSRP measurement accuracy requirement in R16 is:

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

These requirements align with the LTE SD-RSRP accuracy requirement except lower Es/Iot side condition. We propose the following:

Proposal 1: Reuse L1 SL-RSRP measurement accuracy requirement and side condition for both SL-RSRP in indirect to direct switch, and SD-RSRP direct to indirect switch.

Since the discovery signal is in a dedicated resource pool or with SPS transmission in certain periods, we can use the period of the dedicated resource pool or SPS transmission period as the discovery period. For the indirect to direct path switch, which measures SL-RSRP from data instead of the discovery signal, at least the discovery signal is available every discovery period for measurement if the data is sparser than the discovery signal, and the discovery period can apply to this case.

Proposal 2: Use the period of the discover signal as the discovery period.

Since the use cases is similar to LTE ProSe relay, we propose to reuse measurement period requirement from LTE ProSe:

| Discovery Period [s] | Tmeasure,ProSe_Relay_Intra [s] (number of discovery periods) | Tevaluate, ProSe_Relay_intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| NOTE 1: Time depends upon the configured Discovery period. |  |  |

Since the 1 slot measurement can already fulfill the accuracy requirement, we don’t see any issue for reusing LTE requirement, which allows 4 discovery periods for measurement.

Proposal 3: Reuse LTE ProSe relay measurement requirement.

## Cell reselection requirement for NR SL discovery on non-serving carrier

The scenario for cell reselection is explained in the following:

Uu is on carrier frequency F1, but SL timing is from carrier frequency F2, and SIB18 provides the information of F2 on F1. The requirement applies when UE performs cell reselection from serving cell on carrier frequency F1 to neighboring cell on carrier frequency F2. If SL timing is from carrier frequency F1, it is legacy cell reselection.

This scenario applies to relay UE but not remote UE. RAN2 didn’t define any specific procedures for relay UE cell reselection and handover, i.e., the legacy procedures should apply. In fact, this LTE requirement is the same as legacy intra-frequency cell reselection requirement. Therefore, we don’t see the need to define an additional requirement as above for a SL relay UE specific scenario.

Proposal 4: Do not define an additional requirement for a SL relay UE specific scenario, i.e., NR SL discovery on non-serving carrier.

## Interruptions from SL to Uu

In NR R16 SL scenarios agreed in RAN4, SL and Uu are on different bands with separate RF chain and baseband resources. Therefore, the discovery signal reception and data communication interruptions in LTE ProSe incoverage scenarios are not applicable to NR SL relay unless we consider scenarios beyond R16 SL.

Proposal 5: The discovery signal reception and data communication interruptions in LTE ProSe incoverage scenarios are not applicable to NR SL relay when we consider RAN4 R16 SL scenarios.

R16 V2X requirement specifies the SL communication configuration interruption on Uu for general SL communication, and it is applicable to SL relay discovery and communication configurations.

Proposal 6: R16 V2X requirement specifies the SL communication configuration interruption on Uu for general SL communication, and it is applicable to SL relay discovery and communication configurations.

## RRM requirement scope

Since RAN2 excluded SL relay from SL DRx enhancement discussion, SL relay requirement should be applicable only to UEs without SL DRx.

Proposal 7: SL relay requirement should be applicable only to UEs without SL DRx.

When relay and remote UEs are on different synchronization sources, they can still communicate as long as the timing offset is not too large to cause radio link failure under R16 V2X framework. Since UE has to handle different propagation delays in multiple SL communications, even all the nodes are with the same timing, we don’t see the need to introduce specific requirements or tests for the relay and remote UEs with different sync resources. However, remote UE and relay UE can communicate if the timing difference is small enough to maintain communication.

Proposal 8: Do not introduce specific requirements or tests for the relay and remote UEs with different sync resources.

# Conclusions

Proposal 1: Reuse L1 SL-RSRP measurement accuracy requirement and side condition for both SL-RSRP in indirect to direct switch, and SD-RSRP direct to indirect switch.

Proposal 2: Use the period of the discover signal as the discovery period.

Proposal 3: Reuse LTE ProSe relay measurement requirement.

Proposal 4: Do not define an additional requirement for a SL relay UE specific scenario, i.e., NR SL discovery on non-serving carrier.

Proposal 5: The discovery signal reception and data communication interruptions in LTE ProSe incoverage scenarios are not applicable to NR SL relay when we consider RAN4 R16 SL scenarios.

Proposal 6: R16 V2X requirement specifies the SL communication configuration interruption on Uu for general SL communication, and it is applicable to SL relay discovery and communication configurations.

Proposal 7: SL relay requirement should be applicable only to UEs without SL DRx.

Proposal 8: Do not introduce specific requirements or tests for the relay and remote UEs with different sync resources.

# Reference

[1] R4-2115373