3GPP TSG-RAN WG4 Meeting #101-e	R4-2117645

1 –12 November 2021

Source:	Qualcomm Incorporated

Title:	Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP

Agenda item:	7.24.2

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN4#98bis-e new inter-band V2X combinations V2X_n79A-n47A and V2X_n79A-47A were introduced [1,2] and in RAN4#99-e MSDs for n47 and B47 were agreed for these V2X combinations [3]. However, MSD for band n79 has yet to be decided. In this paper we present the justification for the required MSD for n79 for these V2X band combinations.

# 2. 	Discussion

In [1,2] the inter-band combinations V2X_n79A-n47A and V2X_n79A-47A were introduced. In RAN4#100-e there was no agreement on the MSD value for n79 for these band combinations. MSD values ranging from 1 dB to 3.3 dB were presented [4,5]. The calculations noted in [5] are based on commercially available parts and lab measurements. Also, the MSD value only governs the maximum possible relaxation. If a UE implementation is able to operate well with less MSD then it is permitted to do so.

Observation1: MSD values ranging from 1 dB to 3.3 dB were suggested for n79 in RAN4#100-e

Observation2: It is noted that UEs need not operate with the maximum specified MSD. They can use the lower MSD values that enables the specifications to be met.

We see no harm in specifying the maximum MSD value from those suggested in RAN4#100-e of 3.3 dB for n79 for these V2X combinations as it in no way harms those UEs that can operate well with less MSD.

Observation3: Specifying the maximum MSD from the range discussed in RAN4#100-e in no way harms any UEs that can use lower MSDs. On the contrary it permits a broader range of UEs to meet the required specifications

Therefore, we make the following proposals

Proposal 1: The MSD value of 3.3 dB be adopted for band n79 in V2X_n79A-47A

Proposal 2: The MSD value of 3.3 dB be adopted for band n79 V2X_n79A-n47A

# Conclusion

We present reasons for why the MSD value of 3.3 dB should be adopted for n79. We make the following observations and proposals:

Observation2: MSD values ranging from 1 dB to 3.3 dB were suggested for n79 in RAN4#100-e

Observation2: It is noted that UEs need not operate with the maximum specified MSD. They can use the lower MSD values that enables the specifications to be met.

Observation3: Specifying the maximum MSD from the range discussed in RAN4#100-e in no way harms any UEs that can use lower MSDs. On the contrary it permits a broader range of UEs to meet the required specifications

Proposal 1: The MSD value of 3.3 dB be adopted for band n79 in V2X_n79A-47A

Proposal 2: The MSD value of 3.3 dB be adopted for band n79 V2X_n79A-n47A

# Annex - TP for TR 37.875 on MSD for n79 for V2X_n79A-n47A and V2X_n79A_47A

## --Start of changes1

#### 6.2.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below. The MSD values given in tables 6.2.4.1-1and 6.2.4.1-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.2.4.2-1

Table 6.2.4.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.2.4.1-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of Uu band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

## --End of changes1

## --Start of changes2

#### 6.3.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below. The MSD values given in tables 6.3.4.4-1 and 6.3.4.4-2 shall apply for all n79 and 47 BWs and SCSs combinations given in table 6.3.3.2-1

Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X

| UL band | DL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | 47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.3.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of Uu band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

--End of changes2

# References

[1] R4-2105367, CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A, RAN4#98-bis, Apr. 12-20 2021

[2] R4-2105368, CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A, RAN4#98-bis, Apr. 12-20 2021

[3] R4-2107815, Discussion and TP for TR 38.375 on SMD for V2X_n79A-n47A and V2X_n79A_47A, RAN#99-e, May 19-28 2021

[4] R4-2113420, Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A-47A, RAN4#100-e, August 16 -27 , 2021

[5] R4-2112285, Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP, RAN4#100-e, August 16 -27 , 2021