3GPP TSG RAN WG4 Meeting #101-e	R4-2117654

Online, 1st – 12th Nov. 2021

Agenda item:	7.14.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-21-11: EN-DC_1-11_n79

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-11_n79 was approved in RAN#93[1]. This TP is to capture the basic aspects for the EN-DC. The MSD exception value needs to be updated to complete the combo.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 2-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-11A_n79A5 | DC_1A_n79A DC_11A_n79A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

3.  Co-existence

When Uplink EN-DC configuration is DC_1A_n79A, (1) 4th IMD of (B1 - n79) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n79A, (2) 3rd IMD of (B11 - n79) will fall into Rx band of Band 1.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B11 | B1+n79 | B11+n79 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0 |  | 0.3 |
| B11 | 0.3 |  | 0 | 0.3 |
| n79 |  | 0 | 0 | 0 |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B11 | B1+n79 | B11+n79 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B11 | 0 |  | 0 | 0 |
| n79 |  | 0 | 0 | 0 |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n79 | 1 | 0.3 |
|  | 11 | 0.3 |
|  | n79 | 0 |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n79 | 1 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

5.  Reference sensitivity exceptions

As mentioned in section 3, REFSENS exceptions due to IMD3 and IMD4 are expected. The following values are proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-11A_n79A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |
|  | n79 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |
|  | 11 | TBD | TBD | TBD | TBD | TBD | FDD | IMD4 |
| DC_1A-11A_n79A | 11 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |
|  | n79 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |
|  | 1 | TBD | TBD | TBD | TBD | TBD | FDD | IMD3 |

6.  Conclusion

This paper proposes study results of EN-DC 1-11_n79.

TP is shown in Annex.

7.  Reference

[1]  RP-212174  WID revision: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL); rapporteur: Huawei

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_1-11_n79

### 5.x.1	Configurations for DC_1-11_n79

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-11A_n79A5 | DC_1A_n79A DC_11A_n79A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n79A, (1) 4th IMD of (B1 - n79) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n79A, (2) 3rd IMD of (B11 - n79) will fall into Rx band of Band 1.

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n79 | 1 | 0.3 |
|  | 11 | 0.3 |
|  | n79 | 0 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n79 | 1 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B1 and n79 to Band11 Rx and IMD3 of B11 and n79 to Band1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-11A_n79A | 1 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |
|  | n79 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |
|  | 11 | TBD | TBD | TBD | TBD | TBD | FDD | IMD4 |
| DC_1A-11A_n79A | 11 | TBD | TBD | TBD | TBD | N/A | FDD | N/A |
|  | n79 | TBD | TBD | TBD | TBD | N/A | TDD | N/A |
|  | 1 | TBD | TBD | TBD | TBD | TBD | FDD | IMD3 |

[Unchanged Parts Skipped]