3GPP TSG-RAN WG4 Meeting # 101-e										R4-2117728

Electronic Meeting, November 01-12, 2021

Agenda item:	8.5.3.3

Source: 	CMCC

Title: 	Discussion on other RF radiated requirements for NR repeater

Document for:	Discussion

1. Introduction

In last RAN4 #100 e-meeting, it is approved radiated RF requirement follows the same principle of conduct requirement definition. In this contribution, we focus on EVM, ACRR and out of band gain related conducted and radiated requirements.

2. Discussion

## 2.1 EVM for FR1 and FR2

The main agreement of EVM in last meeting is listed as below:

Define following EVM levels linked to different modulation scheme and repeater declare which EVM level is supported. - EVM level linked to 256QAM - FFS: EVM level linked to low data rate e.g. QPSK - EVM level linked to 64QAM RAN4 will further discuss how to specify EVM into specification

The key point is all above three EVM requirements are not mandatory for all repeaters and repeater could declare which EVM requirement is supported. About EVM declaration, EVM limit declaration and modulation scheme declaration are two options for further discussion.

Based on our testing, although EVM limits caused by repeater itself are not totally same for different modulation schemes, the difference is almost negligible.

Table 1 EVM testing results

| Modulation scheme | Signal source EVM | EVM after repeater | comment |
| --- | --- | --- | --- |
| 64QAM | 0.73% | 1.53% | Output power equals to 20dBm |
| 256QAM | 0.75% | 1.58% | Output power equals to 20dBm and the gain is also the same as 64QAM |

Observation 1: the EVM limit caused by repeater itself is almost irrelated to modulation scheme.

In fact, repeater itself doesn’t differentiate modulation scheme and modulation scheme is only used by signal source for conformance testing. For repeater, it is suggested to just list all three EVM limits into the spec and leave it to repeater to declare which EVM limits is supported.

Proposal 1: the EVM limits for repeater are listed as below for both FR1 and FR2

| EVM limit level | [17.5%], 8%, 3.5% |
| --- | --- |
| Note: all three EVM limits are not mandatory for all repeaters and repeater could declare which EVM limits are supported |  |

To simplify testing, the same testing signal for NR is suggested to be reused. If so, modulation scheme should be defined for repeater testing. The best way is to use corresponding modulation scheme based on the chosen EVM limit that repeater support, i.e. 256 QAM for 3.5% EVM, 64QAM for 8% EVM and QPSK for 17.5% EVM.

Proposal 2: the modulation scheme for repeater testing is the modulation scheme corresponding to EVM limit that repeater support for both FR1 and FR2.

## 2.2 Out of band gain for FR1 and FR2

The main interference mechanisms to define out of band gain for both FR1 and FR2 include

1) amplification of unwanted emission from other sources including donor BS or other transmitter using the same frequency range as repeater

2) re-amplification and distortion of other operators’ wanted carrier which are outside passband

3) amplification of thermal noise

4) emissions generated by repeater itself

2.2.1 The criteria of out-of-band gain definition:

Since repeater is also regarded as one kind of network node, unwanted emission from its output port should follow the same value as gNB. This means the sum of above four kinds of interference should be less than unwanted emission limit of gNB. However, above four kinds of interference already include the unwanted emission from repeater itself which in general will also follow the same unwanted emission requirements as gNB. It seems impossible to guarantee total unwanted emission at repeater’s output port follows gNB’s requirements.

One candidate criteria to define out of band gain is to ensure all the unwanted power except for the emission generated by repeater itself meet the same unwanted emission requirement as gNB. Victim node regards all the unwanted emission coming from two co-located network node, one from repeater and the other from another network node. But in fact, the unwanted emission coming from repeater itself and other network nodes through repeater’s amplification.

Proposal 3: Out-of-band gain requirement is defined in such a way that the sum of outside-passband unwanted power is less than gNB unwanted emission limits + 3dB for both FR1 and FR2.

2.2.2 interference source assumption

For repeater, the main interference source is local around base stations. Simulation results in TR 25.956 show outside-passband interference are similar regardless of distance between aggressor gNB and repeater. Therefore, we could assume the interference source has the same output power as donor gNB and same distance between repeater and donor gNB.

Proposal 4: interferer source is assumed with the same power and same distance as donor BS when define out-of-band gain requirements for both FR1 and FR2.

2.2.3 calculation of out of band gain

Table in annex shows the parameters for calculating out of band gain. Out of band gain should be calculated as below:

Amplification of unwanted emission from interference source + amplification of wanted signal from interference source using outside passband frequency range + amplification of thermal noise < OBUE requirements of gNB

Following table shows result of out-of-band gain related to unwanted emission frequency offset range and pathloss between repeater and interference source for FR1.

Table 2. out-of-band gain for category B WA for NR bands above 1GHz

| PL | 0-5MHz frequency offset | 5-10MHz offset | 10~fmax offset |
| --- | --- | --- | --- |
| 70dB | 50 | 43 | 32 |
| 100dB | 80 | 73 | 62 |
| 130dB | 109 | 102 | 91 |

Table 3. out-of-band gain for MR

| PL | 0-5MHz frequency offset | 5-10MHz offset | 10~fmax offset |
| --- | --- | --- | --- |
| 53dB | 30 | 23 | 20 |
| 83dB | 60 | 53 | 50 |
| 100dB | 77 | 70 | 67 |

Table 4. out-of-band gain for LA

| PL | 0-5MHz frequency offset | 5-10MHz offset | 10~fmax offset |
| --- | --- | --- | --- |
| 45 | 21 | 14 | 14 |
| 75dB | 51 | 44 | 44 |
| 100dB | 76 | 69 | 69 |

Analysis shows pathloss between repeater and gNB is essential for out of band gain calculation. For repeater it may deployed at cell edge or at cell center due to large penetration loss of building. For WA, 100dB is one suitable assumption since it could represent both the pathloss of cell edge(100dB) and the sum of MCL(70dB) and severe penetration loss (30dB). For MR, 83dB maybe one suitable assumption while for LA 75dB maybe one suitable assumption.

Proposal 5: out of band gain for WA, MR and LA is suggested as in table 2,3,4 with 100dB, 83dB and 75dB PL assumption respectively for FR1.

## 2.3 ACRR for FR1 and FR2

The same criteria to define out of band gain requirement also apply for ACRR and we have following assumption

Table 5 ACRR related to PL for WA

| PL | WA |
| --- | --- |
| 70dB | 25 |
| 100dB | 55 |
| 130dB | 84 |

Table 6 ACRR related to PL for MR

| PL | MR |
| --- | --- |
| 53dB | 8 |
| 83dB | 38 |
| 100dB | 55 |

Table 7 ACRR related to PL for LA

| PL | LA |
| --- | --- |
| 45dB | 0 |
| 75dB | 30 |
| 100dB | 55 |

Proposal 6: ACRR requirement is suggested as 55dB, 38dB and 30dB for WA, MR and LA respectively with 100dB, 73dB and 65dB PL assumption respectively for FR1 .Proposal 7: for FR2 before defining out-of-band gain and ACLR requirement it is suggested to align the assumption for pathloss between repeater and interference source, beamforming gain toward repeater of interference source at first.

3. Conclusions

In this contribution, other radiated requirements except for power and emissions are discussed with following observation and proposals:

Observation 1: the EVM limit caused by repeater itself is almost irrelated to modulation scheme.

Proposal 1: the EVM limits for repeater are listed as below for both FR1 and FR2

| EVM limit level | [17.5%], 8%, 3.5% |
| --- | --- |
| Note: all three EVM limits are not mandatory for all repeaters and repeater could declare which EVM limits are supported |  |

Proposal 2: the modulation scheme for repeater testing is the modulation scheme corresponding to EVM limit that repeater support for both FR1 and FR2.

Proposal 3: Out-of-band gain requirement is defined in such a way that the sum of outside-passband unwanted power is less than gNB unwanted emission limits + 3dB for both FR1 and FR2.

Proposal 4: interferer source is assumed with the same power and same distance as donor BS when define out-of-band gain requirements for both FR1 and FR2.

Proposal 5: out of band gain for WA, MR and LA is suggested as in table 2,3,4 with 100dB, 83dB and 75dB PL assumption respectively for FR1.

Proposal 6: ACRR requirement is suggested as 55dB, 38dB and 30dB for WA, MR and LA respectively with 100dB, 73dB and 65dB PL assumption respectively for FR1.

Proposal 7: for FR2 before defining out-of-band gain and ACLR requirement it is suggested to align the assumption for pathloss between repeater and interference source, beamforming gain toward repeater of interference source at first.

# 4. Annex

Table parameters to calculate out of band gain

| RF metric | limit | comment |
| --- | --- | --- |
| DL output power from interference source A | 43dBm/100MHz for WA 38dBm/100MHz for MR 24dBm/100MHz for LA | Interference mechanism is the amplification and distortion of wanted signal from interference source A which using outside passband frequency range |
| Pathloss between repeater and interference source A | PL |  |
| DL unwanted emission from interference source B | Refer to 38.104 | Interference mechanism is the amplification and distortion of unwanted emission from interference source B which using inside passband frequency range |
| Pathloss between repeater and interference source B | PL |  |
| Thermal noise | -174dBm/Hz + corresponding out-of-band gain | Interference mechanism is the amplification of thermal noise out-of-side passband |
| The interference source A and B are both assumed have the same output power as repeater for each class. |  |  |

5. Reference

[1] R4-2115721, WF on other conducted requirements, CMCC