3GPP TSG-RAN WG4 Meeting # 101-e										R4-2117729

Electronic Meeting, November 01-12, 2021

Agenda item:	8.5.3.2

Source: 	CMCC

Title: 	Discussion on repeater emission related radiated requirements

Document for:	Discussion

1. Introduction

In last RAN4 #100 e-meeting, a new WF on NR repeaters RF requirement is approved with following agreements

1)	Spurious emissions will be the same as the BS spurious emissions

2)	Additional spurious emissions will be the same as the BS additional spurious emissions

3)	EESS protection the same as the UE requirements will be added without the NS concept.

About ACLR – this is being discussed for conducted, the same principle will be used for radiated although FR2 values will be different and can be further discussed once the principles are agreed (for conducted)

In this contribution, we focus to discuss repeater emission requirements only for FR2.

2. Discussion

As discussed in conducted contribution, we have following observation:

Observation 1: In-passband ACLR requirement matters only when no wanted signal is transmitting at adjacent channel.

The tables in annex shows that in some cases when repeater’s amplification gain is relatively less, e.g. less than 94dB for WA, adjacent channel emission for ACLR requirement would be higher than the amplified noise floor, making in-passband ACLR non-negligible. As stated in previous spec, maximum gain is assumed as 90dB. If so, at least for WA, in-passband adjacent channel emission is higher than amplified noise floor. This same principle seems also apply for both LA and MR because in most cases the gain is less than 87dB for such two classes.

Observation 2: for WA, MR and LA, adjacent channel emission inside passband is larger than amplified noise floor with the same ACLR assumption as gNB spec.

The main factors contributing to in-passband ACLR comes from original output ACLR from signal source and non-linearity caused by repeater itself. It’s hard to require repeater have the same ACLR requirement as gNB, i.e. 28dB because it doesn’t have digital filter. There is a trade-off between interference and cost. Therefore, it is suggested to leave this requirement for manufacture. Maybe for repeater not sensitive to cost, more stringent ACLR requirement could be achievable whist for repeater sensitive to cost, relax ACLR requirement may be acceptable.

Proposal 1: it is suggested that in-passband ACLR is based on manufacturer’s declaration to trade off cost against RF requirement.

For outside ACLR requirement, it is suggested to reuse the same requirement as gNB spec to achieve the same adjacent channel protection as NR spec.

Proposal 2: the same ACLR requirement as gNB still apply for repeater’s DL outside-passband ACLR for each class respectively. Here the ACLR include the relative ACLR and absolute ACLR.

3. Conclusions

In this contribution, NR repeater emission related radiated requirements are discussed with following observation and proposals:

Observation 1: In-passband ACLR requirement matters only when no wanted signal is transmitting at adjacent channel.

Observation 2: for WA, MR and LA, adjacent channel emission inside passband is larger than amplified noise floor with the same ACLR assumption as gNB spec.

Proposal 1: it is suggested that in-passband ACLR is based on manufacturer’s declaration to trade off cost against RF requirement.

Proposal 2: the same ACLR requirement as gNB still apply for repeater’s DL outside-passband ACLR for each class respectively. Here the ACLR include the relative ACLR and absolute ACLR.

# 4. Annex

Table comparison of amplified noise and adjacent channel unwanted emission for 24.25-33.4GHz

| DL | WA |  | MR |  | LA |  |
| --- | --- | --- | --- | --- | --- | --- |
| ACLR | 28dBc When output power larger than 15dBm/MHz | -13dBm/MHz When output power larger than 15dBm/MHz | 28dBc When output power larger than 8dBm/MHz | -20dBm/MHz When output power less than 8dBm/MHz | 28dBc When output power larger than 8dBm/MHz | -20dBm/MHz When output power less than 8dBm/MHz |
| BW for each carrier | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Adjacent channel emission | (Pout-28)/MHz | -13dBm/MHz | (Pout-28)/MHz | -20dBm/MHz | (Pout-28)/MHz | -20dBm/MHz |
| NF assumption | 7dB | 7dB | 7dB | 7dB | 7dB | 7dB |
| Gain range to ensure adjacent channel emission is higher than noise floor | Gain<94dB | Gain<94dB | Gain<87dB | Gain<87dB | Gain<87dB | Gain<87dB |

# 5. Reference

[1] R4-2115722, WF on NR Repeater FR2 RF , huawei