3GPP TSG RAN WG4 Meeting #101-e	R4-2117753

Electronic Meeting, November 1-12, 2021

Agenda item:			5.1.5

Source:	vivo

Title:	Remaining issues on CSI-RS L3 measurement core requirements

Document for:	Discussion

- Introduction

In the RAN4#100-e meeting, there were continued discussions on core requirements for CSI-RS based L3 mobility. One remaining open issue is captured in the WF [1].

We provided our views on core requirements for CSI-RS based L3 measurement in [2, 3, 4, 5] in the past meetings. In this contribution, we further provide our views on the remaining open issue in [1].

- Discussion

The UE behaviour when the timing offset exceeds the threshold was extensively discussed in the past meetings. Two options are listed for further discussion. In the RAN4#100-e meeting, following agreements were made.

Sub-topic 1-2 UE behavior when the timing offset exceeds the threshold with single FFT assumption - Agreements: - No accuracy requirements will be defined for the case when the timing offset exceeds the threshold with single FFT assumption. - If timing offset exceeds the threshold - Option 1: UE does not report CSI-RS based L3 measurements - Option 2: UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption - Option 3: UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption

Firstly, for the case when timing offset exceeds the threshold, UE with single FFT implementation is not required to meet any accuracy requirements as it will not be defined.

Then, UE behaviour needs to be clarified for the case when timing offset exceeds the threshold.

UE does not report measurement results when the timing offset exceeds threshold in option 1. If so, UE may never perform CSI-RS based measurement between serving cell and neighbour cells for intra frequency measurement due to large timing offset, especially in Heterogenous network. The reason of no reported measurement results is unknown to network. If event triggered measurement is configured then it may be due to event condition is not met, or it could be due to timing offset threshold is not met. This may also highly degrade UE mobility performance.

For Option 2 and 3, our understanding is that it is up to UE implementation whether CSI-RS based L3 measurement results is reported or not though UE is not required to report when timing offset exceeds the threshold. However, if UE reports the measurement results, it may need clarification on reporting requirements in sections 9.10.2.4 and 9.10.3.4.

In existing requirements, when measurement results are reported with periodic reporting, event triggered periodic reporting, and event triggered reporting, corresponding accuracy requirements should be met.

Reported CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2.3, 10.1.3.3, 10.1.7.2, 10.1.8.2, 10.1.12.2 and 10.1.13.2.

In section 10.1.2.3 for accuracy requirements:

The accuracy requirements in Table 10.1.2.3.1-1 are valid under the following conditions: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each associated SSB. - Conditions for intra-frequency measurements are fulfilled according to Annex B.2.12 for a corresponding Band for each relevant CSI-RS to be measured. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1.2.3.1-1. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

So, the accuracy requirements are only valid for the case when timing offset is no larger than the threshold. If UE reports measurement results for the case when timing offset exceeds the threshold, there is no accuracy requirements to meet. If option 2 is followed it means there are accuracy requirements to meet if timing offset exceeds threshold, which would contradict agreements that no accuracy requirements will be defined for the case.

Therefore, we think option 3 is a more appropriate way to conclude the issue. UEs with multiple FFT implementation can still meet the measurement reporting requirements and accuracy requirements. UEs with single FFT implementation may or may not meet accuracy requirements.

Proposal 1: UE is not required to report CSI-RS based L3 measurements when timing offset exceeds the threshold. If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption.

- Summary

In this contribution we further provided our views on the remaining open issue for CSI-RS based measurement. Based on analysis following proposal is present.

Proposal 1: UE is not required to report CSI-RS based L3 measurements when timing offset exceeds the threshold. If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption.

- References

- R4-2115321	WF on CSI-RS based L3 measurement requirements, CATT

- R4-2101767	Remaining issues on CSI-RS L3 measurement core requirements, vivo

- R4-2106614	Remaining issues on CSI-RS L3 measurement core requirements, vivo

- R4-2109933	Remaining issues on CSI-RS L3 measurement core requirements, vivo

- R4-2112539	Remaining issues on CSI-RS L3 measurement core requirements, vivo