3GPP TSG-RAN4 #101-e	R4-2117786

Electronic Meeting, 1st Nov – 12th Nov 2021

Title:	Testability issue with FR2 CSI-RS based RLM TCs

Source:			Anritsu Corporation

Agenda Item:			4.1.7

Document for:			Approval

# 1.	Introduction

At RAN4 #100-e, one CR was submitted to change test conditions of FR2 RRM CSI-RS-based radio link monitoring (RLM) test cases [1]. It was triggered by the previously identified issue in RAN5 [2][3] listed as the issue #2, and the proposal in the CR was to change the angle of arrival setup from 2 AoA to 1 (beam peak). However due to a concern of test case coverage reduction, during the #100-e discussion the group preferred to investigate a feasibility of the test while maintaining the 2 AoA setting.

In this contribution we discuss the possible solution while still maintaining the 2AoA setting.

# 2.	Discussion

2.1 Identified issue with FR2 RRM RLM test cases

As mentioned above, the issue was first identified in RAN5 and shared in the group. We extract the contents from the RAN5 paper [2] below for reference.

Extract from [2]

2.2: Angle of Arrival gives insufficient dB range Summary of issue and why it needs to be resolved: Some CSI-RS-based RLM Test cases specify 66 allocated RBs and Spherical Coverage Angles of Arrival (setup 3). The total Io power calculated from #RBs together with the Noc and Es/Noc values is higher than the test equipment can provide when MU and Test Tolerances are taken into account. Example: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM A.7.5.1.5 specifies 66 allocated RBs, with Noc -92.1dBm/15kHz and Cell 2 Es/Noc +2dB during T1. When MU and Test Tolerances are taken into account, this would require a total Io power higher than the test equipment can provide. Scope: Assessment required, but Test cases A.7.5.1.5, A.7.5.1.6 + related A.5 TCs are known to have this issue. Likely way forward: CSI-RS are distributed, so it is unlikely the number of allocated RBs can be reduced. Investigate changing the Angle of Arrival to Rx Beam Peak, so that a lower Noc and hence lower Io can be used.

Table 2.1-1 below is the actual estimation result of possible Io (total power at centre of QZ) based on the current conditions in TC A.5.5.1.5.1 (i.e. 66 RBs). As can be seen in the table, maximum Io may reach -51.25 dBm/ChBW when we take into account of a test tolerance. Now current assumption of achievable DL power from the test equipment is shown in the Table 2.1-2, extracted from TR 38.903 [4]. As CSI-RS based RLM test case has fading, what we can observe from this estimation is that it is challenging to provide the output power from the test equipment condition under the currently specified number of data RBs (i.e. 66 RBs) allocated also with the 2AoA for band n261 and higher frequency.

Observation 1: With the current parameter of CSI-RS based RLM test cases, there is a case that estimated Io exceeds the power level that the test equipment can provide when we consider a test tolerance.

Observation 2: It is challenging to carry out the CSI-RS-based RLM test under the current test conditions also with 2AoA at frequencies for band n261 and higher.

Table 2.1-1: Estimation of possible Io for TC A.5.5.1.5.1-3

Table 2.1-2 Achievable DL power from test equipment (from TR 38.903 [4])

|  | FR2a(Mid) Band n257/n258 | FR2b(Mid) Band n260/n261 | FR2c(Mid) Band n259 |
| --- | --- | --- | --- |
| CW power limit | -32.8 | -36.4 | -39.4 |
| CREST(Static) | 13 | 13 | 13 |
| Static Max. DL Power | -45.8 | -49.4 | -52.4 |
| CREST(Faded) | 17.71 | 17.71 | 17.71 |
| Faded Max. DL Power | -50.51 | -54.11 | -57.11 |

2.2 Consideration on a possible solution

To solve the issue above while keeping the 2AoA condition, we need to reduce the total power (Io) in some way. A straightforward solution is to reduce the number of RBs for example down to 24 like the similar test case for example A.5.5.1.1 SSB-based RLM test. And there would be two possible ways to reduce the number of allocated RBs; option 1 to reduce the BWP size, or option 2 to reduce the allocated RB size (OCNG) while keeping the BWP size (100MHz). These two ideas for the resource allocation are depicted in appendix A at the end of this paper. However as also already written in the previous RAN5 paper [2], we need to be careful with the physical layer (RAN1) specification with regards to the bandwidth of the CSI-RS when we consider the reduction of the allocated RB numbers. In TS 38.214 [5] clause 5.1.6.1.1, there is a limitation on the number of resource blocks. Corresponding description is extracted as follows.

Extract from TS 38.214 [5] cl. 5.1.6.1.1

the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks.

From the description above, it is not allowed to configure CSI-RS for tracking with 24 RB allocation also with 66 RB BWP setting. In other words, the tracking RS (TRS) exists at the resources from 25th to 66th RB even when we confine the OCNG allocation down to 24 RBs as far as the BWP setting is 66 RB, by which total power at symbols where CSI-RS exists should have more power than other symbols.

Observation 3: It is not allowed to configure CSI-RS for tracking with 24 RB allocation also with 66 RB BWP setting, which makes the total power at symbols where CSI-RS exists bigger than other symbols.

Hence, the remaining approach is option 1 to reduce the BWP size to 24RBs (minimum one). Table 2.2-1 shows the calculation results of the applied Io when we reduce the allocated RBs to 24. As can be seen in the table, Io is now reduced to -55.65 dBm/ChBW. The estimated Io level is now reduced lower than the achievable DL power for n257, n258, n260 and n261. However, it is still higher than the achievable level for band n259.

Observation 4: One possible solution is to reduce the BWP size to 24 RBs.

Observation 5: If BWP size is changed to 24 RBs, then the FR2 CSI-RS based RLM test cases will become testable with 2 AoA condition at least for band n257, n258, n260 and n261.

Table 2.2-1: Estimation of possible Io for TC A.5.5.1.5.1-3 with 24 RB data allocation

Note that there is no meaning to reduce the RB numbers to allocate OCNG less than 24RBs because of the existence of SSB (240 sub carriers and 20RBs). So, the testability issue for band n259 and higher frequency will remain.

Another possibility is as already discussed but to test at the beam peak with 1AoA test setup. Table 2.2-2 shows the analysis for the Io when the test setup is changed to 1AoA. Io at maximum is -62.91dBm/ChBW and is less than the achievable DL power even for band n259. FFS for band n262 since RAN5 has not defined the achievable level for the band n262 yet.

Table 2.2-2: Estimation of possible Io for TC A.5.5.1.5.1-3 with 1AoA (beam peak) setup assumption

Observation 6: If test setup is changed to 1AoA (beam peak), then the FR2 CSI-RS based RLM test cases will become testable for FR2-1 (up to band n259). FFS for band n262.

Now given the current situation that the group prefers to keep the 2 AoA test condition, though there is still the testability issue with the band n259 and higher range, and also there is a need to adjust parameters for the test tolerance in RAN5, it is better we take the option 1 (i.e. reduce the BWP size to 24 RBs). Besides since we already have the precedent that we define 24 RBs for data RBs in A.5.5.1.1 SSB-based RLM TC, there should be no reason to prevent RAN4 agreeing option 1.

Proposal 1: For CSI-RS based RLM test cases in FR2, reduce the BWP size setting to 24 RBs.

- Conclusion

In this contribution we discussed the testability issue with CSI-RS based RLM test cases in FR2 with 2AoA condition and also the possible solution while still maintaining the 2AoA setting.

Observation 1: With the current parameter of CSI-RS based RLM test cases, there is a case that estimated Io exceeds the power level that the test equipment can provide when we consider a test tolerance.

Observation 2: It is challenging to carry out the CSI-RS-based RLM test under the current test conditions also with 2AoA at frequencies for band n261 and higher.

Observation 3: It is not allowed to configure CSI-RS for tracking with 24 RB allocation also with 66 RB BWP setting, which makes the total power at symbols where CSI-RS exists bigger than other symbols..

Observation 4: One possible solution is to reduce the BWP size to 24 RBs.

Observation 5: If BWP size is changed to 24 RBs, then the FR2 CSI-RS based RLM test cases will become testable with 2 AoA condition at least for band n257, n258, n260 and n261.

Observation 6: If test setup is changed to 1AoA (beam peak), then the FR2 CSI-RS based RLM test cases will become testable for FR2-1 (up to band n259). FFS for band n262.

Proposal 1: For CSI-RS based RLM test cases in FR2, reduce the BWP size setting to 24 RBs.

Example of changes for the test cases in TS 38.133 are shown in the Appendix B below.

# 4.	References

[1] R4-2111868, “Update FR2 CSI-RS-based RLM Test cases”, Anritsu corp., RAN4 #100-e, Electronic meeting

[2] R5-212634, “FR2 RRM test cases: Known Issues”, Anritsu Ltd., RAN5 #91-e, Electronic meeting

[3] R5-213219, “FR2 RRM test cases: Known Issue List”, Ericsson, RAN5 #91-e, Electronic meeting

[4] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”,  v16.9.0, 2021-09

[5] TS 38.214, “Physical layer procedures for data”, v16.7.0, 2021-09

# Appendix A: Resource allocation image

Option 1: Reducing BWP size to 24 RBs					Option 2: Reducing allocated RB size (OCNG) to 24 RBs

# Appendix B: Example of changes in TS 38.133

<< Start of change >>

A.3.9.2.2	Dedicated BWP

Table A.3.9.2.2-1: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference BWP |  |  |  | DLBWP.1.1 | DLBWP.1.2 | DLBWP.1.3 | DLBWP.1.4 |  |
| Starting PRB index |  |  |  | 0 | RBb Note 1 | RBa Note 2 | 0 |  |
| Bandwidth |  | RB |  | Same as RF channel defined in each test | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 24 for SSB SCS 120 kHz 24 for SSB SCS 240 kHz |  |
| Note 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. Note 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3.10. |  |  |  |  |  |

<< Unchanged sections skipped >>

A.3.9.3.2	Dedicated BWP

Table A.3.9.3.2-1: Uplink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference BWP |  |  |  | ULBWP.1.1 | ULBWP.1.2 | ULBWP.1.3 | ULBWP.1.4 |  |
| Starting PRB index |  |  |  | 0 | RBb Note 1 | RBa Note 2 | 0 |  |
| Bandwidth |  | RB |  | Same as RF channel defined in each test | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 25 for SSB SCS = 15KHz, 51 for SSB SCS = 30KHz, 32 for SSB SCS = 120KHz 48 for SSB SCS = 240KHz | 24 for SSB SCS 120 kHz 24 for SSB SCS 240 kHz |  |
| Note 1: RBb is same as RBb for DLBWP.1.2 as defined in Table A.3.9.2.2-1. Note 2: RBa is same as RBa for DLBWP.1.3 as defined in Table A.3.9.2.2-1. |  |  |  |  |  |

<< Unchanged sections skipped >>

#### A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode

##### A.5.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when no DRX is used. This test will partly verify the FR2 TDD PSCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.5.1-1, A.5.5.1.5.1-2, A.5.5.1.5.1-3 and A.5.5.1.5.1-3A below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.5.1-1 shows the variation of the downlink SNR in the E-UTRAN PCell and the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms). In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.5.5.1.5.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.5.5.1.5.1-2: General test parameters for FR2 PSCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active E-UTRA PCell |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  | 1 |
| Active PSCell |  |  | Cell 2 |
| RF Channel Number |  |  | 2 |
| Duplex Mode |  |  | TDD |
| BWchnannel |  | Config 1, 2 |  | 100: NRB,c = 66 |
| Data RBs allocated | Config 1, 2 |  | 24 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1, 2 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2 |  | DLBWP.1.4 |
| UL initial BWP configuration | Config 1, 2 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2 |  | ULBWP.1.4 |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.3 TDD |
|  | Config 2 |  | CCR.3.1 TDD CCR.3.3 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
|  | Config 2 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
|  | Config 2 |  | SMTC.1 |
| PDSCH/PDCCH | Config 1 |  | 120 KHz |
| subcarrier spacing | Config 2 |  | 120 KHz |
| CSI-RS for RLM | Config 1, 2 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.2 |
| CP length |  |  | Normal |
| Out of sync | DCI format |  | 1-0 |
| transmission parameters | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI | Config 1 |  | CSI-RS.3.1 TDD |
| reporting | Config 2 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.35 |
| T3 |  | s | 0.35 |
| D1 |  | s | 0.31 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: E-UTRAN is in non-DRX mode under test. |  |  |  |

Table A.5.5.1.5.1-3: Cell specific test parameters for FR2 for CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  |  | Setup 3 defined in A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | Not sent |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1, 2 | dB | 2Note 11 | -6Note 11 | -15 |  |  |  |
| SNR on RLM-RS2 | Config 1, 2 |  | Not sent |  |  | 2Note 11 | -14 | -15 |
|  | Config 1, 2 | dBm/ 15kHz | -92.1 |  |  | -92.1 |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  | TDL-A 30ns 75Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.5.5.1.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |  |

<< End of change >>

Similar corrections are necessary with A.5.5.1.6, A.7.5.1.5, and A.7.5.1.6.