3GPP TSG-RAN WG4 Meeting #101-e	R4-2117853

# Electronic Meeting, November 01-12, 2021

Title:	Discussion on power control and relaxation framework for FR2 inter-band UL CA

Source:	Samsung

Agenda item:	8.4.2.2.1

Document for:	Discussion

1.	Introduction

After per-band requirement scheme was agreed for FR2 inter-band UL CA, the discussion has been focusing on how to derive the relaxation values in UL CA requirements. The progress on the relaxation discussion is a little slow. In our understanding, one of the reason is that companies have diverged interpretation on some fundamental issues like total power concept and independent power control, the other reason is lacking of consensus on the relaxation framework.

In this contribution, we present our view on these fundamental issues and relaxation framework.

2. 	Discussion

2.1	independent power control and total power concept

CA power control scheme is the scope of RAN1 responsibility. However, for FR2, RAN4 has agreed not to introduce p-NR-FR2 or p-UE-FR2 in Rel-17. For FR2 inter-band UL CA/DC, there is no clear rule in RAN1 yet. In past RAN4 meetings, RAN4 has discussed the LS [1, R4-2107605(R1-2104018)] from RAN1:

……

RAN1 is discussing below changes as one possible solution to define power control for uplink CCs of MCG in FR2 and uplink CCs of SCG in FR2 in NR-DC case without requiring p-NR-FR2

----start TP to sub clause 7.6.2 of TS 38.213----------

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, or if a UE is configured with an MCG using NR radio access in FR2 and with a SCG using NR radio access in FR2 and is not configured with p-NR-FR2, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

--------- end TP ----------------

However, RAN1 cannot determine if it is a feasible solution and would like to ask RAN4 inputs about the feasibility, for the following two possible cases:

1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2.

2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2.

……

Though above LS is for FR2 NR-DC, it is very similar situation for FR2 inter-band UL CA. RAN4 discussion on this issue was also closely coupled with FR2 inter-band UL CA requirements. In last RAN4 meeting, a WF [2, R4-2115081] is approved with following agreements:

- Seeking to have a common understanding of “independent power control”

  - Option 1 (“independent power control” means per CG power control and there is no total power limitation.) is agreeable.

- Will there be realistic total power limitation even without the definition of “p-NR-FR2”? If so, what is the implication on the understanding of “independent power control”?

  - Wait for the outcome of total power concept discussion in FR2 inter-band UL CA, before evaluating the NR-DC case

  - LS will be sent to RAN1 after the conclusion of above

Total power concept for FR2 inter-band UL CA was proposed in RAN4 FR2 RF WI thread. It is unclear in both RAN1 and RAN4. It seems RAN1 is relying on p-UE-FR2 which is not available yet and waiting for RAN4 input. We have introduced this issue in our previous contribution [3, R4-2112575], now based on the latest WF above, it is necessary to discuss independent power control and total power concept in FR2 RF WI.

RAN4 has confirmed that MOP requirements for FR2 inter-band UL CA based on IBM between different frequency groups are per-band. However, per-band requirements do not exactly mean independent power control. There is no agreement yet that Independent Beam Management means independent power control.

Independent power control for FR2 NR-DC means per CG power control and there is no total power limitation. Similarly we can say that independent power control for FR2 inter-band UL CA means per band power control and there is no total power limitation.

Proposal 1:	RAN4 need to discuss if independent power control apply for FR2 inter-band UL CA based on IBM between different frequency group, with the understanding that independent power control for FR2 inter-band UL CA means per band power control and there is no total power limitation.

Here the key issue is total power concept. The approved WF [4, R4-2114961] of RAN4#100e meeting captured following agreements on total power concept:

- applicability of total power concept

  - Agreement

    - No new requirements than the per-band based requirement package of max EIRP, max TRP, min peak EIRP, EIRP spherical coverage.

    - Further study the impact of total power concept to max EIRP, max TRP, min peak EIRP, EIRP spherical coverage, and how to address it.

The possibly impacted items are narrowed down to max EIRP, max TRP, min peak EIRP, EIRP spherical coverage.

For max EIRP and max TRP, when per-band scheme was agreed for UL CA based on IBM between different frequency groups, the per-band scheme was based on regulation feedback that max EIRP and max TRP apply per-band independently. So the total power concept has no impact to max EIRP and max TRP for IBM between different frequency groups.

For min peak EIRP and EIRP spherical coverage, it is more related to the relaxation framework than total power limitation. Min peak EIRP and EIRP spherical coverage are directional metrics, so there is no practical meaning to sum two EIRP values at different direction. Moreover, On the other hand, total power limitation in conductive domain for FR1 requires either 3dB reduction per-band or dynamic power sharing among bands, however, in FR2 there are no conductive metrics to quantify the total power limitation. So in our view, total power concept can be decoupled with independent power control, i.e., for FR2 inter-band UL CA, there is no “total” power limitation even though there may be EIRP relaxation for “each” band.

Observation 1:	for FR2 inter-band UL CA based on IBM between different frequency groups, there is no “total” power limitation even though there may be EIRP relaxation for “each” band.

Based on above observation, it can be concluded that independent power control apply for FR2 inter-band UL CA based on IBM between different frequency groups. However, the total power concept also reflects the necessity to relax the maximum output power requirement to address the power consumption and heating issue especially for power class 3 mobile devices. Moreover, in the approved WF [4, R4-2114961] it was also agreed that UE power consumption under MOP status should be addressed and further study how to address it. So relaxation due to total power concept can be discussed together with other relaxation factors.

Proposal 2:	total power concept can be decoupled with independent power control. The relaxation due to total power concept can be absorbed into the relaxation framework discussion

2.2	relaxation framework for inter-band UL CA

We have discussed varieties of relaxation factors for UL CA no matter MOP or MPR to be addressed in. To summarize, we can see following 3 main categories of relaxations:

- Relaxation due to CA operation (i.e. the X&Y values to address common coverage, MBR, concurrent operation, total power concept, power consumption and heating, etc.)

- Relaxation due to PA-PA interaction (i.e. the relaxation to address emission requirements either in MOP or MPR)

- Relaxation due to regular MPR (i.e. the regular MPR for different waveform/modulation/RB/BW/etc.)

To merge above power relaxations into a reasonable format, first of all we need to make clear the basis of CA MPR. Is CA MPR based on CA MOP or SC MOP (Single Carrier MOP)? Logically CA MPR should be based on CA MOP, but on the other hand we also see many cases in 3GPP specification that CA MOP equals to SC MOP though there is always non-zero MPR, in this case, CA MOP is not a rigid physical requirement but only plays the role of reference benchmark for that power class. For example, in FR2 intra-band CA, CA MOP requirement is specified the same as SC MOP while in MPR section all MPR values are non-zero. If looking into MPRNC_CA requirement for UE power class 3 in TS38.101-2, we can see near 6dB relaxation from MOP due to CA operation is allowed, while more relaxation could be added on top of that to address different waveform/modulation/BW:

Table 6.2A.2.4.2-1: MPRNC_CA for UE power class 3

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.3 | ≤ 9.8 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 6 | ≤ 7.5 | ≤ 8.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 7 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |

Another example in FR1 is the dual layer UL MIMO MOP, which is also specified the same MOP requirement as SC MOP, but there is non-zero MPR for CP-OFDM waveform in MPR section. In the latest TxD discussion, we can also see similar cases where non-zero MPR always apply.

Observation 2:	there are precedent examples that CA MOP only plays as reference benchmark for that power class and CA operation relaxation is absorbed in non-zero MPR, e.g. about 6dB “CA operation relaxation” in MPRNC_CA for UE power class 3

Based on observation 2, we see the possibility that CA MOP also equals to SC MOP for inter-band CA. With this possibility and considering discussions in previous meetings, we can see there are 3 options for relaxation framework:

- Option 1: CA MOP = SC MOP;

    - {CA operation relaxation, modulation/waveform/BW/RB relaxation, PA-PA relaxation }  MPR

- Option 2: CA MOP = SC MOP – {CA operation relaxation};

    - { modulation/waveform/BW/RB relaxation, PA-PA relaxation }  MPR

- Option 3: CA MOP = SC MOP – {CA relaxation,  PA-PA relaxation};

    - { modulation/waveform/BW/RB relaxation}  MPR

Note: different relaxation factors may overlap and double counting should be avoided when necessary.

Option 2 and 3 are under discussion (X&Y values apply to min peak EIRP and spherical coverage; PA-PA relaxation apply to either CA MOP or CA MPR to be down-selected). Option 1 is newly added based on Observation 2.

Comparing Option 2 and 3, we can see technical issue in Option 3 because the relaxation will be double counted for PA-PA relaxation and regular MPR. The correct calculation should be max of PA-PA relaxation and regular MPR. If PA-PA relaxation is taken into CA MOP, then the total power reduction will be the sum of PA-PA relaxation and regular MPR which leads to double counting. Consequently, PA-PA relaxation should be taken into MPR, and Option 3 can be excluded.

Proposal 3:	PA-PA relaxation should be taken into MPR than MOP

Then the issue left is how to handle “CA operation relaxation”, either in MOP (i.e. Option 2) or in MPR (i.e. Option 1). If going with Option 2, then it would be the 1st case in 3GPP where CA MOP does not equal to SC MOP. It seems a fundamental issue and further discussion is needed.

Proposal 4:	MOP and MPR requirements for inter-band UL CA should be discussed and agreed in a package. Detailed relaxation framework shall be down-selected between Option 1 and Option 2.

- Option 1: CA MOP = SC MOP;

    - {CA operation relaxation, modulation/waveform/BW/RB relaxation, PA-PA relaxation }  MPR

- Option 2: CA MOP = SC MOP – {CA operation relaxation};

    - { modulation/waveform/BW/RB relaxation, PA-PA relaxation }  MPR

3. 	Conclusion

Proposal 1:	RAN4 need to discuss if independent power control apply for FR2 inter-band UL CA based on IBM between different frequency group, with the understanding that independent power control for FR2 inter-band UL CA means per band power control and there is no total power limitation.

Observation 1:	for FR2 inter-band UL CA based on IBM between different frequency groups, there is no “total” power limitation even though there may be EIRP relaxation for “each” band.

Proposal 2:	total power concept can be decoupled with independent power control. The relaxation due to total power concept can be absorbed into the relaxation framework discussion

Observation 2:	there are precedent examples that CA MOP only plays as reference benchmark for that power class and CA operation relaxation is absorbed in non-zero MPR, e.g. about 6dB “CA operation relaxation” in MPRNC_CA for UE power class 3

Proposal 3:	PA-PA relaxation should be taken into MPR than MOP

Proposal 4:	MOP and MPR requirements for inter-band UL CA should be discussed and agreed in a package. Detailed relaxation framework shall be down-selected between Option 1 and Option 2.

- Option 1: CA MOP = SC MOP;

    - {CA operation relaxation, modulation/waveform/BW/RB relaxation, PA-PA relaxation }  MPR

- Option 2: CA MOP = SC MOP – {CA operation relaxation};

    - { modulation/waveform/BW/RB relaxation, PA-PA relaxation }  MPR

4. 	References

- R4-2107605(R1-2104018)    “Further Reply LS on power control for NR-DC”, RAN1

- R4-2115081    “WF on FR2 power control for NR-DC”, OPPO

- R4-2112575    “Discussion on FR2 inter-band UL CA”, Samsung

- R4-2114961    “WF on FR2 UL CA”, Samsung