3GPP RAN WG4 Meeting #101-e		R4-2117990

Online, November 1st – 12th, 2021

Agenda item:	8.3.2.6

Source:	Apple

Title:	Views on SCell dropping for UL CA

WI/SI:	NR_RF_FR1_enh-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The SCell power reduction or dropping in UL CA maximum output power condition was first identified in FR2 as a RAN5 conformance test issue which was brought up in RAN4 #96-e meeting [1,2]. The issue was further discussed in the next few RAN4 meetings with the focus on whether equal PSD condition for intra-band UL CA should be maintained for testing maximum output power associated requirements [3]. And the conclusion was drawn in an LS to RAN5 with RAN4’s recommendation that equal PSD is a preferred test condition to verify the UL CA requirements, but the testing details are up to RAN5 [4].

While the issue was thought being closed in RAN4, it was further brought up in last two RAN4 meetings with the concern that the SCell power reduction or dropping, though recognized as an expected UE behavior according to the PCell prioritization rule in UL CA power control defined in RAN1 specifications [5], would cause impact on UL throughput in the field [6]. And a new requirement in RAN4 to limit the serving cell output power was proposed to intend to resolve the issue [7,8] which was postponed as the necessity for introducing a new RAN4 requirement is not yet clarified. On the other hand, a WF on “solution for SCell dropping issue” was approved in last RAN4 meeting [9] and an LS was sent to RAN1 to solicit their understanding of this low priority CC dropping issue and the possible spec impact [10]. In this contribution, we share our further views on SCell dropping in UL CA and propose to hold off the consideration of any specification changes in RAN4 and RAN1 until the SCell dropping is confirmed as a real field issue.

# Discussion

Intra-band UL CA has been introduced since LTE Rel-10 specifications around 10 years ago and the feature is also carried on in NR FR1 Rel-16 and FR2 Rel-15 specifications. There were numerous 4G devices in the field already supporting UL CA operation as well as with freshly launched 5G UEs. However, there has not been UL CA SCell dropping issue reported in the field as recently identified in RAN5 FR2 conformance tests for UL CA requirements. Therefore, whether SCell dropping in UL CA would really happen in the field or result in any performance impact is subject to further investigation. On the other hand, SCell power reduction or dropping is known as an expected UE behavior according to the PCell prioritization rule in RAN1 specifications which in principle should not do more harm than good to the network operation.

In our view, there is a clear distinction between the conformance test and field operation under the maximum output power condition. In conformance test, the tester would continue sending the TPC “UP” commands to the DUT till its output power saturates which would be an indication of reaching UE’s PCMAX. Unlike in real network, there is no UE power headroom (PHR) reporting back to the tester during the TPC “UP” processing in the conformance test. As a result, UE would continue scaling back its SCell output power to leave more power to PCell according to the prioritization rule till SCell is dropped. On the other hand, in real network, the base station would adjust the TPC based on the UL signal SNR condition and UE’s PHR. If under UL CA there is insufficient SNR for PCell and UE is also running out of the PHR for CA, then the network may schedule UE to deactivate SCell or even reduce the PCell resource allocation in order to boost up the PCell SNR. Therefore, even without the PCell prioritization rule, there should be a mechanism for network to deactivate SCell to maintain the PCell performance. And in our view the SCell deactivation by the network should be a better way to manage the network performance instead of leaving UE to drop SCell by itself where the SCell power scaling is essentially out of network’s control.

Observation 1: There is a clear distinction between the conformance test and field operation under the maximum output power condition.

Observation 2: In conformance test, there is no UE power headroom (PHR) reporting back to the tester during the TPC “UP” processing.

Observation 3: In real network, the base station would adjust the TPC based on the UL signal SNR condition and UE’s PHR.

Observation 4: Even without the PCell prioritization rule, there should be a mechanism for network to deactivate SCell to maintain the PCell performance.

Observation 5: SCell deactivation by the network should be a better way to manage the network performance instead of leaving UE to drop SCell by itself where the SCell power scaling is essentially out of network’s control.

On the other hand, the new requirement as proposed in last RAN4 meeting by limiting the serving cell output power is virtually no difference with the existing TPC mechanism. For example, scheduling a DPCMAX,c = 3 dB to UE would be equivalent to network stop sending TPC “UP” command when PCell PHR is approaching 3 dB. The new requirement looks to be only effective in conformance test to prevent SCell from dropping since the tester does not have the knowledge of UE PHR in order to adjust or stop TPC “UP” commands. Therefore, whether the new requirement would really benefit the network operation is still subject to further investigation. If the new RAN4 requirement is to only resolve the UL CA conformance test issue, we do not think it is necessary as it would not only create more RAN4 specifications workloads but also increase UE test burden. The UL CA testing details should be up to RAN5 as already consented in the LS to RAN5 [4].

Observation 6: The new requirement as proposed in last RAN4 meeting by limiting the serving cell output power is virtually no difference with the existing TPC mechanism.

Observation 7: If the new RAN4 requirement is to only resolve the UL CA conformance test issue, it would not be necessary as it not only creates more RAN4 specifications workloads but also increases UE test burden.

In summary, based on our assessment above, we think further clarification is needed as whether UL CA SCell dropping would really take place in the field to impact the network performance. With that said, we propose to hold off the consideration of any specification changes in RAN4 and RAN1 until the SCell dropping in UL CA is confirmed as a real field issue but not merely a conformance test issue.

Proposal: 3GPP to hold off the consideration of any specification changes in RAN4 and RAN1 until the SCell dropping in UL CA is confirmed as a real field issue but not merely a conformance test issue.

# 3	Conclusion

In this contribution, we share our further views on SCell dropping in UL CA and propose to hold off the consideration of any specification changes in RAN4 and RAN1 until the SCell dropping is confirmed as a real field issue.

Observation 1: There is a clear distinction between the conformance test and field operation under the maximum output power condition.

Observation 2: In conformance test, there is no UE power headroom (PHR) reporting back to the tester during the TPC “UP” processing.

Observation 3: In real network, the base station would adjust the TPC based on the UL signal SNR condition and UE’s PHR.

Observation 4: Even without the PCell prioritization rule, there should be a mechanism for network to deactivate SCell to maintain the PCell performance.

Observation 5: SCell deactivation by the network should be a better way to manage the network performance instead of leaving UE to drop SCell by itself where the SCell power scaling is essentially out of network’s control.

Observation 6: The new requirement as proposed in last RAN4 meeting by limiting the serving cell output power is virtually no difference with the existing TPC mechanism.

Observation 7: If the new RAN4 requirement is to only resolve the UL CA conformance test issue, it would not be necessary as it not only creates more RAN4 specifications workloads but also increases UE test burden.

Proposal: 3GPP to hold off the consideration of any specification changes in RAN4 and RAN1 until the SCell dropping in UL CA is confirmed as a real field issue but not merely a conformance test issue.

# 4	References

R4-2009656 “NR SCC UL power drop behavior with EN-DC UE in FR2”, Anritsu Corporation, 3GPP TSG-RAN WG4 Meeting #96-e, August 17th – 27th, 2020

R4-2011695 “WF on NR SCC UL power drop behavior in FR2”, Anritsu Corporation, 3GPP TSG-RAN WG4 Meeting #96-e, August 17th – 27th, 2020

R4-2016994 “WF on NR SCC UL power drop behavior in FR2”, Ericsson, 3GPP TSG-RAN WG4 Meeting #97-e, November 2nd – 13th, 2020

R4-2103124 “LS on SCell dropping”, OPPO, 3GPP TSG-RAN WG4 Meeting #98-e, January 25th – February 5th, 2021

3GPP TS 38.213 V16.6.0 (2021-06)

R4-2107762 “WF on introduction of power limits for serving cells of UL CA”, Ericsson, 3GPP TSG-RAN WG4 Meeting #99-e, May 19th – 27th, 2021

R4-2109957 “Introduction of power limits for serving cells of UL CA”, Ericsson, 3GPP TSG-RAN WG4 Meeting #99-e, May 19th – 27th, 2021

R4-2112826 “Resolution of the Scell dropping (power reduction) problem”, Ericsson, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021

R4-2114949 “WF on solution for Scell dropping issue”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021

R4-2114950 “LS on scell dropping issue of CA”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021