3GPP TSG-RAN WG4 Meeting #101-e															R4-2118150

Electronic Meeting, November 1 – 12, 2021

Agenda item:	4.1.9

Source: 	Qualcomm Incorporated

Title: 	Frequency bands for testing of A-GNSS sensitivity requirements in NR and LTE

Document for:		Discussion and Decision

# 1	Introduction

At RAN4#98-e meeting, the LS from RAN5 on testing requirements for A-GNSS Sensitivity [1] was first discussed. The actions to RAN4 were as follows:

To RAN 4 group ACTION: RAN5 respectfully asks RAN 4 for guidance on the LTE and NR frequency bands, and band combinations, impacting the A-GNSS Sensitivity requirements in LTE and NR, and in particular in EN-DC, taking into account possible intermodulation and other interference mechanisms that may affect the GNSS bands. In the case that this action is expected to take some time to complete, RAN 5 would greatly appreciate one or more status reports on the progress.

At RAN4#100-e progress was made on the EN-DC band combinations for the testing the A-GNSS sensitivity requirements and a corresponding CR to TS 38.171 was agreed in [2].

However, there was no progress on the LTE and NR frequency bands impacting the A-GNSS sensitivity requirements in LTE and NR.

In this contribution, we discuss the LTE and NR frequency bands possibly impacting the A-GNSS sensitivity requirements. Accordingly, draft CRs to TS 36.171 and 38.171 are proposed in the Annex of this contribution.

# 2.	LTE and NR frequency bands impacting the A-GNSS Sensitivity requirements

The current A-GNSS requirements in TS 36.171 [3] and TS 38.171 [4] do not provide any guidance on frequency bands for testing the requirements. Normally, this means that the performance of a test case is not influenced by the bearer band used for the test scenario and the test can be performed with any bearer band within a radio technology supported by the DUT. However, when the UE transmitter is operating it can affect the GNSS receiver sensitivity, dependent on the frequency band used for the transmission.

The following interference products are typically considered in GNSS interference scenarios:

(1)	Harmonic products (mainly 2nd harmonic, H2)

(2)	2-Transmitter 2nd order intermodulation products (IM2)

(3)	2-Transmitter 3rd order intermodulation products (IM3)

(4)	Cross-band isolation.

Based on the above considerations, a harmonic intermodulation analysis was performed in [5]. Harmonic and intermodulation products were considered for on device transmitters operating on LTE and NR frequency bands and band combinations. For the GNSS L1 frequency bands, the following high-risk bands potentially impacting the GNSS receiver sensitivity were found [5]:

Band 13:	2nd harmonic of UL TX falling into GPS, Galileo and BDS L1, E1, and B1 bands, respectively.

Band 14:	2nd harmonic of UL TX falling into GPS and Galileo L1, E1 bands, respectively.

Band 44:	2nd harmonic of UL TX falling into GPS, Galileo, BDS, and GLONASS L1, E1, B1, and G1 					bands, respectively.

Band 24:	Potential cross-band isolation issue (UL carrier adjacent to GNSS L1 band)

Note, we do not differentiate E-UTRA and NR operating bands above (since LTE or NR does not make a significant difference here), although, some LTE bands have not yet been specified for NR.

Proposal 1:	Regarding the RAN5 question on the LTE and NR frequency bands impacting the A-GNSS sensitivity requirements, provide the following response to RAN5:
RAN4 has identified the following E-UTRA/NR frequency bands potentially impacting the A-GNSS Sensitivity Requirements: Bands 13, 14, 44 24.

The test frequencies need to be selected appropriately for the desired test purpose. For example, the mid-range UL frequency for band 13 is set to 782 MHz [6]. The 2nd order harmonics would then be a jammer at 1564 MHz which falls outside the BDS B1 band for example.

Since TS 36.171 and TS 38.171 currently do not provide any guidance on frequency bands for testing the A-GNSS sensitivity requirements, the above identified high-risk bands should be captured in the core requirements. Draft CRs are provided in the Annex of this contribution.

Proposal 2:	Endorse the draft CRs to TS 36.171 and TS 38.171 provided in the Annex of this contribution.

# 3.	Summary

In this contribution, we discussed the RAN5 question on the LTE and NR frequency bands impacting the A-GNSS sensitivity requirements. The following proposals were made.

Proposal 1:	Regarding the RAN5 question on the LTE and NR frequency bands impacting the A-GNSS sensitivity requirements, provide the following response to RAN5:
RAN4 has identified the following E-UTRA/NR frequency bands potentially impacting the A-GNSS Sensitivity Requirements: Bands 13, 14, 44 24.

Proposal 2:	Endorse the draft CRs to TS 36.171 and TS 38.171 provided in the Annex of this contribution.

# References

[1]	R4-2100021, "LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE", RAN5.

[2]	R4-2115449, "Big CR to TS 38.171 on requirements maintenance (Rel-16)".

[3]	3GPP TS 36.171, "Requirements for Support of Assisted Global Navigation Satellite System (A-GNSS)".

[4]	3GPP TS 38.171, "Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[5]	R4-2109002, "Frequency bands for testing of A-GNSS sensitivity requirements in NR and LTE", Qualcomm 	Incorporated.

[6]	3GPP TS 36.508, "Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance 	testing".

# Annex

3GPP TSG-RAN Meeting #101-e	R4-21xxxxx

# Electronic Meeting, November 1 – 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.171 | CR | xxx | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Frequency bands for testing of A-GNSS sensitivity requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some configurations, the performance of the A-GNSS sensitivity test may be influenced by the bearer band used for the test scenario. However, the A-GNSS test requirements and scenarios do not specify any specific bands in which the test cases should be validated. RAN5 currently tests the A-GNSS (and A-GPS) sensitivity requirements in each UE supported frequency band. However, this is unnecessary and may result in excessive testing time since not all bearer bands affect the GNSS reception bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Only the frequency bands which can generate 2nd order harmonics and other distortions falling into the GNSS reception bands need to be tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear which frequency bands should be used for verifying A-GNSS sensitivity requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.171 CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 37.571 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See R4-2100021: "LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE" (RAN5). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## B.1.7	E-UTRA Frequency and frequency error

In all test cases other than Sensitivity in clause 5.1 the E-UTRA frequency used shall be the mid range for the E-UTRA operating band. The E-UTRA frequency with respect to the GNSS carrier frequency shall be offset by +0.025 PPM.

For verifying the sensitivity requirements in clause 5.1, the sensitivity tests shall be performed in each frequency band listed in Table B.1.7-1. The E-UTRA frequency and channel configuration shall be selected to ensure second order harmonics and other distortion will fall into the following GNSS receiver bands for the particular GNSS (where supported by the UE):

-	GPS L1 C/A:				1574.3970 – 1576.4430 MHz

-	Galileo E1 / GPS L1C:	1573.3740 – 1577.4660 MHz

-	GLONASS G1:			1597.5515 – 1605.8860 MHz

-	BDS B1I:					1559.0520 – 1563.1440 MHz

NOTE:	If the DUT does not support any of the frequency bands listed in Table B.1.7-1 the sensitivity tests in clause 5.1 can be performed in any frequency band supported by the DUT.

Table B.1.7-1: E-UTRA operating bands for verifying GNSS sensitivity

| E-UTRA operating bands | 13,14, 24, 44 |
| --- | --- |

3GPP TSG-RAN Meeting #101-e	R4-21xxxxx

# Electronic Meeting, November 1 – 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | xxx | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Frequency bands for testing of A-GNSS sensitivity requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some configurations, the performance of the A-GNSS sensitivity test may be influenced by the bearer band used for the test scenario. However, the A-GNSS test requirements and scenarios do not specify any specific bands in which the test cases should be validated. RAN5 currently tests the A-GNSS (and A-GPS) sensitivity requirements in each UE supported frequency band. However, this is unnecessary and may result in excessive testing time since not all bearer bands affect the GNSS reception bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Only the frequency bands which can generate 2nd order harmonics and other distortions falling into the GNSS reception bands need to be tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear which frequency bands should be used for verifying A-GNSS sensitivity requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 36.171 CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 37.571 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See R4-2100021: "LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE" (RAN5). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## B.1.7	E-UTRA or NR frequency and frequency error

In all test cases other than Sensitivity in clause 5.1 with E-UTRA frequency, the E-UTRA frequency used shall be the mid-range for the E-UTRA operating band. The E-UTRA frequency with respect to the GNSS carrier frequency shall be offset by +0.025 PPM.

In all test cases other than Sensitivity in clause 5.1 with NR, the NR frequency used shall be as specified in TS 38.508-1 [20], clause 4.3.1. The NR frequency with respect to the GNSS carrier frequency shall be offset by + 0.025 PPM.

For verifying the sensitivity requirements in clause 5.1 with NR single carrier, the sensitivity tests shall be performed in each frequency band listed in Table B.1.7-1. The NR frequency and channel configuration shall be selected to ensure second order harmonics and other distortion will fall into the GNSS receiver bands as defined in clause B.1.13.2 for the particular GNSS.

NOTE:	If the DUT does not support any of the frequency bands listed in Table B.1.7-1 the sensitivity tests in clause 5.1 can be performed in any frequency band supported by the DUT.

Table B.1.7-1: NR operating bands for verifying GNSS sensitivity

| NR operating bands | n14 |
| --- | --- |