3GPP TSG-RAN WG4 Meeting #101-e                                                             R4-2118196

Electronic Meeting, Nov. 1-12, 2021

Source:	ZTE Corporation

Title:	TP for TR 37.717-21-11:  DC_3A-7A_n38A

Agenda Item:	7.14.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on  DC_3A-7A_n38A for TR37.717-21-11[1] .

- Reference

- TR 37.717-21-11, Rel-17 Dual Connectivity (DC) of 2 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.6.0

- Text Proposal

----- Start of TP -----

  - 5.x			DC_3-7_n38

    - 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A_n38A17,18 | N/A | 3A | N/A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

To avoid duplicated NOTEs in the TS38.101-3 specification, it is proposed to add ‘or band n38’ on top of the existing NOTE 18 in table 5.5B.4.2 in the TS38.101-3, shown as above.

    - 5.x.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

    - 5.x.3	∆TIB and ∆RIB values

For DC_3-7_n38, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n38 | 3 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n38 | 3 | 0 |
|  | 7 | 0 |
|  | n38 | 0.2 |

    - 5.x.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

----- End of TP -----