3GPP TSG-RAN WG4 Meeting #101-e                                                            R4-2118220

Electronic Meeting, November 01-12, 2021

Source:	Samsung

Title:	Further Discussion on Transparent TxD – ULFPTx related

Agenda item:			8.7.3.2

Document for:	Discussion

# 1. Introduction

In RAN Plenary #92-e, RAN plenary approve the new work item on “UE RF requirements for Transparent Tx Diversity (TxD) for NR”, regarded the official work assignment tasked by RAN plenary on the long-lasting RAN4 discussion on transparent TxD which can be dated back to Rel-15 [1]. Specifically, the following objectives are set up by RAN plenary:

UE Requirements for the feature Transparent Tx Diversity (TxD): - Phase 1: - Complete needed specification changes to such items as spectral flatness, MPR, EVM, ACLR for all power classes to enable UE to implement Transmit Diversity (TxD) [RAN4] - The concluded agreements related to Tx Diversity (TxD) shall be followed - UE capability signalling of Transparent Tx Diversity (TxD) and allowing early implementation from Rel-15 [RAN2] - Phase 2: - Specify, if necessary, changes to enable Transmit Diversity (TxD) capable UE to support SRS antenna switching and Uplink full power transmission (ULFPTx). [RAN4, RAN2] - Leverage existing work for this feature done up to now. [RAN4, RAN2] - A TP will be provided for TR including previous background and agreements documented in TR format in the first meeting Note: Running CR is utilized to reflect the progress for phase 1 and phase 2. Phase 1 and phase 2 requirements can be studied in parallel, but phase 1 requirements have high priority to be finished firstly Note: The relationship between transparent TxD requirements and requirements on ULFPTx and SRS switching can be discussed in phase 1. In particular it should be agreed during phase 1 whether the requirements to be derived during phase 2 for ULFPTx and SRS switching should apply regardless of TxD capability/requirements/behaviour or not.

In this contribution, we would like to focus on the impact of introducing transparent TxD on existing Uplink Full Power Tx (ULFPTx) related requirement, and provide our viewpoints accordingly.

# 2. Discussion

## 2.1 Rel-16 ULFPTx Feature Review

In Rel-16, RAN1 has introduced the feature of UL full power transmission (ULFPTx) and RAN4 identified the corresponding impacts on RAN4 requirement. Here we would like to introduce the ULFPTx feature briefly, by presenting the three ULFPTx modes:

#### 2.1.1 ULFPTx Mode 1 (ul-FullPowerTransmission-r16 = ’fullpowerMode1’):

ULFPTx Mode 1 is proposed for the UEs with non-Coherent or partial coherent UL-MIMO codebook, with codbookSubset = nonCoherent. Particularly, new TPMI (originally for fullCoherent codebook in Rel-15) is enabled for partial-/non-coherent codebook as full-power transmission purpose for Rel-16 ULFPTx Mode 1 capable UE, while the same power scaling mechanism as Rel-15 is used. Although it is not explicitly required, ULPFTx Mode 1 is more reasonable to be applied for UE with either (1) UEs with no full-rated PAs on any of TX chains, or (2) UEs with full rated PAs on the subset of TX chains. Because RAN4 discussion is only restricted to 2TX case, so here below is the case which introduce RAN4 specification impact:

Fig. 1. Illustration of Rel-16 ULFPTx Mode 1

It should be noted that ULFPTx Mode 1 is different from Rel-15 full-coherent codebook transmission, because to utilize the new TPMI, Mode-1 UE need to have some implementation-wise solution to avoid nulling at the whole bandwidth, e.g., small-delay based CDD, which is the same mechanism discussed for transparent TxD.

#### 2.1.2 ULFPTx Mode 2 (ul-FullPowerTransmission-r16 = ’fullpowerMode2’):

Compared with Mode-1/0, ULFPTx Mode 2 is more complex, since two mechanisms are specified, where one of them is configured by gNB.  Similar to Mode 1, although it is not explicitly required, ULPFTx Mode 2 is more reasonable to be applied for UE with either (1) UEs with no full-rated PAs on any of TX chains, or (2) UEs with full rated PAs on the subset of TX chains.

- ULFPTx Mode 2 Mechanism-1 (SRS port virtualization):

  - In this Mechanism-1, UE is configured multiple SRS resources having different number of ports, while the full power transmission is achieved if SRI is indicated for the SRS with one port. In this mechanism-1, the same power scaling as Rel-15 is applied.

Fig. 2. Illustration of Rel-16 ULFPTx Mode 2 (Mechanism-1, SRS port virtualization)

- ULFPTx Mode 2 Mechanism-2 (TPMI indication):

  - In this Mechanism-2, UE reports existing TPMI(s) available for full power transmission, and the only difference comes from the different behaviour of power scaling:

    - For full power TPMI, Rel-16 power scaling factor s = 1 is applied;

    - For non-full-power TPMI, Rel-15 power scaling factor is applied, as illustrated below.

Fig. 3. Illustration of Rel-16 ULFPTx Mode 2 (Mechanism-2, TPMI indication)

The UE behaviour description is captured in TS38.213, in which Mechanism-1 ULFPTx with SRS port virtualization is highlighted by the yellow part, and Mechanism-2 ULFPTx for full power TPMI by the cyan part, while the grey one for non-full-power TPMI and conventional Rel-15 scaling behaviour.

- if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2, - s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', - s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and

#### 2.1.3 ULFPTx Mode 0 (ul-FullPowerTransmission-r16 = ’fullpower’):

ULFPTx Mode 0 is proposed for the UEs with full-rated PAs on each TX chain. For UE supporting ULFPTx Mode 0, Rel-16 power scaling factor s = 1, which is compared with Rel-15 power scaling factor (i.e., s = ratio of # non-zero PUSCH transmission power to # of SRS ports). For the below case, Rel-16 ULFPTx Mode-0 can enable the full power transmission while Rel-15 mechanism can only achieve half of power, i.e., s = ½ for TPMI = {1 0}.

Fig. 4. Illustration of Rel-16 ULFPTx Mode 0

Observation-1: Rel-16 ULFPTx feature can be categorized into Mode-0 (“fullpower” mode), Mode-1 and Mode-2, and in Mode-2 there are two mechanisms, i.e., Mechanism-1 for SRS port virtualization and Mechanism-2 for TPMI indication.

## 2.2 ULFPTx MOP Requirement and Transparent TxD

In Rel-16 RAN4 work for NR MIMO enhancement, for FR1, the MOP requirement of ULPFTx is introduced as below. Furthermore, based on the latest endorsed CR (R4-2115110), the requirement applicability for fallback DCI is also revised.

<from Section 6.2D.1 UE maximum output power for UL MIMO> For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1-1 shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. Table 6.2D.1-3: PUSCH Configuration for uplink full power transmission (ULFPTx) ULFPTx Mode; Transmission scheme; DCI format; Modulation; Number of layers; Number of Tx Port; TPMI index. Mode-1; Codebook based uplink; DCI format 0_1; DFT-s-OFDM, CP-OFDM NOTE3; 1; 2; 2. Mode-2; Codebook based uplink; DCI format 0_1; DFT-s-OFDM, CP-OFDM; 1; 2; 0 or 1NOTE2. Mode-full power; Codebook based uplink; DCI format 0_1; DFT-s-OFDM, CP-OFDM; 1; 2; 0,1. NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ULFPTxModes set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated.. If UE not -indicating Tx diversity [xx, TS38.306] is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

Observation-2: In the Section 6.2D.1 MOP requirement for UL-MIMO, there is not requirement applicable to the UE supporting Tx diversity scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission.

#### 2.2.1 ULFPTx Mode 1 (ul-FullPowerTransmission-r16 = ’fullpowerMode1’):

Specifically, the row of “Mode-1” in above configuration table for MOP, is corresponding to “ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1”:

- It should be noted that, the Mode-1 row in table is only corresponding to the case where Mode-1 capable UE is scheduled as “fullpowerMode1” in their PUSCH-config, but nothing related to Mode-1 capable UE is scheduled as “fallback DCI”:

  - Since transparent TxD is not yet supported in Rel-16 discussion, we can’t achieve the claimed power class by “fallback DCI” if all PAs are non-full-rated PAs, and RAN4 has a WF (R4-2008462) to clearly elaborate this problem:

  - For transparent TxD UE, it is needed (in this Rel-17 transparent TxD WI) to be discussed where/how the “fallback DCI” needs to be redirect; however, it is another story, which will be addressed in Section 2.3.

Observation-3: After TxD UE is introduced in Rel-17, MOP requirement of Rel-16 ULFPTx Mode-1 UE needs no revisit.

#### 2.2.2 ULFPTx Mode 2 (ul-FullPowerTransmission-r16 = ’fullpowerMode2’):

For the row of “Mode-2” in above configuration table for MOP, is corresponding to “ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2”,

- It should be noted that “Note-2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]” only corresponds to Mode-2 Mechanism-2 (TPMI indication), as cyan highlighted text copied from TS 38.213 in Section 2.1.

- In other words, Mechanism-1 of Mode-2 is not covered in MOP requirement, but it is intentional based on the following reason:

  - For UE supporting Mechanism-1 of Mode-2, the same MOP requirement can be covered as TxD UE:

    - E.g., for a 23dBm + 23dBm PC2 UE, whether or not it claims its support of ULFPTx Mode-2, it anyway needs to satisfy PC2 when “single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission”.

  - Therefore, it is not a problem at all, that current TS38.101 don’t explicitly list Mechanism-1 of Mode-2, because it is just UE’s requirement for UE’s RF capability, which is already covered by the fallback DCI, and we don’t need another conformance test dedicated for Mechanism-1.

Observation-4: After TxD UE is introduced in Rel-17, MOP requirement of Rel-16 ULFPTx Mode-2 UE needs no revisit.

#### 2.2.3 ULFPTx Mode 0 (ul-FullPowerTransmission-r16 = ’fullpower’):

For the row of “Mode 0”, TPMI 0 and 1 is used to achieve the full power transmission (with Rel-16 defined power scaling s =1). Because ULFPTx Mode 0 is proposed for the UEs with full-rated PAs on each TX chain, we don’t expect the MOP requirement for ULFPTx Mode 0 is related to Transparent TxD.

Observation-5: After TxD UE is introduced in Rel-17, MOP requirement of Rel-16 ULFPTx Mode-0 UE needs no revisit.

## 2.3 Expected MOP Requriement Impact for Fallback DCI

As mentioned above, for UE which needs transparent TxD to achieve full power, e.g., ULFPTx Mode 1 UE, and Mechanism-1 of ULFPTx Mode 2 UE, the MOP requirement needs to redirected to the section (that is suffix G based on current RAN4 discussion). However, the redirect is not only for Rel-16 ULFPTx UE, but also applied for a normal UL-MIMO supported UE which also rely on transparent TxD to achieve full power if scheduled by fallback DCI.

Observation 6: For ULFPTx Mode 1 UE and Mode-2 UE with Mechanism-1 (SRS port virtualization), if fallback DCI is scheduled, the MOP requirement needs to be redirected to suffix G to enable transparent TxD usage. But the same redirect is not only for ULPFTx but also for Rel-15 UL-MIMO UE which rely on transparent TxD.

Accordingly, we believe the current requirement in Section 6.2G.1 already works:

### 6.2G.1 UE maximum output power for Tx Diversity For UE supporting Tx Diversity, the maximum output power as indicated by UE power class in Table 6.2.1-1is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

Proposal 1: The MOP requirement for TxD UE (with or without ULFPTx configured) shall be referred to Section 6.2G.1.

## 2.4 Expected MPR Requriement Impact

In current ULFPTx MPR requirement introduced in Rel-16 eMIMO, the same MPR requirement introduced in 1TX is used, which is the same requirement used for normal Rel-15 UL-MIMO. For TxD UE, the MPR requirement should be provided in the Section 6.2G.2 instead, so the following changes is expected for UL-MIMO requirement (with or without ULFPTx):

### 6.2D.2 UE maximum output power reduction for UL MIMO For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. For UE not indicating Tx diversity [xx, TS 38.306], and supporting uplink full power transmission (ULFPTx) for UL MIMO, the allowed MPR for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply. If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

Proposal 2: For UE supporting TxD (with or without ULFPTx configured), MPR requirement specified in Section 6.2G.2 shall be applied, while the requirement in Section 6.2D.2 only apply for non-TxD UE.

# 3. Conclusion

In this contribution, we provide our analysis on the impact of introducing transparent TxD particularly on existing Rel-16 eMIMO uplink full power transmission (ULFPTx) related requirement, and accordingly the following proposals and observations are obtained:

Observation-1: Rel-16 ULFPTx feature can be categorized into Mode-0 (“fullpower” mode), Mode-1 and Mode-2, and in Mode-2 there are two mechanisms, i.e., Mechanism-1 for SRS port virtualization and Mechanism-2 for TPMI indication.

Observation-2: In the Section 6.2D.1 MOP requirement for UL-MIMO, there is not requirement applicable to the UE supporting Tx diversity scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission.

Observation-3: After TxD UE is introduced in Rel-17, MOP requirement of Rel-16 ULFPTx Mode-1 UE needs no revisit.

Observation-4: After TxD UE is introduced in Rel-17, MOP requirement of Rel-16 ULFPTx Mode-2 UE needs no revisit.

Observation-5: After TxD UE is introduced in Rel-17, MOP requirement of Rel-16 ULFPTx Mode-0 UE needs no revisit.

Observation 6: For ULFPTx Mode 1 UE and Mode-2 UE with Mechanism-1 (SRS port virtualization), if fallback DCI is scheduled, the MOP requirement needs to be redirected to suffix G to enable transparent TxD usage. But the same redirect is not only for ULPFTx but also for Rel-15 UL-MIMO UE which rely on transparent TxD.

Proposal 1: The MOP requirement for TxD UE (with or without ULFPTx configured) shall be referred to Section 6.2G.1.

Proposal 2: For UE supporting TxD (with or without ULFPTx configured), MPR requirement specified in Section 6.2G.2 shall be applied, while the requirement in Section 6.2D.2 only apply for non-TxD UE.

# 4. Reference

[1] RP-211597, “New WI: UE RF requirements for Transparent Tx Diversity (TxD) for NR”, Nokia, Qualcomm.