# 3GPP TSG-RAN WG4 Meeting #101-e	R4-2118275

# Electronic Meeting, 1st – 11th November, 2021

Source: 	vivo

Title: 	Analysis on handheld UE EIS requirements for 52.6~71 GHz

Agenda Item:	8.16.3.2

Document for:	Approval

Introduction

In the last meeting, peak EIS proposal was brought up for handheld UE type. However, since the antenna element number has not been decided yet, no further discussion on this topic during the meeting [1]. In this contribution, we give our analysis on handheld UE EIS requirements for 52.6~71 GHz based on 1x8 antenna element number assumption.

Discussion

We didn’t have deep discussion on EIS requirement in the last meeting since we haven’t reached a consensus on the assumption of antenna array size. For handheld UE EIS requirement, we had the following WF [1]:

- Handle peak EIS in future meetings. For one reason array sizes have not been settled, No further discussion this meeting.

In this contribution, we give the initial EIS analysis for handheld UE based on a feasible UE design. In our companion paper [2], we give our preference on the assumption of antenna element number 1x8, assuming it will strike a balance between complexity and performance. In the EIS analysis, we also use the same assumption to reach a value for handheld UE EIS.

In FR2-2, the receiver characteristics are specified over the air (OTA). The REFSENS is defined as EIS level in the Rx beam peak direction. The REFSENS can be calculation as:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +Total Insertion loss – Antenna Array Gain

Based on 1x8 antenna element number assumption, we performed Rx link budget analysis for 52.6~71 GHz as shown in Table 1. We assume these values for the parameters in Table 1 based on a feasible design for handheld UE.

Table 1. Rx link budget analysis for FR2-2

| Parameter | Unit | Proposal |
| --- | --- | --- |
| Frequency range | GHz | 52.6 – 71 |
| SNR Requirement | dB | -1 |
| Bandwidth | MHz | 100 |
| Thermal Noise | dBm/Hz | -174 |
| Noise Figure | dB | 18 |
| # of antennas in an array | / | 8 |
| Average antenna element gain | dBi | 4 |
| Realized antenna array gain | dBi | 8.5 |
| Diversity Gain | dB | 0 |
| Antenna gain roll-off loss versus frequency | dB | 1.5 |
| Mismatch and transmission line loss | dB | 4 |
| Beamforming Loss | dB | 3.5 |
| Form factor Integration losses | dB | 7 |
| Total Insertion Loss | dB | 14.5 |
| REFSENS | dBm | 74 |

We can observe from Table 1 that the EIS level for 100MHz in 52.6~71 GHz is 74 dBm/100MHz.

Observation 1: The single-band peak EIS requirement for 52.6~71 GHz is 74 dBm/100 MHz based on 1x8 antenna element number assumption.

Conclusion

This contribution gives an initial analysis on UE EIS requirement for 52.6~71 GHz. The following observation is made:

Observation 1: The single-band peak EIS requirement for 52.6~71 GHz is 74 dBm/100 MHz based on 1x8 antenna element number assumption.

- References

[1] R4-2114989, WF on 60 GHz UE RF requirements, Qualcomm Inc, RAN4#100-e.

[2] R4-2118274, Analysis on handheld UE EIRP and spherical coverage requirements for 52.6~71 GHz, vivo, RAN4#101-e.