# 3GPP TSG-RAN WG4 Meeting #101-e	R4-2118276

# Electronic Meeting, 1st – 11th November, 2021

Source: 	vivo

Title: 	Coexistence simulation results for 52.6~71 GHz

Agenda Item:	8.16.5

Document for:	Approval

Introduction

In the last meeting, it was agreed to focus on indoor deployments for coexistence simulation [1]. And the simulation assumptions were updated in the Annex. Just before RAN4#101-e meeting, the calibration work for coexistence simulation was finished among companies. In this contribution, we further provide our simulation results for co-existence study of 60/70 GHz.

Discussion

Simulation Description

The co-existence simulation scenarios for 52.6~71 GHz are summarized in Table 1. The deployment scenarios in this simulation are indoor office C in TR 38.808. The simulation includes 60/70 GHz frequency with 100/400 MHz channel bandwidth. In the simulation, both UL and DL co-existence are considered. The simulation assumptions are captured in Table 3. The simulation methodology follows the description in TR 38.803.

Table 1: Summary of initial simulation scenarios for B52.6G

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 60 GHz | Indoor office |
| 2 | eMBB | NR, 400MHz | NR, 400MHz | DL to DL | 60 GHz | Indoor office |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 70 GHz | Indoor office |
| 4 | eMBB | NR, 400MHz | NR, 400MHz | DL to DL | 70 GHz | Indoor office |

Uplink throughput results

For indoor office C scenario, the UL-UL coexistence simulation results are shown in Figure 1. The edge loss and average loss of UL throughput for indoor scenarios are shown in Figure 1a) and 1b), respectively. It can be seen in this table, ACIR of 20 dB value can keep the edge loss of UL throughput under 5% and the value of 8 dB can meet the threshold of average 5% UL throughout loss.

| a) Edge loss of UL throughput for indoor deployments | b) Average loss of UL throughput for indoor deployments |
| --- | --- |

Figure 1. Uplink throughput results for indoor deployments

Observation 1: For UL, ACIR value of 20 dB can meet the 5% edge throughput loss for 52.6~71 GHz for indoor office scenario.

Observation 2: For UL, ACIR value of 8 dB can meet the 5% average throughput loss for 52.6~71 GHz for indoor office scenario.

Downlink throughput results

For indoor office C scenario, the DL-DL coexistence simulation results are shown in Figure 2. The edge loss and average loss of DL throughput for indoor scenarios are shown in Figure 2a) and 2b), respectively. It can be seen in this table, ACIR of 26 dB value can keep the edge loss of DL throughput under 5% and the value of 16 dB can meet the threshold of average 5% DL throughout loss.

| - Edge loss of DL throughput for indoor deployments | - Average loss of DL throughput for indoor deployments |
| --- | --- |

Figure 2. Downlink throughput results for indoor deployments

Observation 3. For DL, ACIR value of 26 dB can be sufficient to meet the edge 5% throughput loss.

Observation 4. For DL, ACIR value of 16 dB can be sufficient to meet the average 5% throughput loss.

Conclusion

This contribution provides further co-existence simulation results for carrier frequency 60/70 GHz with 100/400 MHz channel bandwidths in indoor office scenario. The co-existence simulation results are summarized in Table 2.

Table 2. Summary of coexistence simulation results

|  | Indoor office (60GHz) |  |  |  | Indoor office (70GHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100M |  | 400M |  | 100M |  | 400M |  |
| DL: ACIR (5% TP loss) | 15.58 (Avg.) | 25.51 (Edge) | 15.00 (Avg.) | 22.36 (Edge) | 15.15 (Avg.) | 22.00 (Edge) | 14.74 (Avg.) | 21.95 (Edge) |
| UL: ACIR (5% TP loss) | 7.34 (Avg.) | 18.95 (Edge) | 7.82 (Avg.) | 16.25 (Edge) | 7.18 (Avg.) | 19.58 (Edge) | 8.00 (Avg.) | 15.58 (Edge) |

The following observations are made:

Observation 1: For UL, ACIR value of 20 dB can meet the 5% edge throughput loss for 52.6~71 GHz for indoor office scenario.

Observation 2: For UL, ACIR value of 8 dB can meet the 5% average throughput loss for 52.6~71 GHz for indoor office scenario.

Observation 3. For DL, ACIR value of 26 dB can be sufficient to meet the edge 5% throughput loss.

Observation 4. For DL, ACIR value of 16 dB can be sufficient to meet the average 5% throughput loss.

- References

[1] R4-2114993, WF on co-existence simulation for NR_ext_to_71GHz, Qualcomm, RAN4#100-e.

- Annex

Table 3: List of coexistence simulation parameters

| System Parameters | Deployment | Indoor office C in TR 38.808 (optional: Indoor office A in TR 38.808). Dense urban scenario A in TR 38.808 with ISD = 30 meters |
| --- | --- | --- |
|  | Carrier Frequency | 60 GHz, 70 GHz |
|  | Channel BW | 100Mhz and 400MHz |
|  | SCS | 120KHz for 100MHz and 960KHz for 400MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office model in TR 38.901. Umi model in TR 38.901 |
|  | LBT | No LBT considered (optional: consider LBT) |
|  | UE to BS 2D distance | 0 meters for indoor deployment |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) for indoor deployment (1,1,16,16,2) for dense urban deployment |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Indoor: Table A.2.1-7 in TR 38.802 for ceiling mount UMi: Table 7.3-1 in TR 38.901 |
|  | EIRP limit | 40 dBm for indoor deployment 52.8 dBm for dense urban deployment |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Indoor and UMi: Table A.2.1-8 in TR 38.802 |
|  | EIRP limit | 20 dBm |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability model defined in TR 38.803 |
|  | Maximum conducted power (per polarization) | 0 dBm |
|  | Minimum conducted power (per polarization) | -20 dBm |
|  | SNR target | 15 dB |
|  | CL-xile | 66 for 100 MHz and 60 for 400 MHz. |