# 3GPP TSG-RAN WG4 Meeting #101-e	R4-2118281

# Electronic Meeting, 1st – 11th November, 2021

Source: 	vivo

Title: 	Discussion on con-current band combinations for SL HPUE

Agenda Item:	7.24.1

Document for:	Approval

Introduction

SL HPUE for inter-band con-current band combination is identified as the Rel-16 left over issues [1], which is treated in Rel-17. There are extensive band combinations for inter-band con-current band combinations introduced in Rel-16 and Rel-17. However, the specific band combinations for SL HPUE has not been discussed. In this contribution, we discussed the example band combinations for supporting SL HPUE.

Discussion

Introducing HPUE for inter-band con-current band combinations is a Rel-16 leftover issue [1], which is treated in Rel-17. If Rel-17 does not introduce a band combination for SL HPUE, it will be left to Rel-18. So, we suggest to at least introduce at least one example band combinations for supporting HPUE.

In Rel-17, the inter-band con-current band combinations [2] are as follows:

Table 1-1: Release 17 NR V2X band combinations

| V2X Band combination | REL independent from |
| --- | --- |
| V2X_n39-n47 | Rel-16 |
| V2X_n40-n47 | Rel-16 |
| V2X_n41-n47 | Rel-16 |
| V2X_n78-n47 | Rel-16 |
| V2X_n79-n47 | Rel-16 |
| V2X_n39_47 | Rel-16 |
| V2X_n40_47 | Rel-16 |
| V2X_n41_47 | Rel-16 |
| V2X_n78_47 | Rel-16 |
| V2X_n79_47 | Rel-16 |
| V2X_3_n47 | Rel-16 |
| V2X_39_n47 | Rel-16 |
| V2X_40_n47 | Rel-16 |
| V2X_41_n47 | Rel-16 |

Note: All band combinations in table 1-1 that are release independent from Rel-16 are optional

These inter-band con-current band combinations can be summarized as these patterns:

- NR band nX + NR V2X band n47

- NR band nX + LTE V2X band 47

- LTE band nX + NR V2X band n47

Band n47 already supports HPUE [3], according to the progress of the WI SL enhancement. However, the power class for the inter-band combinations can only be PC3, which means the total maximum output transmission power for SL and Uu can reach to 23 dBm. However, RAN4 agreed to introduce HPUE for inter-band con-current operation. So, we suggest to introduce example band combinations V2X_nX_n47 for SL HPUE. Operators can make the request for inter-band con-current HPUE according to the example band combinations.

Proposal 1: Introduce example inter-band con-current combinations V2X_nX_n47 for SL HPUE.

Proposal 2: Collect the inter-band con-current combination request from operators for SL HPUE in Rel-17.

Conclusion

This contribution discusses example inter-band con-current band combinations for supporting SL HPUE. The following proposals are made:

Proposal 1: Introduce example inter-band con-current combinations V2X_nX_n47 for SL HPUE.

Proposal 2: Collect the inter-band con-current combination request from operators for SL HPUE in Rel-17.

- References

[1] TR 38.785 V0.3.0

[2] R4-2111950, TR 37.875, Band combinations for V2X con-current operation, CATT, RAN4#100-e

[3] R4-2114985, Way forward on PC2 NR V2X, Huawei, HiSilicon, RAN4#100-e.