# 3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2118288

Electronic Meeting, Nov. 1st – 12th, 2021

Source: 	vivo

Title: 	Further discussion on impact of multi-panel reception requirements

Agenda Item:	8.19.2.1

Document for:	Discussion

Introduction

The RAN4 impact of further enhancements on MIMO has been discussed in RAN4#100-e. The discussion for multi-panel reception requirements were discussed and documented in the Moderator’s summary [1]. During the discussion, most companies believe that there is no impact for the minimum requirements. However, there is still different view on this and in the approved WF [2], there are following agreements:

- RAN4 will further determine if additional requirement is required for the multi-panel reception

- The RF impact for the multi-panel UE will be discussed in in RAN4#101-e based on the contribution

  - Aim is to decide on the impact in RAN4#101-e given the time limitation

RAN1 do not have new agreements yet on this topic, and in previous meeting, the [3] already provides a detailed analysis on this with simulation results. In this contribution, the analysis of [3] have been re-submitted with certain updates, new observations based on RAN1’s progress is also supplemented.

Discussion

The simultaneous multi-TRP transmission with multi-panel reception was introduced according to WID [4].

The main receiver requirements are briefly listed as following:

- Reference sensitivity power level

- EIS spherical coverage

- Maximum input level

- Adjacent channel selectivity

- Blocking characteristics

- Spurious emissions

For reference sensitivity level, currently it is based on the peak performance of one panel. Since typical multi-panel implementation is targeted to improve spherical coverage performance rather than peak performance, it is not likely for a UE’s peak performance, no matter Tx or Rx, to be impacted by simultaneous operation.

Observation 1: The reference sensitively power level is not likely to be improved by multi-panel reception, since multi-panel implementation is targeted for spherical coverage.

It seems in the first stage that the EIS spherical coverage is the most likely to be impacted, since multi-panel reception would involve some combination and diversity between the two panels for some scenario. However, it is also possible that this combining effect could be minimum.

In order to study this effect, a simulation was done for spherical coverage in [3] and re-submitted as following:

In the simulation, two panel Tx spherical coverage using combination and selection were simulated respectively, and since the Tx are Rx are symmetrical, the Tx spherical coverage can also show the effect of EIS spherical coverage. The assumptions are given in following table.

Table 1. Assumptions for Spherical coverage comparison

|  | Assumption 1 | Assumptions 2 |
| --- | --- | --- |
| Frequency range | n257 | N257 |
| Antenna element | 1×4 | 1×4 |
| Antenna module in total | 2 | 2 |
| Finite UV test points | Y | Y |
| Beam phase shifter controller | 45° | 45° |
| Antenna module/set location | Top & Bottom | Top & Bottom |
| Front cover | Glass | Glass |
| Back cover | Glass | Plastic |
| Side cover | Metal | Plastic |

Figure 1. Antenna module location

In the simulation, two panels are simulated, which are located at the top and bottom respectively. Different Back/Side cover are also simulated as shown in Assumption 1/2 in above table. In each assumption, the spherical coverages are compared between the combination of 2 panels and selection of 1 panel for operation. The simulation results are depicted in figures 2 and 3.

Figure 2. Spherical coverage comparison for Assumption 1 (Back/Side: Glass/Metal)

Figure 3. Spherical coverage comparison for Assumption 2 (Back/Side: Plastic / Plastic)

It can be seen that for assumption 1, the spherical coverage almost stays the same with the introduction of 2 panel simultaneous combination, while for assumption 2 there is 1~2dB performance gain at 50 percentile or lower. It may be implied that the glass/metal made the combination of signals for those two panels more difficult, at least under this layout assumption.

Here is an updated observation with clearer wording compared to:

Observation 2 (Supplemented): According to simulation, the gain introduced by simultaneous operation and combination of 2 panels for spherical coverage can be negligible, compared to active one panel at a time. Even in more favourable (not likely typical) condition e.g. plastic back/side, the performance difference can still be small, e.g. 1dB etc.

Furthermore, one panel operation can always be the baseline, and there is no way that simultaneous operation has worse performance. So, it is also safe to reuse current minimum requirements for multi-panel reception.

Observation 3: It is also safe to reuse current minimum requirements for multi-panel reception for EIS spherical coverage since it is the baseline performance.

For other receiver requirements, they do not have much relation with multi-panel reception.

Observation 4: Other receiver requirements are not impacted by multi-panel reception.

Here is a new observation based on more analysis on RAN1’s situation. Multi-panel simultaneous Rx is non-configurable, and no agreements have been made on whether the status would be reported or not. According to currently status, it is highly likely RAN1 would keep the status in Rel-17. This would be quite challenging for

Observation 5 (New): Multi-panel simultaneous Rx is non-configurable, and even likely not to be reported which is still not confirmed yet. Thus make the testability quite challenging even if some merits of testing can be found.

Based on previous observations, the following proposal is provided:

Proposal: Do not define requirements for UE supporting multi-panel simultaneous reception.

Conclusion

In this paper, the multi-panel reception impact to the UE reception requirements were analysed. Some of the contents in last meeting have been resubmitted with update:

Observation 1: The reference sensitively power level is not likely to be improved by multi-panel reception, since multi-panel implementation is targeted for spherical coverage.

Observation 2 (Supplemented): According to simulation, the gain introduced by simultaneous operation and combination of 2 panels for spherical coverage can be negligible, compared to active one panel at a time. Even in more favourable (not likely typical) condition e.g. plastic back/side, the performance difference can still be small, e.g. 1dB etc.

Observation 3: It is also safe to reuse current minimum requirements for multi-panel reception for EIS spherical coverage since it is the baseline performance.

Observation 4: Other receiver requirements are not impacted by multi-panel reception.

Observation 5 (New): Multi-panel simultaneous Rx is non-configurable, and even likely not to be reported which is still not confirmed yet. Thus make the testability quite challenging even if some merits of testing can be found.

Based on previous observations, the following proposal is provided:

Proposal: Do not define requirements for UE supporting multi-panel simultaneous reception.

- References

[1] R4-2115041, Email discussion summary for [100-e][141] NR_feMIMO, Samsung, RAN4#100-e

[2] R4-2114994, WF on FeMIMO impacts to RF requirements, Samsung, RAN4#100-e

[3] R4-2113016, Discussion on impact of multi-panel reception requirements, vivo, RAN4#100-e

[4] RP-202024, Revised WID: Further enhancements on MIMO for NR, RAN 89-e