3GPP TSG-RAN WG4 Meeting #101-e                                                            R4-2118331

# Electronic Meeting, November 01-12, 2021

Agenda Item:	8.13.5.4

Source:	MediaTek Inc.

Title:	Discussion on timing requirements in NTN

Document for:		Discussion

Introduction

In this paper, NTN RRM timing requirements are discussed, including the NTN timing error (Te,NTN) requirement, common TA, gradual timing adjustment requirements, and TA adjustment accuracy requirement.

NTA,common

- FFS whether to define a separate accuracy requirement for self-estimated TA common (NTA,common).

- FFS whether to define a separate accuracy requirement for the combination of  NTA,UE-specific+NTA,common.

The common TA NTA,common was also discussed in the last meeting [1].

The corresponding RAN1 agreement for NTA,common is also attached below as a reference.

In our understanding, the NTA,common is indicated by network for the timing delay introduced by feeder link. Since NTA,common is indicated by network and no UE self-estimating is needed, it seems similar to the NTA,offset. In other words, UE can determine the timing based on (NTA,common  + NTA,offset), and thus the requirement of NTA,common can be captured and tested in the Te,NTN requirement, as the approach of NTA,offset.

Observation 1: NTA,common  and NTA,offset  are both network-controlled and not UE self-estimated.

Proposal 1: Similar to NTA,offset, NTA,common can be covered in total advanced timing and be tested with the timing error limit Te,NTN,

NTN UE initial transmit timing error

- For initial transmit timing requirement in NTN (Te_NTN), Te_NTN = Te + Te_GNSS + Te_SAT

  - Te is the legacy timing error

  - Te_GNSS is the GNSS accuracy

    - Note: Te_GNSS shall include the total RTT error

      - FFS the clarification on total RTT error

    - FFS how to derive Te_GNSS from the GNSS positioning accuracy

  - Te_SAT is the serving-satellite position estimation error

    - Note: Te_SAT shall include the total RTT error

      - FFS the clarification on total RTT error

  - FFS if the equation shall be included into the specification or only Te_NTN values shall be included

- GNSS accuracy assumption for timing requirements

  - For UL SCS = 15 kHz and 30 kHz: 2-D position error is 50m

  - For UL SCS = 60kHz in FR1: FFS

In last meeting, the considerations of initial transmit timing requirement in NTN (Te,NTN) and the GNSS position error assumption were discussed in WF [1].

Te_GNSS and Te_SAT are further discussed in this section.

Te_GNSS:

For one-way, timing error due to the 2-D error of 50 m is 5.12 Ts (50 m / 10^8 m/s). Thus, for RTT, it contributes 5.12 Ts x 2  = 10.24 Ts.

Te_SAT:

For one-way, we assume the positions error of 3.87 m, by referencing (R1-2106556 [2]), attached below for reference:

As can be seen in the following table: By taking into account propagation model errors, satellite position can be predicted 60 seconds ahead with only max error of 1.47m and 5 minutes ahead with max error of 3.87m in case of  initial 3D Position RMS Error = 0.5 m and 3D Velocity RMS Error = 0.5 mm/s (Typical” Precision Orbit Determination used for telecommunication satellites).

Thus, for RTT, it contributes 0.4 Ts x 2 = 0.8 Ts.

Other factors and margin:

We assume 2 Ts error margin for UE self-estimation error in service link (UE specific TA, excluding GNSS), applying common TA parameters.

As a result, we propose Te_NTN = Te + Te_GNSS (10.24 Ts) + Te_SAT (0.8 Ts) + margin (2 Ts) = Te + [13] Ts.

It can also be observed that the maximum timing difference between 2 UEs can be 2*Te_NTN + 7.3 Ts (245 ns max delay spread in FR1), assuming one UE with + Te_NTN  and the other UE with - Te_NTN . As shown in Table 1, the maximum timing difference between 2 UEs can be within the CP duration of UL with SCS = 15 kHz (34-40% CP) and with SCS of 30 kHz (69-74% CP). However, for UL with SCS = 60 kHz, the maximum timing difference between 2 UEs will exceed the CP duration (132%-148%) and thus it may need more discussion for the case of 60 kHz.

Table 1:  The maximum timing difference between 2 UEs

| SSB SCS | UL SCS | UL CP |  |  | TN Te |  |  | Te_GNSS | Te_SAT | margin | delay spread |  | Max. timing diff. b/w 2 UEs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kHz | kHz | us | Ts |  | Ts (64Tc) | CP% |  | Ts | Ts | Ts | Ts |  | Ts | CP% |
| 15 | 15 | 4.7 | 144 |  | 12 | 8% |  | 10.24 | 0.8 | 2 | 7.3 |  | 57 | 40% |
|  | 30 | 2.3 | 72 |  | 10 | 14% |  |  |  |  |  |  | 53 | 74% |
|  | 60 | 1.2 | 36 |  | 10 | 28% |  |  |  |  |  |  | 53 | 148% |
| 30 | 15 | 4.7 | 144 |  | 8 | 6% |  |  |  |  |  |  | 49 | 34% |
|  | 30 | 2.3 | 72 |  | 8 | 11% |  |  |  |  |  |  | 49 | 69% |
|  | 60 | 1.2 | 36 |  | 7 | 19% |  |  |  |  |  |  | 47 | 132% |

Proposal 2: For UL SCS = 15 kHz and 30 kHz, Te_NTN = Te + [13] Ts. FFS UL SCS = 60 kHz.

NTN UE gradual timing adjustment requirements

- UE behaviour for gradual timing adjustment for NTN UE

  - Option 1: UE performs timing adjustment for downlink reception timing drifting and UE specific TA change separately.

  - Option 2: UE performs timing adjustment with combining downlink reception timing drifting and UE specific TA change as one adjustment.

To specify the UE requirement regarding the gradual timing adjustment for the NTN UE, the UE behaviour should be clarified first. From the last meeting [1], two behaviours are listed for discussion.

To our understanding, the legacy gradual timing adjustment is to require UE to adjust timing when “DL timing of the reference cell is changed”. However, in NTN UE, UE is also required to adjust timing as UE pre-compensation for UE-specific TA, based on the updated UE position and satellite position. If UE has performed UE pre-compensation for UE-specific TA, then the definition of “DL timing of the reference cell is changed” is unclear.

Proposal 3: When UE has performed UE pre-compensation for UE-specific TA, the amount of gradual timing adjustment should be clarified.

To our understanding, when UE transmits UL transmission, it could not separate (1) the downlink reception timing drifting and (2) UE specific TA change. Our understanding on (1) and (2) are provided below.

(1) downlink reception timing drifting and gradual timing adjustment requirement

In TN, it is handled by the gradual timing adjustment requirement.

As shown in Figure 1, when the delay is increased by ∆D due to the longer distance, UE applies the ∆D for UL transmission, the timing received at satellite/gNB will be 2 x ∆D, because the travelling time for UL will also be increased by ∆D.

Figure 1 UE behaviour of Gradual timing adjustment in TN

(2) UE specific TA change

In NTN, when the delay is increased by ∆D, and UE will be required to pre-compensate this increase, in the opposite direction to the predicted reception timing drifting.

Figure 2: UE behaviour of timing pre-compensation in NTN

Based on our understanding, when UE transmit UL transmission by applying the UL timing based on (1) and (2), the (1) and (2) will be combined and couldn’t be separated.

Proposal 4: UE performs timing adjustment with combining downlink reception timing drifting and UE specific TA change as one adjustment. (Option 2)

Since the downlink reception timing drifting for gradual timing adjustment requirement and UE specific TA change cannot be separated, then one single requirement will be sufficient. Otherwise, two separate requirements both cover the downlink reception timing drifting will cause unnecessary confusion.

Observation 2: Unnecessary confusion will be caused if the downlink reception timing drifting is coved by both gradual timing adjustment requirement and UE specific TA requirement.

Proposal 5: In NTN, gradual timing adjustment requirement can be replaced by UE specific TA requirement, which can be verified by the timing error limit Te,NTN.

Observations on timing drift in GEO, LEO and R15 gradual timing adjustment rate

The timing delay variation (drifting) in NTN is given in Table 2, according to TR 38.811 [3], and the R15 timing adjustment rate is given in Table 3. It can be observed a big gap between the timing drift in NTN LEO (25 us/sec) and the R15 max timing adjustment rate (0.9 us/sec).

Table 2:  Timing Drift in NTN

|  | GEO satellite at 35786 km | LEO at 600 km | LEO at 1500 km |
| --- | --- | --- | --- |
| Satellite speed | Negligible | 7.6 km/s | 7.1 km/s |
| Timing drift | Negligible | 0.5 us/20ms 2 us / 80ms 25 us/sec | 0.47 us/20ms 1.88 us/80ms 23 us/sec |

Table 3:  R15 gradual timing adjustment rate

| Tq and Tp (Table 7.1.2-3) |  | Min Rate | Max Rate |
| --- | --- | --- | --- |
| FR1 | 5.5 *64*Tc | 180 ns/sec | 0.9 us/sec |
| FR2 | 2.5 *64*Tc | 1.6 ns/20ms 80 ns/sec | 0.4 us/sec |

Therefore, it seems not practical to rely on the existing gradual timing adjustment to cope with the large timing drift of LEO. At least the timing adjustment will not be gradual, and it may cause reception problem at the satellite side.

Observation 3: Timing drift in GEO is negligible. It will have less concern to apply the legacy gradual timing adjustment requirement.

Observation 4: Timing drift in LEO is significant. UE pre-compensation for the UE specific TA change will dominate the timing adjustment. Large timing change at satellite may cause UL reception problem.

TA adjustment accuracy requirements

TA adjustment accuracy requirement in RRC_CONNECTED mode

- Option 1: Reuse the existing timing advance adjustment accuracy requirements defined in TS 38.133.

- Option 2: TA adjustment accuracy requirement can be defined as existing timing advance adjustment accuracy requirements defined in TS 38.133 if UE specific TA is not changed, otherwise plus delay error related with nominal accuracy of GNSS, i.e. 30m.

- Option 3: Further evaluate the TA adjustment accuracy requirement after RAN1 achieve the agreement about the timing relationship of TA command.

- Option 4: RAN4 is to define a relaxed TA adjustment accuracy requirement for NR NTN

The TA adjustment accuracy requirement in RRC_CONNECTED mode has been discussed in the last meeting [1], regarding whether the UE position and satellite position estimation error should be accounted, some options are listed:

Our view is close to Option 2. Since NTN UE keeps performing UE pre-compensation for UE-specific TA (i.e. open loop TA), the accuracy could be impacted by UE position and satellite position estimation error. However, if it assumes the UE specific TA is not changed, which can also be controlled in the tests, then the additional error budget due to UE specific TA is not needed, and it can only consider the UL granularity for the accuracy requirement.

Proposal 6: TA adjustment accuracy requirement can be defined as the existing timing advance adjustment accuracy requirements defined in TS 38.133 if UE specific TA is not changed.

Conclusion

In this contribution, we summarize issues and discuss impact on specifications for solutions for UE UL transmit timing requirement.

Observation 1: NTA,common  and NTA,offset  are both network-controlled and not UE self-estimated.

Proposal 1: Similar to NTA,offset, NTA,common can be covered in total advanced timing and be tested with the timing error limit Te,NTN,

Proposal 2: For UL SCS = 15 kHz and 30 kHz, Te_NTN = Te + [13] Ts. FFS UL SCS = 60 kHz.

Proposal 3: When UE has performed UE pre-compensation for UE-specific TA, the amount of gradual timing adjustment should be clarified.

Proposal 4: UE performs timing adjustment with combining downlink reception timing drifting and UE specific TA change as one adjustment. (Option 2)

Observation 2: Unnecessary confusion will be caused if the downlink reception timing drifting is coved by both gradual timing adjustment requirement and UE specific TA requirement.

Proposal 5: In NTN, gradual timing adjustment requirement can be replaced by UE specific TA requirement, which can be verified by the timing error limit Te,NTN.

Observation 3: Timing drift in GEO is negligible. It will have less concern to apply the legacy gradual timing adjustment requirement.

Observation 4: Timing drift in LEO is significant. UE pre-compensation for the UE specific TA change will dominate the timing adjustment. Large timing change at satellite may cause UL reception problem.

Proposal 6: TA adjustment accuracy requirement can be defined as the existing timing advance adjustment accuracy requirements defined in TS 38.133 if UE specific TA is not changed.

- References

- R4-2115346, WF on GNSS-related and timing requirements for NR NTN, Xiaomi.

- R1-2106556, Considerations on UL timing and frequency synchronization in NTN, Thales.