3GPP TSG-RAN WG4 #101-e	R4-2118353

Electronic meeting, 1st – 12th Nov, 2021

Source: 	Ericsson

Title:  	Active BWP switching delay requirements for extending NR operation to 71GHz

Agenda Item:	8.16.7.4

Document for:	Discussion

# Introduction

The WF for extending current NR operation to 71 GHz [1] was approved in RAN#100-e meeting.  In this contribution we present some proposals on general requirements.

# Disucssion

The issue related to BWP switching delay for FR2-2 was agreed in below:

BWP switching delay for FR2-2 - DCI and timer based BWP switch delay on a single CC - As baseline, follow 600us and 2000us switching delay for Type 1 and Type 2 respectively - FFS: if BWP delay reduction is possible for the operation on 480 and 960 kHz SCS - RAN4 to change Table 8.6.2-1 as follows Table 8.6.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 4; 0.03125; [20]; [65]. 5; 0.015625; [39]; [129]. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. Note: TBWPswitchDelay is the BWP switching delay on single CC - RRC-based BWP switch - Reuse the existing requirements for RRC-based BWP switch - BWP Switching on Multiple CCs - Postpone the discussion for the later stage after requirements for one BWP will be agreed

Proposal  1:  BWP switch delay shall follow the agreements on last meeting, it can be subject to change later once BWP delay reduction is feasible from hardware design perspective.

Proposal  2:  BWP Switching on Multiple CCs shall follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133.

# Conclustion

Proposal  1:  BWP switch delay shall follow the agreements on last meeting, it can be subject to change later once BWP delay reduction is feasible from hardware design perspective.

Proposal  2:  BWP Switching on Multiple CCs shall follow chapter 8.6.2A, chapter 8.6.2B and Chapter 8.6.3A in TS38.133.

# References

[1] R4-2115352 - WF on NR extension to 71 GHz – RRM - Part 2, Qualcomm