3GPP TSG-RAN WG4 Meeting #101-e	R4-2118388

Electronic Meeting, November 01-12, 2021

Agenda item:	8.11.2.3

Source: 	OPPO

Title: 	On NCSG for NR_MG_enh

Document for:	Discussion

Introduction

A WF on NCSG has also been approved in RAN4 #100-e meeting [1]. In this contribution, we provide our view on the remaining issues of Network Controlled Small Gap (NCSG) for RRM measurement gap enhancement in R17.

Discussion

- Issue 1: NCSG applicability and signalling

- Open issues: - FFS: Existing gap applicability in Rel-16 for NR-only measurements and mandatory gap patterns is re-used for NCSG capable UEs. - FFS: Related to NCSG applicability and UE capability support: if UE supports NCSG, it is mandated to support MG patterns from per-UE gap patterns #0, #1, #4-9 for NCSG usage. In case UE supports NCSG and per-FR gap patterns, indicating this via capability, it is mandated to support also per-FR gap patterns #12-19 in FR2 for NCSG usage. - Agreement: - No need to introduce NCSG patterns corresponding to legacy MG patterns #24 and #25. - It is FFS whether to introduce NCSG patterns with longer repetition periodicity (>160ms). - Define NCSG patterns corresponding to legacy patterns #0~#23 - FFS how to indicate the support of NCSG patterns - A subset of mandatory NCSG patterns for UEs supporting NCSG will be defined. FFS on the set of mandatory NCSG patterns.

According to the agreements in the last meeting, NCSG patterns defined in Rel-17 will correspond to a legacy MG pattern. Based on the existing UE capabilities on the support of MG patterns and the support of per-FR MG, we think NCSG pattern can follow the applicability of corresponding MG patterns and no additional signalling is needed. For example, per-FR NCSG is supported if per-FR MG is supported by the NCSG capable UE. And if MG pattern #5 is supported for a NCSG capable UE, then the NCSG patterns corresponding to MG pattern #5 are also supported. Naturally, the NCSG patterns corresponding to MG patterns are also mandatory.

Proposal 1: For a NCSG capable UE, no additional signalling is needed to indicate support the NCSG pattern and per-FR NCSG capability.

- Per-FR NCSG is supported if per-FR MG is supported

- The NCSG patterns corresponding to the supported MG patterns are also supported.

- The NCSG patterns corresponding to the mandatory MG patterns are also mandatory.

- How to indicate the support of NCSG pattern - Option 1: introduce new signaling (separately from NeedForGap) to indicate support of NCSG. - Option 2: introduce new element in NeedForGap to indicate support of NCSG. - Option 3: introduce new signaling (separately from NeedForGap) to indicate support of following cases - Case 1: gap - Case 2: no-gap-with-interruption (or NCSG) - Case 3: no-gap-no-interruption - Option 4: up to RAN2.

In our view, the signalling of NR NSCG may be a bit different from that for LTE NCSG. The “NeedForGap” signalling structure can be reused for NR NCSG as a starting point. Since no reserved bits for extension in “NeedForGap” signalling, option 2 should not be considered. Any other signalling structures are acceptable for us as long as the three cases in option 3 can be supported. These functions required by RAN4 could be achieved by a new signalling alone as proposed by option 3, or by the combination of new signalling and “NeedForGap” signalling. The details regarding how many elements for the new signalling or the relation between NCSG signalling and legacy “NeedForGap” could be left for RAN2 decision.

Proposal 2: New signalling separate from “NeedForGap” is introduced to indicate the support of NCSG.

- Issue 2: NCSG pattern

- Issue 2-2: whether to define separate NCSG patterns for sync and async - Option 1: yes - Option 2: no - Option 3: different patterns in FR1 but same patterns in FR2 - Option 4: FFS - Issue 2-3: whether to consider VIL as a part of NCSG pattern - Option 1: - NOT consider VIL as a part of NCSG pattern, i.e. only keep measurement length and repetition periodicity in the pattern design, and capture VIL separately as interruption requirements (similar to Table 9.1.2-4 in TS38.133). - Option 2: - Keep VIL as a part of the NCSG pattern. - Issue 3-1: whether to replace VIL (visible interruption length) with RRT (RF retuning time) - Option 1: Yes. Introduce absolute RRT to replace VIL. - Option 2: VIL and RRT can be defined separately. - Option 3: only capture VIL in RAN4 spec. RRT can be used to calculate ML in discussion. But no need to capture RRT in RAN4 spec.

In LTE, at most 2 NCSG patterns are defined for each MG pattern, and one MG pattern can be associated with multiple NCSG patterns with different VIL requirements. However, in NR whether VIL is considered as a part of NCSG pattern (issue 2-3 and issue 3-1) is not clear. If VIL is kept as a part of NCSG pattern, then more NCSG pattern IDs are needed due to different VIL length for the synchronous case and asynchronous case. If only MG and periodicity are used to define NCSG pattern, or if VIL is replaced by absolute RTT, then single NCSG pattern could be defined for each MG pattern. To avoid redundant NCSG patterns corresponding to up to 24 MG patterns, we think the interruption requirements should be defined separately and not be kept as a part of NCSG pattern.

Observation 1: Whether separate NCSG patterns are needed for synchronous and asynchronous cases depends on whether VIL is considered as a part of NCSG pattern.

Proposal 3: Not consider VIL as a part of NCSG pattern.

Proposal 4: Not define separate NCSG patterns for synchronous and asynchronous cases.

- Relation between ML, MGL and RRT - Option 1: ML = MGL – 2*RRT - Option 1a: ML = MGL – 2*RRT and ML + VIL1 + VIL2 > MGL - Option 1b: ML = MGL – 2*RRT and ML + VIL1 + VIL2 > MGL, if VIL is defined as the number of interrupted slots - Option 1c: ML = MGL - RRT1 - RRT2 - Option 2: ML + VIL1 + VIL2 = MGL - Option 2a: ML = MGL – VIL1 – VIL2, if VIL is defined as the absolute time - Option 3: - Step 1: Define MLNCSG from legacy gap patterns by MLNCSG = MGLlegacy – 2*RRTlegacy, e.g., - Gap patterns 0-11: MLNCSG = MGLlegacy – 1 (ms) - Gap patterns 12-23: MLNCSG = MGLlegacy – 0.5 (ms) - Step 2: Define RRTNCSG before and after MLNCSG in FR1 and FR2 - Handled by Issue 3-5, e.g., same or longer than RRTlegacy. - Step 3: Define MGLNCSG as MLNCSG + 2* RRTNCSG. - Step 4: Define VIL

For the relation between ML, MGL and RTT, we think option 1a follows the same principle in LTE and can be supported. The length of ML+VIL1+VIL2 may be longer than the MGL of corresponding MG pattern. For option 3, we are not sure why MGLNCSG is needed in NCSG pattern and its value is the same as MGLlegacy in our understanding

Proposal 5: Support option 1a: ML = MGL – 2*RRT and ML + VIL1 + VIL2 > MGL

- Issue 3: Scheduling restriction

- Open issues: - Option 1: during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply - Option 2 - For intra-frequency measurement, existing scheduling restriction requirements apply. - For inter-frequency measurement and the target carrier and the serving cell are in same band, existing scheduling restriction requirements apply except that all symbols in SMTC windows are restricted. - For inter-frequency measurement and the target carrier and the serving cell are in different bands, all symbols in SMTC windows are restricted when scheduling restrictions apply, and whether scheduling restrictions apply depends on UE capability. - NW should be informed whether UE needs scheduling restriction or not for a combination of an inter-frequency target carrier and a serving cell. - Option 3: The existing scheduling restriction requirements defined in TS 38.133 for FR1 shall apply during ML when serving and measured carriers are in FR1. No scheduling restriction is allowed for FR2 during ML when serving carrier and measured carriers are in FR2 and use IBM. - Option 4: Scheduling restriction for NCSG is FFS, and check with RAN2 on the feasibility of informing NW the CBM or IBM between inter-frequency measurements and serving cells in FR2.

During ML, UE is expected to transmit/receive data and perform measurements simultaneously. UE behaviour is similar to measurements without gap and therefore option 2 can be supported by us. We are open to discuss whether to define separate scheduling restrictions considering different UE capability of supporting IBM and/or network capability “simultaneousRxTxInterBandCA”.

Proposal 6: Support option 2 for scheduling restriction requirements and FFS whether UE and/or network capability should be considered.

Conclusion

In this contribution, we provided our observations and proposals on NCSG for RRM measurement gap enhancement.

Observation 1: Whether separate NCSG patterns are needed for synchronous and asynchronous cases depends on whether VIL is considered as a part of NCSG pattern.

Proposal 1: For a NCSG capable UE, no additional signalling is needed to indicate support the NCSG pattern and per-FR NCSG capability.

- Per-FR NCSG is supported if per-FR MG is supported

- The NCSG patterns corresponding to the supported MG patterns are also supported.

- The NCSG patterns corresponding to the mandatory MG patterns are also mandatory.

Proposal 2: New signalling separate from “NeedForGap” is introduced to indicate the support of NCSG.

Proposal 3: Not consider VIL as a part of NCSG pattern.

Proposal 4: Not define separate NCSG patterns for synchronous and asynchronous cases.

Proposal 5: Support option 1a: ML = MGL – 2*RRT and ML + VIL1 + VIL2 > MGL

Proposal 6: Support option 2 for scheduling restriction requirements and FFS whether UE and/or network capability should be considered.

- References

- R4-2115344, WF on R17 NR MG enhancement – NCSG, Apple