3GPP TSG-RAN WG4 Meeting #101-e	R4-2118399

Electronic Meeting, 1st Nov. – 12th Nov. 2021

Agenda Item:	11.1.1

Source:	Ericsson

Title:	LS response on gap handling for MUSIM

Document for:	Discussion

# Introduction

In last RAN2 meeting, a LS for MUSIM was approved and send to RAN4[1].

RAN2 has discussed the mechanism for a UE to notify Network A of its switch from Network A to Network B for MUSIM purpose. Network A is NR and Network B can either be LTE or NR. During the switching from the Network A, the UE can still be RRC connected state in Network A. The following scenarios and gap mechanism were agreed by RAN2. Scenarios and supported gap types - RAN2 aims to support at least the below scenarios 1/2/3 in Rel-17 for cases when the UE is allowed to switch to network B without leaving connected state at network A. Scenarios 1: Periodic switching, including SSB detection/paging reception, serving cell measurement, neighbouring cell measurement including intra-frequency, inter-frequency and inter-RAT measurement; Scenarios 2: SI receiving at network B; Scenarios 3: Aperiodic (one-shot) switching with both transmission and reception at network B but will not enter RRC-connected state in NW B (e.g. no RRC connection Resume/Setup) at network B, including On-demand SI request; - Only per UE level scheduling gap is supported in Rel-17 for non-DC. FFS if we support MR-DC. - Do not support autonomous gaps for MUSIM in Rel-17. Gap configuration and activation - The network is allowed to configure at most 3 gap patterns (for any MUSIM purpose). - Only a single aperiodic gap (for MUSIM) is supported in Rel-17. At most two periodic “gaps” (for MUSIM) and a single aperiodic gap (for MUSIM) is supported in Rel-17. FFS if signalling supports more. - The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap Periodic/Aperiodic/autonomous Gap configuration and activation - The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period. - Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions. Gap configuration assistance information - UE is allowed to include assistance information for setup or release of gaps for both 1) periodic gaps and 2) aperiodic gap in one UEAssistanceInformation Msg. - To report the assistance information, the UE maps the timing info of the Gap on the network B to the network A and reports the mapped timing info to the network A. - For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) may be included. FFS is other information is included (e.g. gap purpose).. For the above Scenario 1 and Scenario 2: - In Network B, System Information is needed for paging reception, serving cell measurement, neighbouring cell measurements including intra-frequency, inter-frequency and inter-RAT measurements. SIBs other than SIB1 are carried in System Information (SI) messages, which are periodically scheduled in SI window. The period of SI scheduling (si-Periodicity) can be {rf8, rf16, rf32, rf64, rf128, rf256, rf512} radio frames. For NR, the SI window Length (si-WindowLength) range can be {s5, s10, s20, s40, s80, s160, s320, s640, s1280} slots, for LTE the SI window Length (si-WindowLength) range can be {ms1, ms2, ms5, ms10, ms15, ms20, ms40} ms. Note: SSB detection is only needed for NR network. For the above Scenario 3: - Only applied when network B belongs to NR, UE can request the on-demand SIs based on RACH procedure. For MSG1 based on-demand SI procedure, only MSG1 and MSG2 transmission and reception are needed. For MSG3 based on-demand SI procedure, all MSG1-MSG4 transmission and reception are needed. RAN2 assumes that at most three MUSIM gap patterns can be configured at the same time to receive and transmit in Network B. RAN2 will continue to discuss the detailed MUSIM gap handling, e.g., gap pattern (exact value for offset, gap cycle and duration), in part based on the RAN4 feedback requested below, and continue to inform RAN4 on these. 2. Actions: To RAN4: ACTION: RAN2 kindly asks RAN4 to answer the questions below. Question 1: Are the existing measurement gap cycle and duration value(s) sufficient to support the above any of Scenarios 1, 2, and 3? Question 2: If the answer to Question 1 is negative, RAN2 would like to request feedback on the gap cycle and duration value(s) for the above scenarios and in particular: - For Scenario 1, could RAN4 provide feedback on the range of value(s) for gap cycle and duration needed to meet the Idle/Inactive mode RRM requirements in Network B? - For Scenario 2, could RAN4 provide feedback on the range of value(s) for gap cycle and duration required to acquire the necessary system information in Network B? - What would be the feasible range of value(s) for gap cycle and duration that can allow the UE stay in Connected mode in Network A for all 3 scenarios? Question 3: What are the impacts of multiple activated MUSIM gaps (at most two periodic gaps and a single aperiodic gap) from RAN4 perspective?

As mentioned in RAN2’s LS, the following UE behaviours for NW B will need the gaps from NW A.

- serving cell measurement

- neighbouring cell measurements including intra-frequency, inter-frequency and inter-RAT measurements

- paging reception

- SIB reading

- on-demand SI request

Measurement and SIB reading is performed periodically. Thus, the periodic gaps are assumed. On-demand SI request will need an aperiodic gap. In the following sections, we will try to analyze each scenario for gap handling and try to response the LS to RAN2.

# General principles for MUSIM gap design

In MU-SIM devices, UE will request the gaps to switch to NW B while having on-going traffic in NW A. From NW A’s perspective, the density of gaps for NW B shall have less interruption to on-going NW A’s traffic. When UE requests the gaps, a reasonable gap periodicity is preferred. For example, when the gap is for Idle mode serving cell measurements with DRX=2.56s, the gap periodicity = 160ms is unreasonable. In other words, lots of gap occasions(15/16 in each DRX cycle) will be wasted.

Proposal 1: The gap periodicity for handling NW B’s behaviour shall be as long as possible to avoid too much interruption to NW A.

Proposal 2: The gap length for handling NW B’s behaviour shall be as short as possible.

After the scheduling gap requested by UE ends, the NW A will assume the UE has resumed its RRC connection. If the UE may still stay in the NW B to finish the activity, then the resources scheduled by NW A will be wasted. Thus, when UE requests a gap, both NW A and UE will believe the UE will resume NW A’s activity immediately after the gap occasion.

Proposal 3: UE is expected to be scheduled immediately by NW A after each gap occasion.

Similar as legacy measurement with gap, due to the UE RF implementation, UE cannot guarantee to conduct measurement simultaneously on two frequency layers. It is assumed that UE will perform only one frequency layer’s measurements in each gap occasion for NW B.

Proposal 4: UE is not expected to perform two frequency layers’ measurements simultaneously in one gap occasion.

# Gap for Idle mode measurements

Requirements for Idle mode measurements

In current spec. TS 38.133, the requirements for measurement periodicity for each UE behaviour in Idle mode are as follows.

#### 4.2.2.2 Measurement and evaluation of serving cell The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1. #### 4.2.2.3 Measurements of intra-frequency NR cells … The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1) for intra-frequency cells that are identified and measured according to the measurement rules. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2. #### 4.2.2.4 Measurements of inter-frequency NR cells If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. … When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell. The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter (see table 4.2.2.4-1) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell. #### 4.2.2.5 Measurements of inter-RAT E-UTRAN cells If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2 … Cells which have been detected shall be measured at least every (NEUTRA_carrier) * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. #### 4.2.2.7 General requirements The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.

From current RAN4 requirement, we can conclude the following observations.

Observation 1: UE will perform serving cell measurements with periodicity = DRX cycle.

Observation 2: UE will perform intra-frequency measurements as follow.

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, no intra-frequency measurements;

- Otherwise: measurements periodicity = Tmeasure

Observation 3: UE will perform inter-frequency measurements as follow.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, only higher priority layer measurements: Thigher_priority_search

- Otherwise: High/equal/low priority layer measurements with measurements periodicity Kcarrier*Tmeasure

Observation 4: UE will perform inter-RAT measurements as follow.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Only higher priority layer measurements Thigher_priority_search

- Otherwise: High/low priority layer measurements: NEUTRA_carrier*Tmeasure

Observation 5: UE may perform intra-freq, inter-freq and inter-RAT meas. within the same DRX cycle.

Observation 6: Serving cell measurements frequency(DRX cycle) may faster than intra-freq, inter-freq and inter-RAT meas.(Tmeasure).

Scenarios for Idle mode measurements

When MUSIM UE requests gaps for NW B Idle mode measurements, UE will perform periodically cell measurements for

- Serving cell evaluation

- Intra-frequency cell evaluation

- Inter-frequency cell evaluation

- Inter-RAT cell evaluation

The possible measurement combinations for intra-frequency, inter-frequency and inter-RAT can be as follows.

Table 1. The cell reselection measurement combination in Idle mode

| Index | Intra-frequency | Inter-frequency | Inter-RAT |
| --- | --- | --- | --- |
| #1 | No intra- meas. | Only higher priority | Only higher priority |
| #2 |  |  | High/low priority |
| #3 |  | High/equal/low priority | Only higher priority |
| #4 |  |  | High/low priority |
| #5 | intra- meas. | Only higher priority | Only higher priority |
| #6 |  |  | High/low priority |
| #7 |  | High/equal/low priority | Only higher priority |
| #8 |  |  | High/low priority |

For example, if UE will perform possible Idle mode measurements as scenario #7 with DRX cycle=0.32s. The serving cell measurement periodicity will be DRX=0.32s(highlighted by green circles). Intra-frequency measurements can be performed at the same time with serving cell measurement. However, inter-frequency measurements will be performed with periodicity Tmeasure = 1.28s(highlighted by green circles). Inter-RAT measurements will be performed with periodicity Thigher_priority_search = 60s. Obviously, if UE only requests one gap for all above serving cell/intra-frequency/inter-frequency/inter-RAT measurements, the gap periodicity will be at least 0.16s. It implies gap waste ratio will be 5/8 per 1.28s. As we mentioned before, the gap design principle is gap periodicity for handling NW B’s behaviour shall be as long as possible to avoid too much interruption to NW A. Thus, it’s reasonable to request two periodical gaps to NW A with one gap periodicity = DRX for serving measurements and another gap periodicity = Tmeasure for inter-frequency measurements. At the same time, an aperiodic gap for inter-RAT measurements is preferred which measurement periodicity is Thigher_priority_search = 60s.

Figure 1: Example of UE cell reselection measurements in Idle mode

Observation 7: One gap for serving cell/intra-frequency/inter-frequency/inter-RAT measurements is not enough for NW B or resource waste for NW A.

Gap design for Idle mode measurements

The summary on possible gap scheduling in each scenario is as follow.

Table 2. Summary on relation of DRX and possible gap scheduling type, MGRP

| Scenario |  | Max Gap Num | DRX | MGRP | Aperiodic gap(s) |
| --- | --- | --- | --- | --- | --- |
| #1 |  | 1 | 0.32/0.64/1.28/2.56 | serving cell = DRX | 2 |
| #2 |  | 2 | 0.32/0.64 | Gap 1: serving cell = DRX Gap 2: inter-RAT = Tmeas | 1 |
|  |  |  | 1.28/2.56 | Gap 1: serving cell/inter-RAT = DRX/2 |  |
| #3 |  | 2 | 0.32/0.64 | Gap 1: Serving cell = DRX Gap 2: Inter-freq = Tmeas | 1 |
|  |  |  | 1.28/2.56 | Gap 1: Serving cell/inter-freq = DRX/2 (If SSB offset different, two gaps with DRX) |  |
| #4 |  | 2 | 0.32 | Gap 1: Serving cell = DRX Gap 2: Inter-freq/inter-RAT = Tmeas/2 | 0 |
|  |  |  | 0.64 | Gap 1: Serving cell/Inter-freq/inter-RAT = DRX/2 (If SSB offset different, two gaps with DRX) |  |
|  |  |  | 1.28/2.56 | Gap 1: serving cell/Inter-freq/inter-RAT meas. = ceiling(DRX/3) |  |
| #5 | 1 |  | 0.32/0.64/1.28/2.56 | Gap 1: serving cell/intra-freq = DRX | 2 |
| #6 | 2 |  | 0.32/0.64 | Gap 1: serving cell/intra-freq = DRX Gap 2: inter-RAT = Tmeas | 1 |
|  |  |  | 1.28/2.56 | Gap 1: serving cell/intra-freq/inter-RAT = DRX/2 |  |
| #7 | 2 |  | 0.32/0.64 | Gap 1: Serving cell/intra-freq = DRX. Gap 2: Inter-freq = Tmeas | 1 |
|  |  |  | 1.28/2.56 | Gap 1: Serving cell/intra-freq/inter-freq = DRX/2 (If SSB offset different, two gaps with DRX) |  |
| #8 | 2 |  | 0.32 | Gap 1: Serving cell/intra-freq = DRX Inter-freq/inter-RAT = Tmeas/2 | 0 |
|  |  |  | 0.64 | Gap 1: Serving cell/Inter-freq/inter-RAT = DRX/2 (If SSB offset different, two gaps with DRX) |  |
|  |  |  | 1.28/2.56 | Gap 1: serving cell/Inter-freq/inter-RAT meas. = ceiling(DRX/3) |  |

To make use of the resource in NW A reasonable, it is expected at most two gaps, one for serving cell measurements and another for inter-frequency/inter-RAT measurements because serving cell and inter-frequency use different measurement periodicities in short DRX cycle(0.32, 0.64s). If DRX cycle is long(1.28s, 2.56s), DRX cycle = Tmeasure. It can be seen only one gap with periodicity= DRX/2 may be applied. Furthermore, considering the SSB offset of inter-frequency may be different with intra-frequency, two gaps are also expected regardless of the DRX cycle.

Based on the above observations for serving cell, intra-frequency, inter-frequency, inter-RAT measurement periodicity, we have the following proposals.

Proposal 5: The possible gap periodicity for UE performing cell reselection measurements in Idle mode for NW B will be DRX cycle(0.32s, 0.64s, 1.28s and 2.56s).

Proposal 6: The legacy gap length can be reused for UE performing cell reselection measurements in Idle mode for NW B.

Proposal 7: At most two independent gaps may be needed to minimize the interruption to NW A and perform the possible measurements for NW B due to

- different meas. periodicity of each frequency layer, or

- different SSB offset among frequency layers

Proposal 8: It’s better to apply aperiodic gap(s) instead of a periodic gap for high priority only measurements because of the long measurement periodicity(60s).

At the same time, we noticed a relaxation Idle mode was introduced in Rel-16. It means the possible gap periodicity for UE performing measurements in relaxation mode will be extended with 3.

Proposal 9: The possible gap periodicity for UE performing measurements in relaxation mode for NW B will be 3*DRX cycle(0.32s, 0.64s, 1.28s and 2.56s).

# Gap for paging monitoring

In NR, according to RAN1 design, SS burst periodicity can be {5, 10, 20, 40, 80, 160}ms, and the SS burst may be transmitted in the first or second half of a radio frame (indicated in MIB). There are two possible ways to multiplex SSB and PO based on RAN2’s agreement:

- SSB FDMed with PO

- SSB TDMed with PO

Figure 2: Three patterns for multiplexing PO with SSB

The summary of length and periodicity of PDCCH monitoring for different patterns are as follow.

Table 3. Summary of length and periodicity of PDCCH monitoring for different patterns

| Patterns | PDCCH Monitoring Occasions Interval | Length of PDCCH Monitoring Occasion per SSB | SFN in which PDCCH Monitoring Occasions are located | Carrier Frequency |
| --- | --- | --- | --- | --- |
| Pattern 1 | Every 20ms | 2 consecutive slots | - SFN mod 2 = 0 for all SSBs OR - SFN mod 2 = 0 for some SSBs and SFN mod 2 = 1 for others. | FR1, FR2 |
| Pattern 2/3 | SS burst set period (5, 10, 20, 40, 80, 160) ms | 1 slot | SFN in which SS burst set is transmitted. (Note: SS burst set can be transmitted in any radio frame of SS burst set period. | FR2 |

In FR2, due to Rx beam issue, UE cannot perform PO monitoring and SSB fine sync. in the same slot even if SSB is FDMed with PO. Meanwhile, additional AGC retuning is needed before UE wakes up for PO monitoring after a long DRX cycle.

Observation 8: UE may wake up twice for AGC retuning by SSB and PO monitoring.

We use the following example to illustrate the gap design for PO monitoring in MU-SIM. UE will wake up to monitor PO in each DRX on duration(DRX=0.32s). However, as mentioned before, UE may need an additional SSB for AGC retuning and fine time sync. before monitoring paging. The time distance between SSB and related PO is uncertain depending on different SSB and PO multiplexing pattern, default/non-default association between SSB and PO, and SSB index indication(ssb-position-in-burst). If the time distance of SSB occasion and PO is larger than a threshold T(purple range), two independent gaps are preferred. Otherwise, one gap with adaptive gap length is preferred. (The gap length depends on the time distance between SSB and PO)

Figure 3: Example of PO monitoring in Idle mode

Proposal 10: If the distance between the SSB for AGC and PO is shorter than [T], one gap with longer gap length can be applied. Otherwise, two independent gaps are preferred. T can be 2*gap margin+1 NW A’s slot(async. between NW A and NW B).

Proposal 11: It’s up to UE to request one gap or two gaps for paging reception, but the total gap length shall less than [L]. L can be SMTC + PO length + T + 2*gap margin.

Proposal 12: The possible gap periodicity for NW B’s paging monitoring will be DRX cycle(0.32s, 0.64s, 1.28s and 2.56s).

# Gap for SIB acquisition

In Idle mode, the requirements for maximum interruption in paging reception was defined when UE switches the serving cell to re-selected cell. In NR, the interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. It implies two SMTCs are needed for timing synchronization before SIB acquisition.

Observation 9: Two additional SMTCs are needed for timing synchronization before SIB acquisition for maximum interruption in paging reception requirements.

Proposal 13: Based on current RAN4 spec for maximum interruption in paging reception, two additional gap occasions with periodicity = SMTC are needed before SIB acquisition.

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

In Rel-16 RRM enhancement WI, RAN4 had already discussed the interruption length and periodicity for SIB1 reading in CGI reading topic. The gap length and periodicity for SIB1 acquisition can refer on the discussion in Rel-16 CGI reading as follow. The maximum duration for SIB1 acquisition will be 6*160ms.

##### 8.2.2.2.14 Interruptions when identifying CGI of an NR cell with autonomous gaps When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell or any activated SCell: - with up to K1 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.2.2.14-1 for each interruption during MIB decoding time period TMIB (ms) specified in clause 9.11. - with up to L1 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.2.2.14-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 1. - with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.2.2.14-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3. Where: - K1 = 6 for the target cell carrier frequency on FR1 and K1 = 25 for the target cell carrier frequency on FR2, and - L1 = TSIB1/20, and - L2 = TSIB1/TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier . Table 8.2.2.2.14-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37.

### 9.11.2 CGI identification of an NR cell with autonomous gaps The UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within: Tidentify_CGI = (TMIB + T SIB1) ms Where: TMIB is the time period used to acquire MIB message. TMIB = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB = 25 * TSMTC ms for target cell carrier frequency on FR2. TSIB1 is the time period used to acquire SIB1 message. TSIB1 = 6 * TRMSI-scheduling ms. Where TRMSI-scheduling is the periodicity with which the SIB1 is actually transmitted by the NR target cell.

Proposal 14: A periodic gap for SIB1 acquisition is preferred and the gap can be released with min{6*160ms, SIB1 acquisition time}.

Proposal 15: The gap length for NW B SIB1 acquisition can be 7ms.

Proposal 16: The gap periodicity for NW B SIB1 acquisition can be 20ms for pattern 1 and SMTC for pattern 2,3.

UE will not only receive SIB1 but also other OSIs. OSIs other than SIB1 are carried in System Information (SI) messages, which are periodically scheduled in SI window. The period of SI scheduling (si-Periodicity) can be {rf8, rf16, rf32, rf64, rf128, rf256, rf512} radio frames. For NR, the SI window Length (si-WindowLength) range can be {s5, s10, s20, s40, s80, s160, s320, s640, s1280} slots, for LTE the SI window Length (si-WindowLength) range can be {ms1, ms2, ms5, ms10, ms15, ms20, ms40} ms. From RAN2 spec, the SI message acquisition is as follow.

##### TS 38.331 ##### 5.2.2.3.2 Acquisition of an SI message For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation. If searchSpaceOtherSystemInformation is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space indicated by searchSpaceOtherSystemInformation. PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

Observation 10: Other SI message acquisition procedure is similar as paging monitoring and SIB1 reading.

However, UE is also required to decode several OSIs sequentially except SIB1 based on schedulingInfoList. Due to different periodicity and SI window, the gaps for different SIs acquisition may be overlapped with each other. RAN4 needs to further study how to handle the collision case.

Figure 4: Example of multiple SIs acquisition

Proposal 17: RAN4 needs to further study how to design the gap to handle the multiple SIs acquisition.

- Whether an aperiodic gap or periodic gap can be applied

- How to handle the multiple SIs scenario, one gap or multiple gaps

- How to handle the scenario if the proximity of two SIs is less than threshold [T1]

# Gap for on-demand SI request

In RAN2 LS, an aperiodic gap is expected for UE requesting the on-demand SIs based on RACH procedure. However, the time duration for the UE to acquire the on-demand SI by using contention based random access in NW B is unpredictable. Thus, multiple trials for RACH transmission are expected for on-demand SI request.

##### 5.2.2.3.3 Request for on demand system information The UE shall: … else if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[13], clause 5.1.1 is met: 2> trigger the lower layer to initiate the random access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfig corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting; 2> if acknowledgement for SI request is received from lower layers: 3> acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2, immediately;

The time distance between RACH transmission(Msg 1) and RAR window is defined in TS 38.213 as follow.

## 8.2 Random access response In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in Clause 10.1. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

Proposal 18: RAN4 to further study how to design the aperiodic gap for on-demand SI:

- Option 1: Multiple short aperiodic gaps for each Msg1, Msg2, (Msg3, Msg4) transmission/reception or their combinations and multiple trials for RACH access

- Option 2: Single long aperiodic gap for the RACH access

# Relation with legacy MG and concurrent gaps in Rel-17

In current Rel-17 concurrent gaps discussion, there are several agreements for multiple gaps as follow.

- Max 2 MGs are assumed in concurrent gaps discussion.

- The rule for gap collision discussion is on-going

- The overhead for multiple gaps discussion is on-going

RAN4 #99 meeting - When UE doesn’t support per-FR gap, - Assume max 2 MGs as a starting point, when defining the requirements (e,g., overlapping, overhead cap, interruption, …) - Larger number can be considered if RAN4 has extra time in Rel-17. - UE capability can be discussed later and independently. RAN4 #100 meeting Issue 4-1: Rule for colliding gap occasions, if one of FO, FPO, PFO, PPO cases is introduced - Agreement: - Define a general rule for UE from the following aspects: - Gap collision handling on UE’s measurement behavior if it is agreed to define the requirements for any or all of the FO/FPO/PFO/PPO/FNO cases - … - the proximity conditions to apply gap collision handling, e.g., a time domain minimal distance [X]ms between the two gap instances - FFS whether the same gap collision handling can be applied to all of the FO/FPO/PFO/PPO/FNO cases Issue 5-1: Whether to define an overhead cap for concurrent gaps - No consensus on defining an overhead cap for concurrent gaps in this meeting - Open issue - Option 1: Yes - Option 2: No - Option 3: Postponed to 2nd phase

When UE supports MU-SIM, additional gaps for supporting UE’s NW B behaviour are needed except the traditional gaps for NW A’s measurements. From our understanding, the discussion on the MU-SIM gaps shall be based on the agreements for concurrent gaps in Rel-17, such as gap collision handling, overhead issues etc. Moreover, RAN4 shall further study the collision handling when MU-SIM gaps are overlapping with legacy MGs.

Proposal 19: RAN4 needs to discuss the MU-SIM requirements after consolidation of concurrent gaps in Rel-17.

Proposal 20: It’s expected to discuss the following aspects about the relation between legacy MG(s) and MU-SIM gaps, such as

- Gap collision within MU-SIM gaps

- Gap collision between MU-SIM gap for NW B and legacy MGs for NW A

- Scheduling restriction in NW A due to MU-SIM gap

- Whether current concurrent gaps’ framework can be applied to MU-SIM gaps

- Overhead when UE supports MU-SIM

- CSSF design for MU-SIM

- Measurement requirements for MU-SIM gaps

- UE capability for multiple gaps and MU-SIM gaps

# Conclusion

In the contribution, we discuss the gap handling for MU-SIM and response the LS. We have the following proposals:

Observation 1: UE will perform serving cell measurements with periodicity = DRX cycle.

Observation 2: UE will perform intra-frequency measurements as follow.

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, no intra-frequency measurements;

- Otherwise: measurements periodicity = Tmeasure

Observation 3: UE will perform inter-frequency measurements as follow.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, only higher priority layer measurements: Thigher_priority_search

- Otherwise: High/equal/low priority layer measurements with measurements periodicity Kcarrier*Tmeasure

Observation 4: UE will perform inter-RAT measurements as follow.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Only higher priority layer measurements Thigher_priority_search

- Otherwise: High/low priority layer measurements: NEUTRA_carrier*Tmeasure

Observation 5: UE may perform intra-freq, inter-freq and inter-RAT meas. within the same DRX cycle.

Observation 6: Serving cell measurements frequency(DRX cycle) may faster than intra-freq, inter-freq and inter-RAT meas.(Tmeasure).

Observation 7: One gap for serving cell/intra-frequency/inter-frequency/inter-RAT measurements is not enough for NW B or resource waste for NW A.

Observation 8: UE may wake up twice for AGC retuning by SSB and PO monitoring.

Observation 9: Two additional SMTCs are needed for timing synchronization before SIB acquisition for maximum interruption in paging reception requirements.

Observation 10: Other SI message acquisition procedure is similar as paging monitoring and SIB1 reading.

Proposal 1: The gap periodicity for handling NW B’s behaviour shall be as long as possible to avoid too much interruption to NW A.

Proposal 2: The gap length for handling NW B’s behaviour shall be as short as possible.

Proposal 3: UE is expected to be scheduled immediately by NW A after each gap occasion.

Proposal 4: UE is not expected to perform two frequency layers’ measurements simultaneously in one gap occasion.

Proposal 5: The possible gap periodicity for UE performing cell reselection measurements in Idle mode for NW B will be DRX cycle(0.32s, 0.64s, 1.28s and 2.56s).

Proposal 6: The legacy gap length can be reused for UE performing cell reselection measurements in Idle mode for NW B.

Proposal 7: At most two independent gaps may be needed to minimize the interruption to NW A and perform the possible measurements for NW B due to

- different meas. periodicity of each frequency layer, or

- different SSB offset among frequency layers

Proposal 8: It’s better to apply aperiodic gap(s) instead of a periodic gap for high priority only measurements because of the long measurement periodicity(60s).

Proposal 9: The possible gap periodicity for UE performing measurements in relaxation mode for NW B will be 3*DRX cycle(0.32s, 0.64s, 1.28s and 2.56s).

Proposal 10: If the distance between the SSB for AGC and PO is shorter than [T], one gap with longer gap length can be applied. Otherwise, two independent gaps are preferred. T can be 2*gap margin+1 NW A’s slot(async. between NW A and NW B).

Proposal 11: It’s up to UE to request one gap or two gaps for paging reception, but the total gap length shall less than [L]. L can be SMTC + PO length + T + 2*gap margin.

Proposal 12: The possible gap periodicity for NW B’s paging monitoring will be DRX cycle(0.32s, 0.64s, 1.28s and 2.56s).

Proposal 13: Based on current RAN4 spec for maximum interruption in paging reception, two additional gap occasions with periodicity = SMTC are needed before SIB acquisition.

Proposal 14: A periodic gap for SIB1 acquisition is preferred and the gap can be released with min{6*160ms, SIB1 acquisition time}.

Proposal 15: The gap length for NW B SIB1 acquisition can be 7ms.

Proposal 16: The gap periodicity for NW B SIB1 acquisition can be 20ms for pattern 1 and SMTC for pattern 2,3.

Proposal 17: RAN4 needs to further study how to design the gap to handle the multiple SIs acquisition.

- Whether an aperiodic gap or periodic gap can be applied

- How to handle the multiple SIs scenario, one gap or multiple gaps

- How to handle the scenario if the proximity of two SIs is less than threshold [T1]

Proposal 18: RAN4 to further study how to design the aperiodic gap for on-demand SI:

- Option 1: Multiple short aperiodic gaps for each Msg1, Msg2, (Msg3, Msg4) transmission/reception or their combinations and multiple trials for RACH access

- Option 2: Single long aperiodic gap for the RACH access

Proposal 19: RAN4 needs to discuss the MU-SIM requirements after consolidation of concurrent gaps in Rel-17.

Proposal 20: It’s expected to discuss the following aspects about the relation between legacy MG(s) and MU-SIM gaps, such as

- Gap collision within MU-SIM gaps

- Gap collision between MU-SIM gap for NW B and legacy MGs for NW A

- Scheduling restriction in NW A due to MU-SIM gap

- Whether current concurrent gaps’ framework can be applied to MU-SIM gaps

- Overhead when UE supports MU-SIM

- CSSF design for MU-SIM

- Measurement requirements for MU-SIM gaps

- UE capability for multiple gaps and MU-SIM gaps

# Reference

[1] R2-2108861, “LS on gap handling for MUSIM”, Vivo

# Annex: draft LS response on gap handling for MUSIM

3GPP TSG-RAN WG4 Meeting #101-e R4-21xxxxx Electronic Meeting, 1st – 12th Nov. , 2021 Title: Reply LS on gap handling for MUSIM Response to: R2-2108861 Release: Release 17 Work Item: LTE_NR_MUSIM-Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Zhixun Tang #### E-mail Address: zhixun.tang@ericsson.com Attachments: - 1. Overall Description: RAN4 thanks the LS from RAN2 on gap handling for MUSIM. During this RAN4 meeting, RAN4 discussed the questions and achieved the following conclusions: Question 1: Are the existing measurement gap cycle and duration value(s) sufficient to support the above any of Scenarios 1, 2, and 3? [RAN4 Response]: From RAN4’s understanding, new measurement gap patterns shall be introduced to support MUSIM. Question 2: If the answer to Question 1 is negative, RAN2 would like to request feedback on the gap cycle and duration value(s) for the above scenarios and in particular: - For Scenario 1, could RAN4 provide feedback on the range of value(s) for gap cycle and duration needed to meet the Idle/Inactive mode RRM requirements in Network B? - For Scenario 2, could RAN4 provide feedback on the range of value(s) for gap cycle and duration required to acquire the necessary system information in Network B? - What would be the feasible range of value(s) for gap cycle and duration that can allow the UE stay in Connected mode in Network A for all 3 scenarios? [RAN4 Response]: - Question 2.A - For scenario 1 serving cell/intra-frequency/inter-frequency/inter-RAT measurements, - At least MGRP = 0.32, 0.64, 1.28 and 2.56s shall be supported. - Additional MGRP = 3*(0.32s, 0.64s, 1.28s and 2.56s) shall be supported if MUSIM supports relaxation Idle mode in NW B. - The gap length for current gap pattern can be reused. - For scenario 1 paging monitoring, - If the distance between the SSB for AGC and PO is shorter than [T], one gap with longer gap length can be applied. Otherwise, two independent gaps are preferred. T can be 2*gap margin+1 NW A’s slot(async. between NW A and NW B). - It’s up to UE to request one gap or two gaps for paging reception, but the total gap length shall less than [L]. L can be SMTC + PO length + T + 2*gap margin. - The possible gap periodicity for NW B’s paging monitoring will be DRX cycle(0.32s, 0.64s, 1.28s and 2.56s). - RAN4 will further discuss the threshold [T] and total gap length [L]. - Question 2.B - For scenario 2 SIB1 acquisition, - A periodic gap is preferred and the gap can be released after min{6*160ms, SIB1 acquisition time}. - The gap length can be 7ms. - The gap periodicity can be 20ms for pattern 1 and SMTC for pattern 2,3. - For scenario 2 OSI acquisition, - An aperiodic gap is preferred. - RAN4 will further discuss how to design the gap to handle the multiple SIs acquisition. - Question 2.C - From RAN4’s understanding, the gap cycle and duration for scenario 1,2 can allow the UE stay in Connected mode in Network A. - For scenario 3, RAN4 will further discuss the feasible value for gap duration that can allow the UE stay in Connected mode in Network A, especially for possible multiple trials for RACH access for Network B. Question 3: What are the impacts of multiple activated MUSIM gaps (at most two periodic gaps and a single aperiodic gap) from RAN4 perspective? [RAN4 Response]: From RAN4’s understanding, two periodic gaps may be insufficient for MUSIM NW B monitoring. - In scenario 1, two periodic gaps may be expected for serving cell and inter-frequency measurements independently due to - different measurement periodicity of each frequency layer, or - different SSB offset among frequency layers - In scenario 1, two additional periodic gaps may be expected for SSB of AGC retuning and paging monitoring due to the time distance between the SSB and PO. - In scenario 2, a periodic gap is expected for SIB1 acquisition. Currently, in Rel-17 concurrent gaps agreements, max 2 MGs are assumed as a starting point, when defining the requirements. (e,g., overlapping, overhead cap, interruption, …) However, at least additional three gaps for NW B will be introduced for MUSIM purpose except the legacy gaps for NW A mobility procedure. It’s expected to discuss the following aspects about the relation between legacy MG(s) and MU-SIM gaps, such as - Gap collision within MU-SIM gaps - Gap collision between MU-SIM gap for NW B and legacy MGs for NW A - Scheduling restriction in NW A due to MU-SIM gap - Whether current concurrent gaps’ framework can be applied to MU-SIM gaps - Overhead when UE supports MU-SIM - CSSF design for MU-SIM - Measurement requirements for MU-SIM gaps - UE capability for multiple gaps and MU-SIM gaps RAN4 prefers to discuss the MU-SIM requirements after consolidation of concurrent gaps WI since there is too much coupling between current on-going discussion for concurrent gaps and MUSIM gaps. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: TSG-RAN4 #101-bis-e Jan. 17 – Jan. 25, 2022 Electronic Meeting TSG-RAN4 #102-e Feb.21 – March 03, 2022 Electronic Meeting