3GPP TSG-RAN WG4 Meeting #101-e                	R4-2118420

Electronic Meeting, Nov. 1-12, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on the CSI-RS based measurement requirements

Agenda item:	5.1.5

Document for:	Discussion

Introduction

In RAN4#100-e meeting, the UE behavior when the timing offset exceeds the threshold was discussed wherein companies still had different understanding [1]. In this paper, we are discussing this issue.

Sub-topic 1-2 UE behavior when the timing offset exceeds the threshold with single FFT assumption

- Discussion

  - TBA

- Agreements:

  - No accuracy requirements will be defined for the case when the timing offset exceeds the threshold with single FFT assumption.

  - If timing offset exceeds the threshold

    - Option 1: UE does not report CSI-RS based L3 measurements

    - Option 2: UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption

    - Option 3: UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE may not meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption

Discussion

In current specification, the timing offset no larger than CP is captured as one of side conditions where the measurement accuracy performance shall apply. There is no explicit description on the UE behavior when the condition is not fulfilled. However, we understood the UE is not expected to report CSI-RS based L3 measurements in this case as the accuracy of the received measurement results may not be qualified and this is not known by the network. If the network uses the received measurement results for mobility evaluation, it may lead to false mobility decision.

On the other hand, the measurement reporting requirements are specified for RRM measurements. As cited below, the reported measurements need to meet the accuracy requirements in section 10. In other words, if the measurement results could not fulfill the accuracy requirements, they should not be reported to the network. Therefore, we believe the UE should not report the measurement results as in Option 1 if the timing offset exceeds a threshold. Alternatively, if some UE is able to solve the timing offset issue on its implementation and generates the measurement results with qualified performance, the UE is allowed to report and the reported measurement results shall fulfill the accuracy requirements. Option 1 and Option 2 are both acceptable for us.

#### 9.10.2.4	Measurement Reporting Requirements

##### 9.10.2.4.1	Periodic Reporting

Reported CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2.3, 10.1.3.3, 10.1.7.2, 10.1.8.2, 10.1.12.2 and 10.1.13.2.

Proposal1: If timing offset exceeds the threshold, both of the options are acceptable:

  - Option 1: UE does not report CSI-RS based L3 measurements

  - Option 2: UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption

Conclusion

This contribution discusses the UE behaviour when timing offset exceeds the threshold. The proposal is summarized as below:

Proposal1: If timing offset exceeds the threshold, both of the options are acceptable:

  - Option 1: UE does not report CSI-RS based L3 measurements

  - Option 2: UE is not required to report CSI-RS based L3 measurements. If UE reports CSI-RS based L3 measurement, then the UE shall meet CSI-RS based L3 measurement reporting requirements in TS 38.133 section 9.10.2.4 and 9.10.3.4 based on the accuracy requirements for the case when the timing offset is below the threshold with single FFT assumption

References

- R4-2115321, WF on CSI-RS based L3 measurement requirements