3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118426

Electronic Meeting, 1st-12th, Nov, 2021

Source:	ZTE Corporation

Title:	Discussion on RRM requirements for HO with PSCell

Agenda item:	8.10.2.2

Document for:	Approval

# 1	Introduction

In the last 100-e meeting, RRM requirements for HO with PSCell were discussed and a WF was agreed [1]. In this contribution ,we further provide our views on the open issues.

- Discussion

Delay requirement design of HO with PSCell

Issue 2-2-2a: How the requirements for parallel processing and sequential processing are defined without considering Tprocessing and RA procedures - Proposals - Option 1: (Xiaomi, Apple, Huawei, vivo, Intel, OPPO, CATT, ZTE, CMCC, MTK) - Different requirements for parallel processing cases and sequential processing cases - Option 2: (Qualcomm, Nokia) - Unified requirements to cover both parallel processing cases and sequential processing cases

In our view, parallel processing and sequential processing are different cases to complete the entire HO with PSCell procedure, so it is reasonable to define requirements for parallel processing cases and sequential processing cases separately.

Proposal 1: Take option 1, different requirements for parallel processing cases and sequential processing cases.

Issue 2-2-2b: Timeline for delay requirements without considering Tprocessing and RA procedures - Proposals - Option 1 (Xiaomi, Apple, Huawei, vivo, Intel, CATT, MTK) - For parallel processing cases, PCell HO and PSCell addition are performed in parallel independently - For sequential processing cases, - Option A: Sequential processing of cell search and timing sync for PCell handover and PSCell addition. (Huawei, vivo) - Option B: Tsearch can be extended for sequential processing cell search, e.g. Tsrch= Tsearch_MCG+Tsearch_SCG and the time for SSB post-processing may also be extended e.g. Tm=2xTmargin - Adopt the same time for loop processing as legacy T∆ i.e. the fine time tracking and acquiring full timing information of the target cell shall be assumed running independently for each CG - Option C: Sequential processing of cell search, timing sync and SSB processing time for PCell handover and PSCell addition. (Apple, vivo (FFS), MTK) - Option D: FFS (CATT, CMCC, OPPO) - Option 2 (Qualcomm) - For both parallel processing cases and sequential processing cases - Option A: - Tsearch can be extended for sequential processing cell search, e.g. Tsrch= Tsearch_MCG+Tsearch_SCG and the time for SSB post-processing may also be extended e.g. Tm=2xTmargin - Adopt the same time for loop processing as legacy T∆ i.e. the fine time tracking and acquiring full timing information of the target cell shall be assumed running independently for each CG - Option B: - Other options are not precluded

This issue depends on the result of Issue 2-2-2a. For parallel processing, PCell HO and PSCell addition are performed in parallel independently.  In sequential processing case, the UE uses PCell timing as the reference timing to acquire the location of the SSB of PSCell.  So for sequential processing, at least cell search, timing sync and SSB processing are performed sequential.

Proposal 2: Option 1 and option 1C is preferred.

Generic RACH assumption

Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell - Proposals - Option 1a (ZTE, Nokia, vivo, CATT, Ericsson, Nokia): - Include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell. No need to mention 2-step or 4-step in HO with PSCell requirements. - Option 1b (ZTE, CATT, vivo): - RAN4 shall define delay requirements for HO with PSCell for both 2-step and 4-step RA. Impact on delay requirements depends on timeline with respect to parallel processing of RA. - Option 2 (Apple, Xiaomi, Qualcomm, OPPO, Intel, MTK): - For requirement of HO with PSCell, RAN4 starts the discussion with 4 step RACH first and FFS on 2 step RACH. - Option 3 (Qualcomm): - Define the ending points as Pcell PRACH and PSCell PRACH respectively by assuming 4-step RACH

This issue has been discussed for several meetings. As specified in 5.1.1 in TS 38.321, both 2-step RA and 4-step RA are options for network access . The UE selects the type of random access at initiation of the random access procedure based on network configuration. Besides, the HO requirements and PSCell addition delay requirements are applicable for both 2-step RA and 4-step RA procedures as indicated in clause 3.6.8 of TS 38.133.

3.6.8 Applicability of 2-step RA and 4-step RA in RRM requirements

Unless explicitly stated otherwise the requirements under the following clauses, where the UE transmits random acess to NR serving cell or NR target cell, are applicable for both 2-step RA and 4-step RA procedures [3]:

-	Handover requirements in clause 6.1, except for clause 6.1.2

-	RRC connection mobility control requirements in clause 6.2, except for clause 6.2.2,

-	UE transmit timing requirements in clause 7.1,

-	PScell addition delay requirements in clause 8.9.2,

-	PSCell change requirements in clause 8.11 and

-	Conditional PSCell change requirements in clause 8.11B.

In our view, the delay requirements for HO with PSCell for both 2-step and 4-step RA should be defined and could be defined at this stage.  Details of the requirements may depend on the conclusion of other issues, such as parallel or sequential processing assumption, which could be discussed later.

- Proposal 3: Include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell.

Issue 2-4-2: RACH occasion collision between Pcell and PSCell - Proposals - Option 1 (Apple, Ericsson, Huawei, vivo, CATT, Qualcomm, MTK): - for FR1+FR1 EN-DC, an additional uncertainty delay due to PSCell RACH collision with PCell UL channels may be introduced if the PSCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.1; - for FR1+FR1 NE-DC, an additional uncertainty delay due to PCell RACH collision with PSCell RACH may be introduced if the PCell RACH cannot be transmitted based on the criteria in TS38.213 section 7.6.2; - otherwise, if target PCell and target PSCell are on the different FRs for EN-DC or NR-DC, no need to consider RO collision issue. - FFS how to capture the extra uncertain delay in spec. - Option 2 (Nokia): - No additional delay as in option 1 is needed to be introduced for LTE-FR1 EN-DC and FR1-LTE NE-DC

The cases of uplink transmission collision for EN/NE-DC scenarios between MCG and SCG have been defined in TS38.213 section 7.6.1 and section 7.6.2 as stated in option 1. This collision is caused by power allocation scheme between NR and LTE on the same FRs.

However, the necessity to define a delay requirement for this case needs more discussion. For NE-DC: E-UTRAN PSCell addition requirement in TS 38.133, we already defined TE-UTRAN-PSCell_DU,which is the delay uncertainty in acquiring the first available PRACH occasion in the E-UTRAN PSCell. The additional uncertainty delay addressed here could be covered by the same parameter.

Observation 1:  The necessity to define a delay requirement for this case needs further discussion.

- Conclusion

Proposal 1: Take option 1, different requirements for parallel processing cases and sequential processing cases

Proposal 2: Option 1 and option 1C is preferred.

- Proposal 3: Include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell.

Observation 1:  The necessity to define a delay requirement for this case needs further discussion.

References

- R4-2115337,WF on further RRM enhancement for NR and MR-DC–HO with PSCell,vivo