3GPP TSG-RAN WG4 Meeting #101-e	R4-2118451

Online meeting, 1-12 Nov, 2021

Source:	Xiaomi

Title:	TP for TR 38.862 on the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

Agenda item:			9.3.2

Document for:	Approval

1. Introduction

This contribution is a text proposal for TR 38.862 v0.3.0 to include rules of the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5.

2. Text Proposal

---Start of changes---

5.5       The maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

To guarantee the BCS4/BCS5 can cover all the possible bandwidth configurations for intra-band CA, the maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to

- min{n*max channel bandwidth, CA bandwidth class, frequency range} for intra-band contiguous CA

- min{n*max channel bandwidth, frequency range- min frequency gaps} for intra-band non-contiguous CA

Where n is the number of aggregated CCs.

---End of changes---