3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118619

# Electronic Meeting, Nov. 1-12, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE RF aspects for the lower 6GHz NR unlicensed operation

Agenda item:	7.1.3

Document for:	Approval

# Introduction

The WID on introduction of lower 6GHz NR unlicensed operation for Europe [1] initiated the work in RAN4 at RAN4#98.

During RAN4#98 a WF was agreed in [2] including the following agreements relevant for UE RF:

UE RF - A-MPR assumptions:

- A-MPR evaluation shall be performed with the following assumptions:

  - A-MPR for PC5 is assessed using band n96 PC5 definitions for VLP and LPI device A-MPR.

  - No filter rejection is assumed <5935 MHz and >6425MHz for spurious emissions

  - Same PA model and calibration setpoint from Band n96 is to be used.

  - A-MPR for other power classes are to be added at a later stage.

  - A-MPR studies for PC5 are to be compared and, if possible, decided at next RAN4 meeting.

UE RF - NS definition:

- NS shall be defined corresponding to LPI deployments and if introduced also VLP deployments.

- The currently, by ECC, defined OOB limit at -45 dBm/MHz for VLP shall be used. If/when a new limit is introduced by ECC the NS(s) will be revised accordingly.

During RAN4#98bis a WF was agreed in [2] including the following agreements relevant for UE RF:

UE RF - MPR values:

- A-MPR for PC5 LPI apply according to Note 5 in table below:

- (A-)MPR for VPL is FFS

- How to capture (A-)MPR in TS is FFS.

UE RF - NS definition:

- NS shall be defined corresponding to LPI deployments and if introduced also VLP deployments (Agreed in R4-2103229).

- Reusing existing PC5 with additional NS limitations to TX power is sufficient to limit MOP for VLP.

During RAN4#99 a WF was agreed in [6] including the following general agreements:

General - LPI and VLP deployment:

- VLP deployment can be included in specification given the available regulations.

  - Further discussion is needed in future meeting about the possible regulation issues.

During RAN4#100 the following agreements were made during GTW:

Agreements:

- The same hardware of UE as for n96 may be reused on the frequency range 5945MHz to 6425MHz no matter whether to define a new band or define new NS for the existing n96.

- For outdoor UEs connecting to the indoor LPI base stations, no solution is needed to meet the regulatory requirements in Rel-17.

- A-MPR for VLP as captured in TR 38.849 (R4-2113692)

The objectives of the WID together with the agreements so far are discussed further in the following sections.

# Discussion

#### NS for Europe

Additional RF requirements can, like done according to the FCC Report and Order FCC 20-51 for NS_53 and NS_54 already defined for n96, be included according to the Harmonised Standard for access to radio spectrum in the 6 GHz range [4] as shown in Table 6.2F.1-2.

Table 6.2F.1-2: Additional requirements for transmit power density [from TS 38.101-1]

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n[1xx] | [NS_xx] | 20, 40, 60, 80 | 5945 – 6425 | 1 |
|  | [NS_yy] | 20, 40, 60, 80 | 5945 – 6425 | 10 |

Note that [4] defines both Low Power Indoor (LPI) and Very Low Power (VLP) usage which could be represented as shown above by respectively NS_xx and NS_yy.

As only PC5 is suggested considered at the moment A-MPR values defined for n96 can be reused as agreed in [5]. However, since the frequency range is truncated A-MPR values for NS_xx and NS_yy might need to be studied and verified before included in section 6.2F.3 of TS 38.101-1.

For unwanted emission requirements band n96 follows n46 which is designed to adopt the requirements posed by the European Harmonised Standard for access to radio spectrum in the 5 GHz range. As the Harmonised Standard for access to radio spectrum in the 6 GHz range adopts e.g. the spectrum emission mask from the 5 GHz range the requirements are mostly already fulfilled. For spurious emissions (Section 6.5F.3 of 38.101-1) the NS_xx and NS_yy also needs to be added with values corresponding to [3] being -45dBm and -22dBm respectively. Both values are to be measured with a bandwidth of 1 MHz.

Proposal 1: 	NSs corresponding to requirements defined in EN 303 687 shall be defined in 38.101-1.

# Conclusion

In this contribution, we discuss the 6 GHz band for unlicensed operation in Europe. We have made following proposals:

Proposal 1: 	NSs corresponding to requirements defined in EN 303 687 shall be defined in 38.101-1.

# References

- RP-202592, WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Nokia Shanghai Bell

- R4-2103229, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Qualcomm, Charter Communications Inc.

- R4-2105383, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

- Draft EN 303 687 v0.0.13, ETSI BRAN

- TR 38.849

- R4-2108020, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia