3GPP TSG-RAN4 Meeting #101-e	R4-2118692

# Electronic meeting, Nov. 1 - 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on Network Controlled Small Gap (NCSG) in 38.133. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NCSG support is discussed in R17 measurement gap enhancement WI and need to be specified in 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RRM requirements about NCSG in 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for NCSG are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 9.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.1.2.2	Network Controlled Small Gap

This clause contains the general requirements on the UE regarding to Network Controlled Small Gap (NCSG).

The requirements in this clause are applicable for UE configured with SA NR, EN-DC, NE-DC or NR-DC operation mode.

UEs shall support the measurement gap patterns listed in Table 9.1.2.2-1 based on the applicability.

- Note: NCSG applicability is FFS.

NCSG can be configured when UE is required to perform SSB based intra-frequency measurement with gap, SSB based inter-frequency measurement with gap, CSI-RS based inter-frequency measurement with gap, inter-RAT E-UTRAN measurement, measurement on de-activated SCelland measurement on dormant SCell.

.NCSG will not be used for 2G/3G measurements and PRS measurements.

Table 9.1.2.2-1 NCSG pattern configuration

| NCSG Pattern Id | RF Retuning time (RRT1, ms) | Measurement Length during which there is no gap (ML, ms) | RF Retuning time (RRT2, ms) | Visible interruption Repetition Period (VIRP, ms) |
| --- | --- | --- | --- | --- |
| 0 | 0.5 | 5 | 0.5 | 40 |
| 1 | 0.5 | 5 | 0.5 | 80 |
| 2 | 0.5 | 2 | 0.5 | 40 |
| 3 | 0.5 | 2 | 0.5 | 80 |
| 4 | 0.5 | 5 | 0.5 | 20 |
| 5 | 0.5 | 5 | 0.5 | 160 |
| 6 | 0.5 | 3 | 0.5 | 20 |
| 7 | 0.5 | 3 | 0.5 | 40 |
| 8 | 0.5 | 3 | 0.5 | 80 |
| 9 | 0.5 | 3 | 0.5 | 160 |
| 10 | 0.5 | 2 | 0.5 | 20 |
| 11 | 0.5 | 2 | 0.5 | 160 |
| 12 | 0.25 | 5 | 0.25 | 20 |
| 13 | 0.25 | 5 | 0.25 | 40 |
| 14 | 0.25 | 5 | 0.25 | 80 |
| 15 | 0.25 | 5 | 0.25 | 160 |
| 16 | 0.25 | 3 | 0.25 | 20 |
| 17 | 0.25 | 3 | 0.25 | 40 |
| 18 | 0.25 | 3 | 0.25 | 80 |
| 19 | 0.25 | 3 | 0.25 | 160 |
| 20 | 0.25 | 1 | 0.25 | 20 |
| 21 | 0.25 | 1 | 0.25 | 40 |
| 22 | 0.25 | 1 | 0.25 | 80 |
| 23 | 0.25 | 1 | 0.25 | 160 |

During the RRT1 and RRT2, the UE is not expected to transmit and receive any data. During ML, the UE is expected to transmit and receive data on the corresponding serving carrier.

The corresponding total number of interrupted slots on serving cells is listed in Table 9.1.2.2-2 for all serving cells in EN-DC, NR standalone (with single carrier, NR CA and NR-DC configuration) and NE-DC.

Table 9.1.2.2-2 The total number of interrupted slots for all serving cells in EN-DC, NR standalone (with single carrier, NR CA and NR-DC configuration) and NE-DC

| NR SCS | Synchronous | Asynchronous |
| --- | --- | --- |
|  | interruption length before measurement | interruption length before measurement |
| 15KHz | 1 slot | 2 slots |
| 30KHz | 2 slots | 3 slots |
| 60KHz | 3 slots | 4 slots |
| 120KHz | 6 slots | 7 slots |

<End of Change 1>