3GPP TSG-RAN WG4 Meeting # 101-e                              R4-2118707

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title: 	TP and Discussion on MPR requirements for intra-band con-current V2X operation

Agenda Item:	8.15.4.4

Document for:	Approval

Introduction

In RAN4#100 meeting, a WF [1] WF on MPR for intra-band V2X con-current operation was approved. Based on the  WF, the candidate options are shown below.

WF: MPR requirements

- Issue 3-1-1: MPR for intra-band V2X con-current UE

  - Candidate options

    - Option 1: Consider MPR requirements for intra-band V2X con-current operation in R4-2112341 to keep the agreed MPR requirements in RAN4 #99-e meeting

    - Option 2: Specify 2dB MPR for 23dBm 64QAM, 4dB MPR for 23dBm 256QAM, 4.5dB MPR for 26dBm 64QAM and 7dB MPR for 26dBm 256QAM, no matter what the RB allocation is.

    - Option 3: The MPR values given in 38.101-3 [2] for cellular EN-DC shall adopt for V2X intra-band con-current operation where separate RF chains are used for Uu and SL.

Although companies provided the corresponding simulation results, the requirements in R4-2112341 are not aligned with current specification. In this paper, we’d like to provide our views about this topic and modify the requirements.

Discussion

Based on the WF [2] on MPR&AMPR simulation assumption for NR V2X intra-band con-current operation, Separate RF architecture is assumed with equal transmit power [3] as below.

  - Channel BW configurations for class B

    - 10MHz (NR SL, 30kHz SCS) + 10/20/40/60/80MHz (NR Uu, 30kHz SCS)

    - 20MHz (NR SL, 30kHz SCS) + 20/40/80MHz (NR Uu, 30kHz SCS)

  - Waveform

    - CP-OFDM(NR SL) + CP-OFDM(NR Uu)

  - Maximum total output power

    - 26dBm : 23dBm 2 PAs

    - 23dBm : Back off 3dB with 23dBm 2 PAs

  - Modulation order

    - QPSK/16QAM/64QAM/256QAM

  - Sub-channel sizes for NR SL

    - 10/24 RB for 10MHz

    - 10/50 RB for 20MHz

  - RB sizes for NR Uu

    - 1 RB for 10/20/40/60/80 MHz

However, the simulation assumptions above are not reflected in current TR. Given the equal transmit power is assumed, that means each RF chain has 3dB back off comparing to the single carrier case. It’s noted that the MPR requirements in clause 6.2.2 should apply for NR Uu RF chain, and the MPR requirements in in clause 6.2E.2 should apply for NR sidelink RF chain. The allowed minimum transmit power for NR V2X intra-band con-current on each RF chain shouldn’t be higher than the single carrier cases. Besides, 23dBm PA is assumed for both power class 2 and power class 3 cases. That means the linearity characteristics are same for PC2 and PC3. The allowed minimum transmit powers are shown below based on the current requirements.

Table 1 The allowed minimum transmit powers for Uu

| Modulation |  | The allowed minimum transmit powers for Uu (dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | 20 |  | 21.5 |
|  | 16 QAM | 20 |  | 21 |
|  | 64 QAM | 19.5 |  |  |
|  | 256 QAM | 16.5 |  |  |

Table 2 The allowed minimum transmit powers for PC5

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | 18.5 | 20.5 |
|  | 16QAM | 18.5 | 20.5 |
|  | 64 QAM | 18.5 |  |
|  | 256 QAM | 16.0 |  |

For modulation 64QAM and 256QAM, the MPR requirements are limited by EVM, no matter what kind of the RB allocations it is and the current tentative requirements can’t meet the MPR requirements for single carrier.

Thus, it’s proposed to do the following correction.

- Text Proposal

<TP for TR 38.785>

- 5.2.4.2.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.2.4.2.2-1 and Table 5.2.4.2.2-2.

Table 5.2.4.2.2-1: MPR simulation assumption for NR V2X intra-band contiguous con-current operation

| parameter | Assumption |
| --- | --- |
| center frequency | 4.5GHz (n79) |
| Bandwidth | 10/20/30/40MHz for NR SL 10/20/40/60/80MHz for NR Uu |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for NR SL CP-OFDM or DFT-S-OFDM for NR Uu |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |

For NR V2X intra-band contiguous con-current operation, simultaneous transmission of PUSCH/PUCCH and NR V2X’s PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.2.4.2.2-2 can be assumed based on current RAN1’s agreement.

Table 5.2.4.2.2-2: NR V2X intra-band contiguous con-current operation UE’ MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes for NR SL | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation for NR SL | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216. No restriction of LCRB for NR Uu |
| Regarding PSCCH / PSSCH multiplexing for NR SL |  |
| PSCCH size for NR SL | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH for NR SL | 0dB |
| NOTE 1: If there is not indicate the detail parameters, then follow TR38.886 for NR SL operation. Also follow TS38.211/TS38.212/TS38.101-1 for NR Uu operation. |  |

The simulation assumptions for combined NR SL and NR Uu are shown as below.

  - Channel BW configurations for class B

    - 10MHz (NR SL, 30kHz SCS) + 10/20/40/60/80MHz (NR Uu, 30kHz SCS)

    - 20MHz (NR SL, 30kHz SCS) + 20/40/80MHz (NR Uu, 30kHz SCS)

  - Waveform

    - CP-OFDM(NR SL) + CP-OFDM(NR Uu)

  - Maximum total output power

    - 26dBm : 23dBm 2 PAs

    - 23dBm : Back off 3dB with 23dBm 2 PAs

  - Modulation order

    - QPSK/16QAM/64QAM/256QAM

  - Sub-channel sizes for NR SL

    - 10/24 RB for 10MHz

    - 10/50 RB for 20MHz

  - RB sizes for NR Uu

    - 1 RB for 10/20/40/60/80 MHz

  - RF architecture

    - Separate RF architecture for NR SL RF chain and Uu RF chain is assumed.

For simultaneous transmission of PSFCH transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For simultaneous transmission of S-SSB transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-3 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-3: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |
|  | 64QAM | ≤ 2 | ≤ 2 |
|  | 256QAM | ≤ 4.5 | ≤ 4.5 |

For bandwidth class B with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2))

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2µ1 + LCRB2 ∙ 2µ2

NRB_alloc= (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2,

NRB,agg=NRB1∙2µ1+ NRB2∙2µ2.

If LCRB1 =0, RBStart_SL&UL = NRB1∙2µ1+ RBStart2∙2µ2,

if LCRB1 > 0, RBStart_SL&UL = RBStart1∙2µ1.

Where, µ1 and µ2 is 0, 1 and 2 for SCS of 15kHz, 30kHz and 60kHz respectively.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-4 for UE power class 3 with bandwidth class B.

Table 5.2.4.1.2-4: Non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ 2 | ≤ 2 | ≤ [3.0] |
|  | 256QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |

For bandwidth classes B with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0, when both SL CC and UL CC are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for SL CC1, RBStart2, LCRB2, and NRB2 are for UL CC2. SL CC1 is the component carrier with lower frequency.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_SL&UL / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2µ1 + (RBStart2 + LCRB2 ) ∙ 2µ2, RBStart_SL&UL = RBStart1∙21

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_SL&UL – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of SL CC1 and UL CC2 respectively.

In contiguous NR V2X intra-band con-current operation, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_SL&UL  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_SL&UL / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_SL&UL – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_SL&UL – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_SL&UL , BWgap and BWGB,low are as defined for the Inner region.

In contiguous NR V2X intra-band con-current operation, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with contiguous RB allocation is specified in Table 5.2.4.1.2-5 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-5: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |
|  | 64QAM | ≤ 4.5 | ≤ 4.5 |
|  | 256QAM | ≤ 7 | ≤ 7 |

For NR intra-band V2X con-current operation with adjacent channel, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 5.2.4.1.1-1 with non-contiguous RB allocation is specified in Table 5.2.4.1.2-6 for UE power class 2 with bandwidth class B.

Table 5.2.4.1.2-6: Non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ 4.5 | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ 7 | ≤ 7 | ≤ 7 |

- References

- R4-2107870, WF on MPR for intra-band V2X con-current operation, LG Electronics

- R4-2103246, WF on MPR&AMPR simulation assumption for NR V2X intra-band con-current operation, LG Electronics

- R4-2109702, MPR for NR V2X intra-band con-current operation with Uu, LG Electronics