3GPP TSG-RAN WG4 Meeting # 101-e                              R4-2118715

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title: 	Initial NTN simulation Results and discussion on ACLR and ACS

Agenda Item:	8.13.2.1

Document for:	Approval

Introduction

In RAN4#100-e, companies provided the simulation results to align with each other. RAN4 made some progress on the NTN simulation assumptions [1]. Based on the approved WF [2], companies can provide the initial NTN simulation results. The initial calibration summary and observations can be found in the contribution [3]. In this paper, we’d like to provide our NTN simulation results and discuss the ACLR and ACS.

Discussion on the scenario

Based on the initial simulation results [4], GEO scenario is restricted by the UL SINR. Considering the 36000km distance between GEO satellite and terrestrial network, the adjacent channel interference is limited. According to other companies’ simulation [5], GEO scenario is not the worst case. Thus, it’s proposed to deprioritize the GEO scenario when RAN4 study the ACLR and ACS requirements for satellite node and UE for satellite communication.

Proposal 1: To deprioritize the GEO scenario when RAN4 study the ACLR and ACS requirements for satellite node and UE for satellite communication.

For TN-NTN adjacent coexistence scenario, RAN4 don’t need to consider the ACIR since the ACLR and ACS requirements for terrestrial network have been specified. The specific analysis is shown below.

Table 1 consideration on TN-NTN adjacent coexistence scenario

|  | TN BS Victim | TN UE Victim | Satellite Node Victim | Satellite UE Victim |
| --- | --- | --- | --- | --- |
| Satellite Node aggressor | Sate ACLR = TBD BS ACS = 46dB | Sate ACLR = TBD UE ACS = 33dB |  |  |
| Satellite UE aggressor | SateUE ACLR = TBD BS ACS = 46dB | SateUE ACLR = TBD UE ACS = 33dB |  |  |
| TN BS aggressor | NA | NA | Sate ACS = TBD BS ACLR = 45dB | SateUE ACS = TBD BS ACLR = 45dB |
| TN UE aggressor | NA | NA | Sate ACS = TBD UE ACLR = 30dB | SateUE ACS = TBD BS ACLR = 30dB |

Observation 1: For TN-NTN adjacent coexistence scenario, RAN4 don’t need to consider the ACIR since the ACLR and ACS requirements for terrestrial network have been specified. The ACLR and ACS requirements for satellite node and UE can be considered directly.

Simulation results for NTN aggressor

For NTN aggressor, BS ACS is 46dB and UE ACS is 33dB as specified. When we simulate the scenario, different ACLR values were considered in order to find the reasonable requirements and worst scenario. The CDF figures of SINR and throughput are shown below.

Victim urban macro – Aggressor LEO600

For Urban Macro AAS, the UL SINR, UL Throughput and DL SINR, DL throughput are shown as below.

Figure 1 CDF of UL SINR for 2GHz urban AAS with LEO600 interference

Figure 2 CDF of UL Throughput for 2GHz urban AAS with LEO600 interference

Figure 3 CDF of DL SINR for 2GHz urban AAS with LEO600 interference

Figure 4 CDF of DL Throughput for 2GHz urban AAS with LEO600 interference

Victim Rural macro – Aggressor LEO600

For Rural Macro AAS, the UL SINR, UL Throughput and DL SINR, DL throughput are shown as below.

Figure 5 CDF of UL SINR for 2GHz Rural AAS with LEO600 interference

Figure 6 CDF of UL Throughput for 2GHz Rural AAS with LEO600 interference

Figure 7 CDF of DL SINR for 2GHz Rural AAS with LEO600 interference

Figure 8 CDF of DL Throughput for 2GHz Rural AAS with LEO600 interference

Victim urban macro – Aggressor LEO1200

For Urban Macro AAS, the UL SINR, UL Throughput and DL SINR, DL throughput are shown as below.

Figure 9 CDF of UL SINR for 2GHz Rural AAS with LEO1200 interference

Figure 10 CDF of UL Throughput for 2GHz Rural AAS with LEO1200 interference

Figure 11 CDF of DL SINR for 2GHz Rural AAS with LEO1200 interference

Figure 12 CDF of DL Throughput for 2GHz Rural AAS with LEO1200 interference

Victim Rural macro – Aggressor LEO1200

For Rural Macro AAS, the UL SINR, UL Throughput and DL SINR, DL throughput are shown as below.

Figure 13 CDF of UL SINR for 2GHz Rural AAS with LEO1200 interference

Figure 14 CDF of UL Throughput for 2GHz Rural AAS with LEO1200 interference

Figure 15 CDF of DL SINR for 2GHz Rural AAS with LEO1200 interference

Figure 16 CDF of DL Throughput for 2GHz Rural AAS with LEO1200 interference

Discussion

Analysis on satellite node transmitter

When satellite node transmit the signal, the adjacent channel interference from satellite node may have an impact on the terrestrial base station or terrestrial UE. The downlink throughput loss of the victim terrestrial UE or terrestrial base station with different ACLR values are shown below.

Table 2 The downlink throughput loss of the victim terrestrial UE under Urban Macro scenario with LEO600 satellite node interference

| Satellite Node ACLR [dB] | 22 | 26 | 30 | 34 | 38 |
| --- | --- | --- | --- | --- | --- |
| Average throughput baseline | 80.598 | 81.322 | 80.203 | 80.199 | 81.337 |
| Average throughput with adjacent interference | 80.57 | 81.294 | 80.203 | 80.192 | 81.337 |
| Average throughput loss | 0.03% | 0.03% | 0.00% | 0.01% | 0.00% |
| 5%-tile throughput baseline | 26.118 | 26.231 | 25.644 | 26.163 | 25.696 |
| 5%-tile throughput with adjacent interference | 26.11 | 26.231 | 25.644 | 26.163 | 25.696 |
| 5%-tile throughput loss | 0.03% | 0.00% | 0.00% | 0.00% | 0.00% |

Table 3 The downlink throughput loss of the victim terrestrial UE under Rural Macro scenario with LEO600 satellite node interference

| Satellite Node ACLR [dB] | 22 | 26 | 30 | 34 | 38 |
| --- | --- | --- | --- | --- | --- |
| Average throughput baseline | 55.337 | 56.013 | 55.13 | 55.71 | 55.825 |
| Average throughput with adjacent interference | 53.819 | 55.24 | 54.867 | 55.526 | 55.666 |
| Average throughput loss | 2.74% | 1.38% | 0.48% | 0.33% | 0.28% |
| 5%-tile throughput baseline | 14.526 | 14.64 | 14.562 | 14.928 | 14.683 |
| 5%-tile throughput with adjacent interference | 13.819 | 14.242 | 14.337 | 14.825 | 14.584 |
| 5%-tile throughput loss | 4.87% | 2.72% | 1.55% | 0.69% | 0.67% |

Table 4 The downlink throughput loss of the victim terrestrial UE under Rural Macro scenario with LEO1200 satellite node interference

| Satellite Node ACLR [dB] | 22 | 26 | 30 |
| --- | --- | --- | --- |
| Average throughput baseline | 56.743 | 54.837 | 56.401 |
| Average throughput with adjacent interference | 54.891 | 53.946 | 55.984 |
| Average throughput loss | 3.26% | 1.62% | 0.74% |
| 5%-tile throughput baseline | 15.239 | 14.764 | 15.328 |
| 5%-tile throughput with adjacent interference | 14.487 | 14.262 | 15.07 |
| 5%-tile throughput loss | 4.93% | 3.40% | 1.68% |

Table 5 The downlink throughput loss of the victim terrestrial BS under Urban Macro scenario with LEO600 satellite node interference

| Satellite Node ACLR [dB] | 22 | 26 | 30 | 34 | 38 |
| --- | --- | --- | --- | --- | --- |
| Average throughput baseline | 36.019 | 36.012 | 35.927 | 35.854 | 35.927 |
| Average throughput with adjacent interference | 36.019 | 36.012 | 35.927 | 35.854 | 35.927 |
| Average throughput loss | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| 5%-tile throughput baseline | 14.836 | 15.247 | 15.099 | 15.546 | 15.054 |
| 5%-tile throughput with adjacent interference | 14.836 | 15.247 | 15.099 | 15.546 | 15.054 |
| 5%-tile throughput loss | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

Table 6 The downlink throughput loss of the victim terrestrial BS under Rural Macro scenario with LEO600 satellite node interference

| Satellite Node ACLR [dB] | 22 | 26 | 30 | 34 | 38 |
| --- | --- | --- | --- | --- | --- |
| Average throughput baseline | 13.72 | 13.757 | 13.835 | 13.905 | 14.07 |
| Average throughput with adjacent interference | 13.71 | 13.754 | 13.835 | 13.905 | 14.069 |
| Average throughput loss | 0.07% | 0.02% | 0.00% | 0.00% | 0.01% |
| 5%-tile throughput baseline | 1.1059 | 1.1102 | 1.104 | 1.1105 | 1.1141 |
| 5%-tile throughput with adjacent interference | 1.1058 | 1.1101 | 1.104 | 1.1103 | 1.1141 |
| 5%-tile throughput loss | 0.01% | 0.01% | 0.00% | 0.02% | 0.00% |

Table 7 The downlink throughput loss of the victim terrestrial BS under Rural Macro scenario with LEO1200 satellite node interference

| Satellite Node ACLR [dB] | 22 | 26 | 30 |
| --- | --- | --- | --- |
| Average throughput baseline | 13.866 | 13.498 | 14.223 |
| Average throughput with adjacent interference | 13.859 | 13.494 | 14.223 |
| Average throughput loss | 0.05% | 0.03% | 0.00% |
| 5%-tile throughput baseline | 1.1279 | 1.1205 | 1.1696 |
| 5%-tile throughput with adjacent interference | 1.1278 | 1.1205 | 1.1696 |
| 5%-tile throughput loss | 0.01% | 0.00% | 0.00% |

Based on the simulation results, Rural Marco – LEO can be considered as the worst scenario. For Urban Macro – LEO scenario, because of the higher SINR and higher co-channel interference, the adjacent channel interference from LEO may have a few impacts on the terrestrial UE. However, for Rural Macro – LEO scenario, due to the larger inter-site distance, throughput loss of Rural UE is more sensitive than Urban.

Observation 2: Rural Marco – LEO (scenario case3) can be considered as the worst scenario when RAN4 specify the ACLR requirements for Satellite Node.

Based on the simulation results, it’s proposed to specify [20~22] dB ACLR requirements for satellite node so that impacts on the terrestrial UE are less than 5% throughput loss at 5%-tile CDF.

Proposal 2: It’s proposed to specify [20~22] dB ACLR requirements for satellite node.

Analysis on satellite UE transmitter

Based on the simulation results, the impacts on the terrestrial network are less than 5% throughput loss at 5%-tile CDF when it’s assumed that ACLR for Satellite UE is 30dB for all the scenarios. However, 30dB ACLR requirement for satellite UE is overdesigned based on the terrestrial-satellite coexistence.

Observation 3: If 30dB ACLR for satellite UE is specified to align with current industry implementation, 30dB ACLR requirement for satellite UE is overdesigned based on the terrestrial-satellite coexistence.

Simulation results for Terrestrial network aggressor

For terrestrial network aggressor, BS ACLR is 45dB and UE ACLR is 30dB as specified. When we simulate the scenario, different ACLR values were considered in order to find the reasonable requirements and worst scenario. The CDF figures of SINR and throughput are shown below.

Victim satellite receiver

Victim LEO600 – Aggressor urban macro

For LEO600, the UL SINR and UL throughput are shown as below.

Figure 17 CDF of UL SINR for 2GHz LEO600 with Urban Macro AAS interference

Figure 18 CDF of UL Throughput for 2GHz LEO600 with Urban Macro AAS interference

Victim LEO600 – Aggressor Rural macro

For LEO600, the UL SINR and UL throughput are shown as below.

Figure 19 CDF of UL SINR for 2GHz LEO600 with Rural Macro AAS interference

Figure 20 CDF of UL Throughput for 2GHz LEO600 with Rural Macro AAS interference

Victim LEO1200 – Aggressor urban macro

For LEO1200, the UL SINR and UL throughput are shown as below.

Figure 21 CDF of UL SINR for 2GHz LEO1200 with Rural Macro AAS interference

Figure 22 CDF of UL Throughput for 2GHz LEO1200 with Rural Macro AAS interference

Victim LEO1200 – Aggressor Rural macro

For LEO1200, the UL SINR and UL throughput are shown as below.

Figure 23 CDF of UL SINR for 2GHz LEO1200 with Rural Macro AAS interference

Figure 24 CDF of UL Throughput for 2GHz LEO1200 with Rural Macro AAS interference

Discussion

When satellite node receive the signal from satellite terminal, the adjacent channel interference from terrestrial Base Station or UE may have an impact on the UL throughput of satellite network. The uplink throughput loss of the victim satellite node with different ACS values are shown below.

Table 8 The uplink throughput loss of the victim LEO600 satellite node under Urban Macro scenario with terrestrial base station interference

| Satellite Node ACS [dB] | 34 | 38 | 42 |
| --- | --- | --- | --- |
| Average throughput baseline | 2.8834 | 3.0785 | 2.7259 |
| Average throughput with adjacent interference | 2.6248 | 2.9636 | 2.6813 |
| Average throughput loss | 8.97% | 3.73% | 1.64% |
| 5%-tile throughput baseline | 1.3427 | 1.4544 | 1.3069 |
| 5%-tile throughput with adjacent interference | 1.2558 | 1.4162 | 1.2846 |
| 5%-tile throughput loss | 6.47% | 2.63% | 1.71% |

Table 9 The uplink throughput loss of the victim LEO600 satellite node under Urban Macro scenario with terrestrial UE interference

| Satellite Node ACS [dB] | 34 | 38 | 42 |
| --- | --- | --- | --- |
| Average throughput baseline | 2.8834 | 3.0785 | 2.7259 |
| Average throughput with adjacent interference | 2.8634 | 3.0527 | 2.7091 |
| Average throughput loss | 0.69% | 0.84% | 0.62% |
| 5%-tile throughput baseline | 1.3427 | 1.4544 | 1.3069 |
| 5%-tile throughput with adjacent interference | 1.3338 | 1.4478 | 1.2982 |
| 5%-tile throughput loss | 0.66% | 0.45% | 0.67% |

Table 10 The uplink throughput loss of the victim LEO600 satellite node under Rural Macro scenario with terrestrial base station interference

| Satellite Node ACS [dB] | 34 | 38 | 42 |
| --- | --- | --- | --- |
| Average throughput baseline | 1.6845 | 1.6973 | 1.6569 |
| Average throughput with adjacent interference | 1.3582 | 1.5471 | 1.5857 |
| Average throughput loss | 19.37% | 8.85% | 4.30% |
| 5%-tile throughput baseline | 0.66601 | 0.67988 | 0.63311 |
| 5%-tile throughput with adjacent interference | 0.5257 | 0.63843 | 0.60861 |
| 5%-tile throughput loss | 21.07% | 6.10% | 3.87% |

Table 11 The uplink throughput loss of the victim LEO600 satellite node under Rural Macro scenario with terrestrial UE interference

| Satellite Node ACS [dB] | 34 | 38 | 42 |
| --- | --- | --- | --- |
| Average throughput baseline | 1.6845 | 1.6973 | 1.6569 |
| Average throughput with adjacent interference | 1.5375 | 1.6464 | 1.6346 |
| Average throughput loss | 8.73% | 3.00% | 1.35% |
| 5%-tile throughput baseline | 0.66601 | 0.67988 | 0.63311 |
| 5%-tile throughput with adjacent interference | 0.6021 | 0.6529 | 0.6215 |
| 5%-tile throughput loss | 9.60% | 3.97% | 1.83% |

Table 12 The uplink throughput loss of the victim LEO1200 satellite node under Rural Macro scenario with terrestrial base station interference

| Satellite Node ACS [dB] | 34 | 38 | 42 |
| --- | --- | --- | --- |
| Average throughput baseline | 0.61925 | 0.59945 | 0.61186 |
| Average throughput with adjacent interference | 0.57635 | 0.58408 | 0.60164 |
| Average throughput loss | 6.93% | 2.56% | 1.67% |
| 5%-tile throughput baseline | 0.40093 | 0.40044 | 0.40401 |
| 5%-tile throughput with adjacent interference | 0.36966 | 0.38537 | 0.39324 |
| 5%-tile throughput loss | 7.80% | 3.76% | 2.67% |

Table 13 The uplink throughput loss of the victim LEO1200 satellite node under Rural Macro scenario with terrestrial UE interference

| Satellite Node ACS [dB] | 34 | 38 | 42 |
| --- | --- | --- | --- |
| Average throughput baseline | 0.61925 | 0.59945 | 0.61186 |
| Average throughput with adjacent interference | 0.59209 | 0.59123 | 0.60658 |
| Average throughput loss | 4.39% | 1.37% | 0.86% |
| 5%-tile throughput baseline | 0.40093 | 0.40044 | 0.40401 |
| 5%-tile throughput with adjacent interference | 0.38395 | 0.3869 | 0.39745 |
| 5%-tile throughput loss | 4.24% | 3.38% | 1.62% |

Based on the simulation results, Rural-LEO scenario can be considered as the worst scenario. The interference from terrestrial Base station may have an impact on the UL throughput loss of satellite network. Since the UL throughput of satellite network is limited by transmit power of satellite UE, UL throughput loss is sensitive to the adjacent channel interference. Based on the simulation results, it’s proposed to specify [38~40] dB ACS for satellite node so that impacts on the satellite node are less than 5% throughput loss at 5%-tile CDF.

Proposal 3: It’s proposed to specify [38~40] dB ACS requirements for satellite node.

Victim satellite UE receiver

Victim LEO600 – Aggressor urban macro

For LEO600, the DL SINR and DL throughput are shown as below under the condition that satellite UE are dropped out of terrestrial network.

Figure 25 CDF of DL SINR for 2GHz LEO600 with urban Macro AAS interference

Figure 26 CDF of DL Throughput for 2GHz LEO600 with urban Macro AAS interference

Victim LEO600 – Aggressor Rural macro

For LEO600, the DL SINR and DL throughput are shown as below under the condition that satellite UE are dropped out of terrestrial network.

Figure 27 CDF of DL SINR for 2GHz LEO600 with Rural Macro AAS interference

Figure 28 CDF of DL Throughput for 2GHz LEO600 with Rural Macro AAS interference

Victim LEO1200 – Aggressor urban macro

For LEO1200, the DL SINR and DL throughput are shown as below under the condition that satellite UE are dropped out of terrestrial network.

Figure 29 CDF of DL SINR for 2GHz LEO1200 with urban Macro AAS interference

Figure 30 CDF of DL Throughput for 2GHz LEO1200 with urban Macro AAS interference

Victim LEO1200 – Aggressor Rural macro

For LEO1200, the DL SINR and DL throughput are shown as below under the condition that satellite UE are dropped out of terrestrial network.

Figure 31 CDF of DL SINR for 2GHz LEO1200 with Rural Macro AAS interference

Figure 32 CDF of DL Throughput for 2GHz LEO1200 with Rural Macro AAS interference

Discussion

Based on the simulation results, the impacts on the terrestrial network are less than 5% throughput loss at 5%-tile CDF when it’s assumed that ACS for Satellite UE is 33dB for all the scenarios. However, 33dB ACS requirement for satellite UE is overdesigned based on the terrestrial-satellite coexistence.

Observation 4: If 33dB ACS for satellite UE is specified to align with current industry implementation, 33dB ACS requirement for satellite UE is overdesigned based on the terrestrial-satellite coexistence.

Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: To deprioritize the GEO scenario when RAN4 study the ACLR and ACS requirements for satellite node and UE for satellite communication.

Observation 1: For TN-NTN adjacent coexistence scenario, RAN4 don’t need to consider the ACIR since the ACLR and ACS requirements for terrestrial network have been specified. The ACLR and ACS requirements for satellite node and UE can be considered directly.

Observation 2: Rural Marco – LEO (scenario case3) can be considered as the worst scenario when RAN4 specify the ACLR requirements for Satellite Node.

Proposal 2: It’s proposed to specify [20~22] dB ACLR requirements for satellite node.

Observation 3: If 30dB ACLR for satellite UE is specified to align with current industry implementation, 30dB ACLR requirement for satellite UE is overdesigned based on the terrestrial-satellite coexistence.

Proposal 3: It’s proposed to specify [38~40] dB ACS requirements for satellite node.

Observation 4: If 33dB ACS for satellite UE is specified to align with current industry implementation, 33dB ACS requirement for satellite UE is overdesigned based on the terrestrial-satellite coexistence.

- References

- R4-2115750, Simulation assumption for NTN co-existence study, Samsung, Thales

- R4-2108093, WF on [313] NTN_Solutions_Part2, Samsung

- R4-2115628, NR-NTN calibration summary and observations, Samsung

- R4-2113428, Initial NTN simulation Results, Huawei, HiSilicon

- R4-2112715, Initial NR-NTN co-ex study ACIR results, Samsung