3GPP TSG-RAN WG4 Meeting # 101-e                              R4-2118717

Electronic Meeting, 1– 12 August, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on UE RF requirements for satellite access

Agenda Item:	8.13.4.2

Document for:	Approval

Introduction

RAN4 can trigger the discussion for UE RF requirements in last meeting. In this paper, we’d like to discuss the remaining issue for UE RF requirements based on the approved WF [1].

Transmitter characteristics

1) General part

In TS 38.101-1, there is a general sentence to describe the antenna connector and assumption of antenna gain as below.

Proposal 1: It’s proposed to reuse this sentence to clarify the Rel-17 handheld UE for satellite access.

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

2) UL modulations

Based on the initial coexistence simulation results, it’s very hard to reach 15dB above SINR for UL. Based on RAN1’s link level simulation to evaluate the spectral efficiency and SNR for QPSK, 16QAM, 64QAM and 256QAM with 10% BELR, the results of the simulation are shown in Figure1, and summarized in Table 1.

Figure1 Spectral efficiency vs. SNR at 10% BLER

Table 1 Evaluation results

| MCS Index | Modulation Order | TBS Index | Spectral Efficiency(bps/Hz) | Coding Rate | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 0 | 0.19 | 0.0950 | -6.2 |
| 1 | 2 | 2 | 0.31 | 0.1550 | -3.9 |
| 2 | 2 | 4 | 0.49 | 0.2450 | -2.1 |
| 3 | 2 | 6 | 0.72 | 0.3600 | -0.5 |
| 4 | 2 | 8 | 0.97 | 0.4850 | 1.2 |
| 5 | 2 | 10 | 1.22 | 0.6100 | 2.9 |
| 6 | 4 | 11 | 1.40 | 0.3500 | 4.2 |
| 7 | 4 | 13 | 1.78 | 0.4450 | 5.7 |
| 8 | 4 | 14 | 1.99 | 0.4975 | 6.5 |
| 9 | 4 | 15 | 2.13 | 0.5325 | 7.2 |
| 10 | 4 | 16 | 2.26 | 0.5650 | 7.8 |
| 11 | 4 | 17 | 2.51 | 0.6275 | 8.6 |
| 12 | 4 | 18 | 2.75 | 0.6875 | 9.5 |
| 13 | 4 | 19 | 2.99 | 0.7475 | 10.4 |
| 14 | 6 | 20 | 3.23 | 0.5383 | 11.7 |
| 15 | 6 | 21 | 3.49 | 0.5817 | 12.6 |
| 16 | 6 | 22 | 3.72 | 0.6200 | 13.4 |
| 17 | 6 | 23 | 3.98 | 0.6633 | 14.1 |
| 18 | 6 | 24 | 4.24 | 0.7067 | 14.9 |
| 19 | 6 | 25 | 4.41 | 0.7350 | 15.4 |
| 20 | 6 | 27 | 4.62 | 0.7689 | 17.6 |
| 21 | 6 | 28 | 4.94 | 0.8029 | 18.3 |
| 22 | 6 | 29 | 5.12 | 0.8573 | 18.7 |
| 23 | 8 | 30 | 5.47 | 0.6816 | 20.8 |
| 24 | 8 | 31 | 5.64 | 0.7117 | 21.7 |
| 25 | 8 | 32 | 5.89 | 0.7340 | 22.1 |
| 26 | 8 | 33 | 6.80 | 0.8482 | 24.7 |
| 27 | 8 | 34 | 7.08 | 0.8852 | 25.6 |
| 28 | 8 | 35 | 7.37 | 0.9209 | 26.4 |

Observation 1: The expected SINR for UL 256QAM can’t be reached for PC3 satellite UE due to the lower UL SINR in the coexistence study.

4) UE additional maximum output power reduction

The additional maximum output power reduction framework for TN UE can be reused for satellite UE. However, satellite operators are encouraged to provide the additional spurious emission requirements based on the regulatory as soon as possible, so that any non-backward compatibility can be avoided.

Proposal 2: The additional maximum output power reduction framework for TN UE can be reused for satellite UE. However, the band-specific AMPR requirements depend on the input about the additional spurious emission.

5) Configured transmitted power

The Configured transmitted power framework for TN UE can be reused for satellite UE since RAN1’s power control framework isn’t changed. However, there is no need to specify some parameters which are only used for TN UE. That means the formula can be simplified.

Observation 2: The Configured transmitted power framework for TN UE can be reused for satellite UE. The formula can be further simplified for satellite UE.

6) Output power dynamics

For Minimum output power, Transmit OFF power and Power control, the framework and requirements for TN UE can be reused for satellite UE.

Proposal 3: The requirements for Minimum output power, Transmit OFF power, Transmit ON/OFF time mask and Power control in clause 6.3 from 38.101-1 are applicable for the satellite UE.

7) Transmit modulation quality

The requirements of TN UE can be reused for satellite UE including EVM, carrier leakage and in-band emissions.

Proposal 4: The requirements for Frequency error and transmit modulation quality in clause 6.4 from 38.101-1 are applicable for the satellite UE.

8) Output RF spectrum emissions

The ACLR requirements should wait for the outcome of coexistence study. For general spurious emissions, Category B limits [3] for type Land mobile service (mobiles and base stations) are reused for current TN UE requirements. If there is no additional input, the general spurious emissions can be applicable for satellite UE.

Proposal 5: The requirements for occupied bandwidth, SEM, general spurious emissions and transmit intermodulation are applicable for the satellite UE.

If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence. It depends on the satellite operators’ input.

Observation 3: If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence.

Receiver characteristics

1) Reference sensitivity

For below 6GHz, the REFSENS level can be calculated by the equation below [4]:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was agreed as -1dB, the value is the same as that for LTE. However, SNR will be further evaluated by link level simulation. Actually, the SNR under AWGN channel has no difference for different SCS and the demodulation performance for DFT-s-OFDM and CP-OFDM is the same as well. Therefore, the same SNR will be used to calculate REFSENS for different SCS.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for LTE refarming bands for NR.

The Noise Figure for S-band and L-bandis assumed as 9dB.

Proposal 6: To specify the REFSENS as below.

Table 2: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| s/n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
| s/n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |

Table 3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| s/n256 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| s/n255 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |

The FRC for receiver requirements for QPSK in clause A.3.2.2 from TS 38.101-1 are applicable for the satellite UE.

2) Maximum input level

Even if the minimum distance between satellite and UE for all the scenario is larger than 35m, the distance between aggressor Macro BS and satellite should be assumed and considered as well. Thus, the Maximum input level for satellite UE should be specified.

Proposal 7: The requirements for Maximum input level in clause 7.4 from TS 38.101-1 are applicable for satellite UE.

3) Blocking characteristics and spurious response

In-band blocking, out-of-band blocking and narrow band blocking requirements can be reused for satellite UE.

Proposal 8: The requirements for Blocking characteristics in clause 7.6 and spurious response in clause 7.7 from TS 38.101-1 are applicable for satellite UE.

4) Intermodulation characteristics

Proposal 9: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE.

- Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: It’s proposed to reuse this sentence to clarify the Rel-17 handheld UE for satellite access.

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Observation 1: The expected SINR for UL 256QAM can’t be reached for PC3 satellite UE due to the lower UL SINR in the coexistence study.

Proposal 2: The additional maximum output power reduction framework for TN UE can be reused for satellite UE. However, the band-specific AMPR requirements depend on the input about the additional spurious emission.

Proposal 3: The requirements for Minimum output power, Transmit OFF power, Transmit ON/OFF time mask and Power control in clause 6.3 from 38.101-1 are applicable for the satellite UE.

Proposal 4: The requirements for Frequency error and transmit modulation quality in clause 6.4 from 38.101-1 are applicable for the satellite UE.

Proposal 5: The requirements for occupied bandwidth, SEM, general spurious emissions and transmit intermodulation are applicable for the satellite UE.

Observation 3: If it’s assumed that the NR NTN is used in the area that there is no IMT BS coverage, there is no need to specify the spurious emissions for UE co-existence.

Proposal 6: To specify the REFSENS as below.

The FRC for receiver requirements for QPSK in clause A.3.2.2 from TS 38.101-1 are applicable for the satellite UE.

Table 2: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| s/n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
| s/n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |

Table 3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| s/n256 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| s/n255 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |

Proposal 7: The requirements for Maximum input level in clause 7.4 from TS 38.101-1 are applicable for satellite UE.

Proposal 8: The requirements for Blocking characteristics in clause 7.6 and spurious response in clause 7.7 from TS 38.101-1 are applicable for satellite UE.

Proposal 9: The requirements for Intermodulation characteristics in clause 7.8 from TS 38.101-1 are applicable for satellite UE.

- References

- R4-2115642, WF on NTN UE RF requirement, Huawei, HiSilicon