3GPP TSG-RAN WG4 Meeting # 101-e                                                                                         R4-2118718

Electronic Meeting, 1–12 November, 2021

Agenda item:	8.13.4.1

Source:	Huawei

Title:	TP for 38.863 on NTN UE transmission characteristics

Release:	Rel-17

Document for:	Approval

1	Introduction

Based on the TR 38.863 work split, we provided a text proposal for NTN UE transmission characteristics in this paper.

2	Text Proposal

<< Start of text proposal  >>

6.3.2	NTN UE transmission characteristics

6.3.2.1 	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed. Handheld PC3 UE is assumed in Rel-17.

6.3.2.2	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.3.2.1-1: UE Power Class

| NR band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| s/n256 | 23 | ±2 |
| s/n255 | 23 | +2/-33 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

6.3.2.3 	MPR/AMPR

TBD.

6.3.2.4 	Output power dynamics

For Minimum output power, Transmit OFF power and Power control, the framework and requirements for existing TN UE can be reused for satellite UE. The minimum output power is -40 dBm for 20 MHz channel bandwidth. The transmit OFF power is -50 dBm for 20MHz.

6.3.2.5 	Frequency error

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the Satellite Node.

6.3.2.6 	Transmit modulation quality

TBD.

6.3.2.7 	Spectrum emission mask

TBD.

6.3.2.8 	ACLR

TBD.

6.3.2.9 	Spurious emissions

TBD.

<< end of text proposal  >>