3GPP TSG-RAN WG4#101-e Meeting	R4-2118771

Electronic Meeting, November 1-12, 2021

Agenda item:		8.4.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Requirements and test cases of UE FR2 UL Gap for UE Tx power enhancement

Document for:		Approval

# Introduction

In RAN4 #100-e, a WF [1] on introduction of UL gaps for proximity sensing was agreed, with the following agreements for the RF and UL MIMO aspects:

Way forward – RF Agreements: - Baseline is to verify that UE correctly behave without phantom and ensure the feasible requirement gain in Rel-17 with different test methods. - “P-MPR report + peak EIRP without phantom”, X dB EIRP gain and P-MPR requirement of Y when UL gap is activated should be achieved compared to the case where no gap is activated - Decide range for X value in this meeting for making decision in future meeting - Option 1: at least 6dB - Option 2: A value between 6dB and 3dB, which is typical in the field - Further discussion on the definition of Y in this meeting - Option 1: Y is absolute value - Option 2: Y is the relative value of gain - Option 3: no P-MPR requirement of Y - FFS on the implementation margin Way forward – Coherent UL MIMO Agreements: WF on performance gain evaluation: - The gain of UL MIMO with UL gap configured has been shown by simulation with UL throughput, but it has not yet been shown how to achieve the same throughput gains in the UE requirements. For the feasibility of UL gaps for UL coherent MIMO it is necessary to show that performance gains are obtained in the UE requirements. - Observations: - In R4-2111383, it shows 20.3% mean throughput gain and maximum 40.7% throughput gain with 40 degree phase error. - In R4-2114492, it shows further 8% throughput gain can be reached by further improve relative phase error requirement. - Study how to ensure throughput gains in the UE requirements. Identify metric(s) for the UE requirement gain, which ensures that gains are achieved if UL gaps are used. Identify how the requirements and tests could be defined to ensure performance gains in the requirements. Throughput gain in the UE requirements is evaluated and throughput gain in the UE requirements with UL gaps compared to the case without UL gaps needs to be achieved to justify UL gaps. - Companies are encouraged to provide analysis for gains achieved for gap pattern examples - Other gap pattern is not precluded

In this contribution, Section 2 presents our views on X and Y values for the Peak EIRP + P-MPR test for UL gaps for UE Tx power enhancements. Furthermore, we propose a test case for ensuring that UE Tx power enhancements using UL gaps are for body proximity sensing, i.e. to ensure that the UL performance gain is obtained from the introduction of UL gaps for proximity sensing as opposed to other internal calibrations. In Section 3, we also discuss Coherent UL MIMO enhancements. Finally, in Section 5, we propose TP of TR38.851 Section 6 on UL gaps.

# UE requirements and test case(s) for UE Tx power enhancement with body proximity sensing

This section discusses the requirements and test case(s) for UE MPE related P-MPR improvement based on accurate user detection with help of UL gaps. Solid UE requirements and test cases are needed to ensure that sufficient performance gains are obtained from the introduction of UL gaps for proximity sensing.

It is important that the test verifies that the UL gaps are yielding enough UE Tx power gain, as is the case with the radar functionality of body proximity sensing, and that UEs equipped with other means of detecting MPE events (e.g. infra-red sensors, etc.) may not use the UL gaps for alternative calibrations in order to avoid any unnecessary UL gap time. Indeed, the UE could also be equipped with other types of proximity sensors, e.g. infra-red sensors, capacitive sensors, or others. It should be avoided that such UEs may then utilize other techniques to achieve P-MPR improvement while exploiting the UL gaps for another calibration than body proximity sensing.

It was discussed in the previous meeting that peak EIRP measurement is not a reliable metric with blocking, as the blocking material itself may affect the peak power measurement. Indeed, the introduction of a material may affect the radiation pattern of the transmitting array, thereby the received UL power. Therefore, RAN4#100-e agreed on a Free Space (FS) test.

Therefore, we propose the following options in relation to the WF: “P-MPR report + peak EIRP without phantom”, X dB EIRP gain and P-MPR requirement of Y when UL gap is activated should be achieved compared to the case where no gap is activated”

For the value of X, the EIRP gain needs to show sufficient value from the introduction of the gaps, as such we support Option 1: at least 6 dB.

For the value of Y, the P-MPR reporting with and without gaps needs to show sufficient improvement in connection with peak EIRP improvement to verify accurate P-MPR reporting, as such we support Option 2: Y is the relative value of gain.

Proposal 1: Support X being at least 6 dB (Option 1).

Proposal 2: Support Y being the relative value of gain (Option 2).

The test aims at ensuring that the gaps will benefit the particular use case of MPE and ensure sufficient P-MPR improvement for radar UEs.

Nonetheless, the test procedure needs to prevent artificial gain for example if a UE is backing off more power than needed during the test. Indeed, it can be simulated that different array designs (e.g. 1x4 vs 1x8) will exhibit different EIRP values to comply with the maximum allowed Power Density (PD) under MPE event (i.e. 1mW/cm2, averaged over an area of 4 cm2 (2x2cm)). Furthermore, the lower the allowed EIRP value under MPE event is, the larger the P-MPR improvement can be for a given maximum peak EIRP capability of the UE; therefore, this value needs to be checked in the test to avoid artificially increasing the P-MPR improvement during the test procedure. The test must prevent UEs showing an artificially high gain in P-MPR report enhancements by setting the minimum value to a lower threshold than actually needed. Therefore, the peak EIRP value measured without the UL gap must remain above a certain limit to ensure performance gain. For example, such limit could be in agreement with FCC test or agreed in RAN4 per array type e.g. 1x4 vs 1x8 arrays.

Proposal 3: Ensure value of peak EIRP without the gaps is above a threshold, e.g. above 23 dBm.

As such, combining peak EIRP measurements with P-MPR reporting in the procedure enables 3GPP to verify:

1.	Peak EIRP measurement with and without UL gaps is sufficient given the array design e.g. 1x4 array or 1x8 array

2.	P-MPR improvement reporting is matching the peak EIRP reporting

Figure 1 plots the curves of the simulated maximum allowed peak EIRP (at 20 % UL duty cycle to represent the common slot configuration of DDDSU) for simulations with a 1x4 linear array, a 2x2 square array and a 1x8 linear array. The array aperture compared to the MPE evaluation area of 2x2 cm² yields a difference of at least 3 dB. As such P-MPR enhancements may be larger for a 1x4 array design than for a 1x8 array design.

Min PC3: 22.4 dBm

Figure 1: Simulated maximum allowed peak EIRP for 20 % UL duty cycle for two types of array design

Furthermore, in order to ensure the P-MPR improvement is a result on the MPE detection, it is important to monitor also the UE Tx power during the gaps.

As such, we propose to cover in the test procedure the two following aspects:

- Peak EIRP without the gaps must be above a threshold

- Monitoring EIRP during the gaps

While the first aspect is to ensure no unnecessary backoff is performed during the test and sufficient P-MPR enhancement is achieved as a result from the scheduling of the UL gaps, the second aspect ensures that the improvement is indeed related to body proximity sensing and no other internal calibration is performed (above a certain power level e.g. emission mask requirement) during the gaps.

The proposed procedure is in free space and as shown in Figure 2:

Figure 2: Proposed procedure to evaluate UE P-MPR enhancements resulting from the UL gap scheduling.

In the first step, the UE is in FS and no UL gaps are scheduled, therefore the radar functionality may not be conclusive. As such, it is expected that the UE exhibits a default P-MPR value larger than 3 dB ensuring MPE compliance, even though it is in FS. As a reference, sequence 1 calculates the value UL peak EIRP P_nogap and P-MPR_nogap in corresponding PHR report.

In sequence 2, the UE is scheduled with UL gaps, the peak EIRP of the UE during the gaps is measured P_ingaps in order to verify that the transmission in-band is below the required emission mask during the gap and no other internal calibration is on-going. For the next UL transmission scheduled after the UL gap, the peak EIPR is again measured P_aftergaps and the corresponding P-MPR report is analyzed P-MPR_gaps.

In the third step, all previously measured values are compared:

|  | Peak EIRP values | P-MPR report |
| --- | --- | --- |
| Before UL gap | P_nogap | P-MPR_nogap |
| During UL gap | P_ingaps | N/A |
| After UL gap | P_aftergaps | P-MPR_gaps |

The difference between P_nogap and P_aftergaps should show sufficient improvement (e.g. 6 dB) to justify scheduling gaps for this UE.

In the outcome of the test where the UE is in power limitation and with large UL duty cycle, it is expected that for radar equipped UEs: P-MPR(PHR_FSnogaps) = P-MPR(PHR_Mat_nogaps) < P-MPR(PHR_Mat_gaps); and, for UEs equipped with other means for MPE detection P-MPR(PHR_FSnogaps) = P-MPR(PHR_Mat_nogaps) = P-MPR(PHR_Mat_gaps).

Proposal 4: Support measuring UE in-band Tx power during the gaps.

The test descriptions do not aim at mandating a specific implementation, only at ensuring that the gaps will benefit the particular use case of MPE and ensure a P-MPR improvement for radar UEs.

Proposal 5: Consider an extra test to validate UEs fulfill current requirements even without UL gaps scheduled.

Test cases for enhancements without the gaps could also be considered for other type of UEs, i.e. UEs not equipped with radar functionality and embedding other means of body proximity sensing e.g. infra-red sensors, capacitive sensors, etc. Test with and without phantoms may be considered for such case. In this way also these UEs can show and support similar performance gains with different implementation choice and without need for UL gaps.

Observation 1: Another test may be defined for UEs embedding other means than radar for body proximity sensing.

After considering the testing aspects, the discussion should include how to write the core requirements while also matching with the selected testing approach.

Proposal 6: Further discussion how to define UE core requirements in addition to the testing aspects.

TP to TR 38.851 Section 6 is proposed in Annex.

# UL gap for coherent UL MIMO

Indeed, we would like to ensure that there are real UE requirement gains if UL gaps for UL MIMO coherency purpose are introduced, since the aim of this item is to improve the UE performance. This item needs to show performance improvement resulting from UL gap introduction, rather than from UL MIMO capability itself. The currently analyses have not yet shown the gains in the UE requirements, but only that coherent UL MIMO provide gains against non-coherent UL MIMO. The gain should be shown between coherent MIMO without the gaps and coherent MIMO with the gaps.

Observation 2: Gains should be shown between coherent MIMO without the gaps and coherent MIMO with the gaps.

In the WF [2], the gap type is agreed as:

•	Type 1: No UL scheduling during the gap is needed. NW can assign those resources to other UE for UL transmission.

•	Type 2: UL scheduling, including dedicated time and frequency resources reserved for self-calibration and monitoring, during the gap is needed. NW cannot assign those resources to other UE for UL transmission.

While the impact on UL degradation is clear for type 1 and type 2, the performance gain is what should be studied further. Type 2 may be used for UL MIMO and will minimize UL degradation if gains in UE requirements are shown and later ensured.

# Conclusion

In this contribution, we made the following proposals and observations:

Proposal 1: Support X being at least 6 dB (Option 1).

Proposal 2: Support Y being the relative value of gain (Option 2).

Proposal 3: Ensure value of peak EIRP without the gaps is above a threshold, e.g. above 23 dBm.

Proposal 4: Support measuring UE in-band Tx power during the gaps.

Proposal 5: Consider an extra test to validate UEs fulfill current requirements even without UL gaps scheduled.

Observation 1: Another test may be defined for UEs embedding other means than radar for body proximity sensing.

Proposal 6: Further discussion how to define UE core requirements in addition to the testing aspects.

Observation 2: Gains should be shown between coherent MIMO without the gaps and coherent MIMO with the gaps.

# References

- R4-2114964, WF on FR2 enhancement part 2: UL gaps, Apple (moderator), August 2021

- R4-2103114, “WF on FR2 enhancement part 3: UL gap”, Apple, RAN4 #98-e

# Annex : TP for TR38.851 Section 6

# UL gaps for self-calibration and monitoring

## 6.1	Phase 1

### 6.1.1	Study of the performance gain over the current baseline

### 6.1.2	Study of RF performance evaluation/testability related to UE self-calibration and monitoring

The test aims at ensuring that the gaps will benefit the particular use case of MPE and ensure sufficient P-MPR improvement for radar UEs. It is important that the test verifies that the UL gaps are yielding enough UE Tx power gain. The test procedure needs to prevent artificial gain for example if a UE is backing off more power than needed during the test.

Sequence 1 calculates the value UL peak EIRP P_nogap and P-MPR_nogap in corresponding PHR report. In sequence 2, the UE is scheduled with UL gaps, the peak EIPR is again measured P_aftergaps and the corresponding P-MPR report is analyzed P-MPR_gaps. Furthermore, the peak EIRP of the UE during the gaps is measured P_ingaps in order to verify that the transmission in-band is below the required emission mask during the gap and no other internal calibration is on-going. The verification sequence checks that the difference between P_nogap and P_aftergaps should show sufficient improvement, that the P_nogap is in accordance with FCC measurements, that P_ingap is below a threshold, that P-MPR enhancement matches peak EIRP enhancements.

### 6.1.3	Study network impact of UE emissions during UL gap

The test also verifies that the improvement is indeed related to body proximity sensing and no other internal calibration is performed during the gaps by monitoring Peak EIRP values during the UL gaps.

## 6.2	Phase 2

### 6.2.1	Specification related aspects

#### 6.2.1.1	UL gap configuration(s)

#### 6.2.1.2	UE capability

#### 6.2.1.3	UE interruptions

### 6.2.2	UE fall back behaviour i.e. if gaps are not available for UE requesting gaps

### 6.2.3	Release independence aspects