| 3GPP TR 38.844 V0.0.5 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for Styles	8

4.1	Heading Styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <B>: <Informative annex title for a Technical Report>	15

Annex <C> (informative): Bibliography	16

Annex <D> (informative): Index	17

Annex <X> (informative): Change history	18

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

One of the aims of 5G is providing bandwidth flexibility. Although this is achieved in general, but for some spectrum allocations the ability to achieve such flexibility needs further study.

# 1	Scope

The present document is the Technical Report of the Study Item on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, approved at TSG RAN #89-e [2]. The purpose of this document is to capture and document the outcome of the objectives stated in the SID.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-202103: SID “Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Existing immediately lower regular channel bandwidth: the closest NR channel bandwidth defined in Rel-17 which is smaller/less than the irregular bandwidth

Existing immediately wider regular channel bandwidth: the closest NR channel bandwidth defined in Rel-17 which is larger/wider than the irregular bandwidth

Irregular bandwidth: an NR bandwidth that is not defined in Rel-17

Overlapping UE channel BW from network perspective: network supports the irregular bandwidth (either by a single carrier or by two overlapping carriers) while each UE operates in an existing lower regular NR channel bandwidth

Overlapping UE channel BW from UE perspective: network supports the irregular bandwidth while some new UEs support two overlapping (RF) carriers

Overlapping CA: the irregular bandwidth is handled by two overlapping component carriers (CCs) with NR channel bandwidth defined in Rel-17. It is network responsibility to prevent collisions between the different component carriers.

Single BB carrier: means that from baseband (RAN1) perspective, there is a single cell with a waveform according to a single carrier

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

BS	Base Station

BW	Bandwidth

CBW	Channel Bandwidth

SCBW	Smaller Channel Bandwidth (Existing immediate lower channel bandwidth)

FR1	Frequency Range 1

RF	Radio Frequency

WCBW	Wider Channel Bandwidth (Existing immediate wider channel bandwidth)

UE	User Equipment

# 4	Background

One of the aims of 5G is providing bandwidth flexibility.  Although this is achieved in general, but for some spectrum allocations the ability to achieve such flexibility needs further study.

Solutions for the following spectrum allocations have been requested so far:

Table 4-1: Summary of operators’ input for irregular channel bandwidth

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

Some techniques have been suggested for re-using existing channel bandwidths which can include but not limited to overlapping UE channel bandwidths, and/or using larger bandwidths than operator licensed bandwidth. This Study Item is needed to evaluate where existing techniques can be used to efficiently utilize operator spectrum allocations, and whether and how new channel bandwidths should be created. The Study shall also analyse if a proprietary solution(s) is sufficient.

## 4.1	Objectives

The following objectives are listed in the SID [2]

1) Identify operator licensed channel bandwidths in FR1 that do not align with existing NR channel bandwidths.

- Only licensed spectrum wider than 5 MHz to be considered in this SID.

- Spectrum block of 33MHz in n28 require further investigation since there is dual duplexer assumption (2x30MHz) for this band.

2) Evaluate the potential use of larger channel bandwidths than operator licensed bandwidth, including the impacts on regulatory emission requirements/UE output power implications and UE ACS/blocking impacts depending on the guard band and the SCS.

3) Study the use of overlapping UE channel bandwidths (from both UE and network perspective) to cover operator’s license spectrum for both UL and DL, and if new gNB channel bandwidths are needed.

NOTE:	For all considered solutions, new (dedicated) channel filters (e.g. non-integer-multiples of 5MHz) are not considered for the UE and not prioritized for the gNB.

4) Identify operator licensed bandwidths that are not compatible with the use of techniques like overlapping UE channel bandwidths. Every proposed method shall be summarized with respect to whether all considered spectrum scenarios are supported or whether there are specific limitations. Some limitations for a specific method shall not disqualify such method if there is a trade-off between flexibility and implementation challenges.

5) Study the complexity and efficiency of adding new channel bandwidths vs. using other including testing aspects.

6) Generic solution(s) should be intended as much as possible, with priority should be given to approaches that avoid the introduction of new channel BWs on the UE side. Proprietary solutions if proven relevant should not be precluded. Spectrally efficient methods providing a fine channel bandwidth granularity as well as low to moderate guard band width and signalling overhead should be preferred

7) Impact on RAN1 and RAN2 should be considered and minimized

8) For any considered solution, UEs not supporting such solution (both legacy and new UEs) should be able to use the next lower supported channel bandwidth in the UL and DL without implications.

9) Impact (if any) on RAN4 requirements should be identified for the preferred solutions.

# 5	General

## 5.1	UE channel bandwidth

The following text is copied from TS38.101-1 for information:

The UE channel bandwidth supports a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

# 6	Result, Analysis outcome

## 6.1	Study of larger Channel BW than licensed BW

### 6.1.1	General Aspects

This clause describes, in general terms, how to utilize an irregular Channel Bandwidth by deploying the “larger channel Bandwidth” method.

The premise idea is that the system is configured with the larger channel bandwidth (indicated in System Information broadcasts as well as gNB filter configurations), but the actual number of scheduled RBs is restricted so that it matches actual spectrum allocation ensuring sufficiently large guard bands.

Figure 6.1.1-1: Using the next larger channel bandwidth (example for 7MHz).

One of the first critical aspects for this approach is the size of guard bands and the anticipated number of schedulable RBs. As for the standard channel bandwidths, both values are captured in the corresponding specification to avoid any misinterpretation on how many RBs can be configured and scheduled. Following the same principle for every irregular channel bandwidth is feasible, but that will create same amount of technical specification work as if the corresponding irregular channel bandwidth were explicitly added to the specifications. Thus, the number of "available" RBs can be calculated based on certain assumptions.

The maximum number of "available" or "schedulable" RBs for a particular irregular channel bandwidth can be calculated based on the assumption of using larger guard bands from the next larger channel bandwidth. As an example, while considering the 7MHz channel bandwidth, it is safe to assume next larger 10MHz channel guard bands, from which number of available RBs can be calculated.

NOTE:	Since a UE will be configured with the channel bandwidth, which is larger than the actual allocation, and it is not expected to provide the usual stop-band attenuation at the edges of the irregular channel bandwidth, it is necessary to verify the level of potential degradation of ACS/blocking. Similarly, the gNB operating with wider channel filters cannot be expected to provide stop-band attenuation at the edges of the irregular channel bandwidth to guarantee the co-existence. Further information on gNB transmit channel filters and ACS/blocking should be provided to assess resulting performance degradation and the gap to the RF performance requirements. The gNB Tx transmitter filter assumption is FFS.

Table 6.1.1-1 below presents example maximum number of available RBs for different irregular channel bandwidths considered in this study item.

NOTE:	Number of available RBs and spectral utilisation are taken from R4-2112365. The gNB Tx transmitter filter assumption used to derive the RB numbers is FFS.

Table 6.1.1-1: Exemplary number of RBs based on the next larger channel guard bands (15kHz SCS).

| Channel (MHz) | Next larger channel (MHz) | Next larger channel guard band (kHz) | Next larger channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 10 | 312,5 | 52 | 29 | 87 |
| 7 | 10 | 312,5 | 52 | 35 | 90 |
| 11 | 15 | 382,5 | 79 | 56 | 91,6 |
| 12 | 15 | 382,5 | 79 | 62 | 93 |
| 13 | 15 | 382,5 | 79 | 67 | 92,8 |

Table 6.1.1-2 below presents similar calculations for 30kHz SCS, from which one can see that combination of 30kHz SCS and the next larger channel is not generally a good approach for small channel bandwidths. The main reason is that 30kHz SCS has much larger guard bands, which immediately impacts number of available RBs. As a small summary, assuming using guard band from the next larger channel the resulting spectrum Utilization would range from 87 to 92.8% for an SCS of 15kHz and 72 to 88.6% for an SCS of 30kHz.

NOTE:	Number of available RBs and spectral utilisation are taken from R4-2112365. The gNB Tx transmitter filter assumption used to derive the RB numbers is FFS.

Table 6.1.1-2: Exemplary number of RBs based on the next larger channel guard bands (30kHz SCS).

| Channel (MHz) | Next larger channel (MHz) | Next larger channel guard band (kHz) | Next larger channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 10 | 665 | 24 | 12 | 72 |
| 7 | 10 | 665 | 24 | 15 | 77,1 |
| 11 | 15 | 645 | 38 | 26 | 85,1 |
| 12 | 15 | 645 | 38 | 29 | 87 |
| 13 | 15 | 645 | 38 | 32 | 88,6 |

### 6.1.2	Signalling and configuration aspects

In this section we provide further signaling details on how to support irregular channels given the 7MHz allocation as an example.

The gNB broadcasts the DL carrier bandwidth and the bandwidth of the initial BWP (BWP#0) in SIB1. For the 7MHz allocation, SIB1 can indicate DL next larger standard channel bandwidth, i.e. 10 MHz, and that the initial DL BWP can be set to 5 MHz:

-	SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth = 52 PRBs / subcarrierSpacing = 15 kHz

-	SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> initialDownlinkBWP-> genericParameters-> locationAndBandwidth = 25 PRBs

Once the UE established the RRC connection, the gNB can account for the UE capabilities and re-configure the UE accordingly. At this point the gNB may override the carrier bandwidth value that the UE obtained from SIB1 and configure a dedicated BWP with a bandwidth that differs from the bandwidth of BWP#0. gNB may configure a larger bandwidth part that will cover the whole 7MHz allocation.

-	ServingCellConfig-> downlinkChannelBW-PerSCS-List-> carrierBandwidth = 52 PRBs, subcarrierSpacing = 15 kHz

-	ServingCellConfig-> downlinkBWP-ToAddModList-> bwp-Common-> genericParameters-> locationAndBandwidth = TBD PRBs

## 6.2	Study of overlapping UE channel bandwidths

### 6.2.1	Overlapping UE CBW

#### 6.2.1.1      General

One way to utilise the whole chunk of irregular spectrum of a particular size is to combine several overlapping channels of next lower standard channel bandwidth. As an example, Figure X-1 shows a case when two overlapping 10MHz carriers cover 13MHz channel bandwidth. From an individual UE perspective, each UE is configured with existing immediately lower channel bandwidth following legacy procedures and signalling: one UE can use the first 10MHz carrier, while another UE can use another carrier. In fact, both UEs can use overlapping part of the spectrum provided that the BS takes care that the overlapping region is allocated to one particular carrier at a time. It should be also noted that from the UE perspective, an existing immediately lower channel bandwidth will be always used, either for initial access (as the channel bandwidth advertised by the network) or as a dedicated channel bandwidth configured by RRC. From the network perspective, the BS will/can use the whole irregular channel bandwidth.

Figure 6.2.1.1-1: Using overlapping carriers (example for 13MHz).

It is worth noting that overall capacity of the cell will be according to the irregular channel bandwidth because the BS can use the full bandwidth. However, since a particular UE will use only one carrier of a smaller bandwidth within the irregular channel bandwidth, the maximum throughput for a single UE will be less than the theoretically possible within the spectrum in case there is only a single UE in the cell. Nevertheless, since there will be multiple UEs in the cell the overall system throughput will not decrease.

#### 6.2.1.2      Detailed description

One of the challenges associated with configuring overlapping carriers for the same spectrum is that both carriers should have aligned grid so that the BS can perform same FFT and schedule resources in the overlapping region.  Moreover for alignment the initial BWP would be beneficial from BS coordination efforts to keep aligned between the UE dedicated channel bandwidth.

Figure 6.2.1.2-1: Using overlapping carriers with single overlapping SSB (example for 13MHz).

Another challenge while aligning RB grids is not an issue for bands above 3GHz that have the SCS based raster, it becomes more challenging for the sub-3GHz band that have 100kHz raster. As a result, carriers can be configured on raster points that correspond to the least common multiple of the channel raster and the RB size. As an example, the least common multiple will be 900kHz in case of the 15kHz SCS, which corresponds to 5RBs. It effectively means that overlapping carriers will not be able to address efficiently any irregular spectrum size and in some case maybe will not be applicable at all. Of course one way to improve spectrum utilisation is to allow shifting carriers in multiples of 1RB, but that will require introduction of new raster points, which will not be supported by legacy UEs.

Figure 6.2.1.2-2 presents an example for the 6MHz channel comprising two 5MHz channels. As can be seen from the figure, centre frequency distance between carriers is 900kHz, which is a multiple of 100kHz channel raster and 180kHz RB size. From an individual UE perspective, it is just a normal 5MHz carrier comprising 25RBs and having the 5MHz channel guard bands. From the BS perspective, it is a 6MHz channel with 30RBs. Figure 6.2.1.2-3 exemplifies how this approach can be used to support the 7MHz irregular channel bandwidth, in which the distance between the carriers is 2RBs i.e. 1800kHz. Finally, Figure 6.2.1.2-4 shows the 11MHz channel that is supported with two 10MHz channels.Referring to Figure 6.2.1.2-2, 6.2.1.2-3 and 6.2.1.2-4, it should be noted that guard bands will not necessarily be symmetrical and the exact guard band size will depend on a particular spectrum allocation, its size, and how the overlapping channels are placed.

Figure 6.2.1.2-2: Detailed overview of overlapping carriers (6MHz channel with 5MHz carriers).

Figure 6.2.1.2-3: Detailed overview of overlapping carriers (7MHz channel with 5MHz carriers).

Figure 6.2.1.2-4: Detailed overview of overlapping carriers (11MHz channel with 10MHz carriers).

Table 6.2.1.2-1 below summarises potential number of schedulable RBs for a scenario when the next smaller overlapping channels are used. To calculate them, it is assumed that distance between individual carriers is a multiple of 900kHz and that the resulting guard bands must meet at least next smaller channel requirements. So, "Channel Nrb", "Channel guard bands", and "Utilisation" represent the network view, while from the UE perspective all the parameters are the same as for the next smaller channel.

Table 6.2.1.2-1: Exemplary number of RBs based on the next smaller overlapping channel (15kHz SCS).

| Channel (MHz) | Next smaller channel (MHz) | Next smaller channel guard band (kHz) | Next smaller channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 5 | 242,5 | 25 | 30 | 90 |
| 7 | 5 | 242,5 | 25 | 35 | 90 |
| 11 | 10 | 312,5 | 52 | 57 | 93,3 |
| 12 | 10 | 312,5 | 52 | 62 | 93 |
| 13 | 10 | 312,5 | 52 | 67 | 92,8 |

Table 6.2.1.2-2 presents similar calculations for the number of available RBs with overlapping carriers, but for the 30kHz SCS. As can be seen from the table, a solution based on the 30kHz SCS overlapping carriers does not provide a good spectral utilisation for certain non-standard channel bandwidths due to the reason that the "distance" between carriers must be a multiple of 1800kHz. Because of that, channel bandwidths such as 7 and 12MHz have more or less good utilisation, whereas 6 and 11MHz do not provide any benefit at all.

Table 6.2.1.2-2: Exemplary number of RBs based on the next smaller overlapping channel (30kHz SCS).

| Channel (MHz) | Next smaller channel (MHz) | Next smaller channel guard band (kHz) | Next smaller channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 5 | 505 | 11 | 11 | 66 |
| 7 | 5 | 505 | 11 | 16 | 82,3 |
| 11 | 10 | 665 | 24 | 24 | 78,5 |
| 12 | 10 | 665 | 24 | 29 | 87 |
| 13 | 10 | 665 | 24 | 29 | 80,3 |

To suppport two overlapping carriers, the network can consider using one or two SSBs. To be more precise, since the the SSB bandwidth is 3.6MHz and the CORESET#0 bandwidth is 4.32MHz, a single SSB/CORESET#0 can be placed into a common part between two overlapping channels. However, this approach works only if the overlapping part is at least 4.32MHz, e.g. it is not applicable to 7MHz irregular channels. So, some irregular channel bandwidth will needed two SSB/CORESET#0. In this case, at least for irregular channel bandwidths <10MHz the network broadcasts two separate SSBs, one for each overlapping regular carrier. In case of irregular channels of a small size, e.g. less than 10MHz, it may create the challenge of aligning SSBs in the time and frequency domain so that they do not overlap thus complicating the gNB scheduling. i.e. for this approach complexity increases to coordinate the overlapping SSB for different UEs. As an example, if a particular irregular channel bandwidth does not allow for placing two SSBs in the same time slots, then the network will have two ensure that they are "multiplexed" accordingly in the time domain.

Table 6.2.1.2-3: Summary of how many SSB/CORESET#0 needed for different channel bandwidths.

| Channel (MHz) | Number of SSB | Time multiplexing for 2 SSBs is needed |
| --- | --- | --- |
| 6 | 1 or 2 | yes |
| 7 | 2 | yes |
| 11 | 1 or 2 | no |
| 12 | 1 or 2 | no |
| 13 | 1 or 2 | no |

This approach works with all the legacy UEs. As mentioned earlier, from an individual UE perspective, this is just a standard Rel-15 channel and no special UE side enhancements are needed. Thus an operator can use this solution with the whole ecosystem of available devices.

#### 6.2.1.3      Configuration and signalling aspects

Since from an individual UE perspective each overlapping carrier is just a legacy carrier, the existing signalling applies. As an example, for the 7MHz allocation the UE can be configured with the 5MHz channel bandwidth, and the initial bandwidth part can be also 5MHz.

-	SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth = 25 PRBs / subcarrierSpacing = 15 kHz

-	SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> initialDownlinkBWP-> genericParameters-> locationAndBandwidth = 25 PRBs

Editor’s note: Signalling aspects to be updated once LS from RAN1/2 is received.  The text above is current RAN4 understanding.

### 6.2.2	Combined UE CBW (one cell)

#### 6.2.2.1      General Aspects

- 	Studied spectrum blocks covered by “main RF carrier” and “additional RF carrier”

-	The “main RF carrier” is Rel-15 compatible and contains the SSB as well as all necessary broadcast information, legacy UEs and UEs which do not support this solution are able to camp on it and be connected without being aware of the “additional RF carrier”

-	The “additional RF carrier” aligned to the “main RF carrier” PRB grid, UEs which support this solution would be reconfigured (once UE capabilities are known) in RRC_CONNECTED to use wider BWP

- 	The “main RF carrier” and the “additional RF carrier” treated as single cell (one carrier from baseband perspective) to allow for a single BWP to cover studied spectrum block in RRC_CONNECTED

- 	Both the “main RF carrier” and the “additional RF carrier” would clearly define the size and position of the guard band which allows for an unambiguous placement of the overlapping channel filters and thus prevents problems with OBUE, ACS or in-band blocking

- 	From UE perspective, supported in downlink only

### 6.2.3	Overlapping CA (two cells)

#### 6.2.3.1      General Aspects

Using the Overlapping CA approach to support irregular spectrum is one of the methods without the need of introduction of new dedicated channel bandwidths for both UE and BS. Figure 6.2.3-1 shows overlapping CA from network perspective, in which BS supports the intra-band overlapping CA, while UEs only supports the single CC with the existing channel bandwidth. And figure 6.2.3-2 shows overlapping CA from UE perspective, in which both BS and UE support the intra-band overlapping CA to use all RBs.

Even two CCs are overlapped in frequency domain, the network can configure different CCs with non-overlapping BWP to avoid the conflict of physical signals or channels.

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Overlapping CA for gNB

Figure 6.2.3-1: Overlapping CA from network perspective

Overlapping CA for gNB

UE3: overlapping CA of CC1 and CC2

CC1: BWP#1

CC2: BWP#2

Figure 6.2.3-2: Overlapping CA from UE perspective

In summary,

- No new gNB channel bandwidth is required

- Legacy UE using existing lower regular NR channel bandwidth can operate in either carrier

- Overlapping CA approach needs to have PRB grid alignment between overlapping CCs

- UE perspective, overlapping CA is optional supported in DL only

## 6.3	Complexity and efficiency study

#### 6.3.1	Combined UE CBW (one cell)

-	Does not require new channel filters for UE and gNB to be designed and tested

-	Requires support of two RF carriers phase aligned on the Tx side to ensure phase continuity on the Rx side

-	Requires UE support of intra-band non-contiguous CA, increased complexity due to combining two RF carriers into one baseband carrier

-	For scenarios with less than 10 MHz, second SSB is not excluded but not recommended due to significant additional overhead (duplicated SSB transmission as well as other radio resources such as PDCCH, CSI-RS, PDSCH (for SIB), CSI for Tracking, etc.). However, second SSB could enable or improve the legacy UEs' use of the further PRBs provided by the additional RF carrier in spectrum scenarios with less than 10 MHz.

-	“Additional RF carrier” not to be on the channel raster to increase spectrum utilization (up to 2 PRBs), it should be noted “additional RF carrier” is used only by UEs which support this solution

-	Proposed BWPs size of the irregular spectrum chunk may have an impact on performance requirements and additional testing

-	High spectrum utilization due to lower internal guard band as well as no additional CA overhead (duplicated common channels and signals such as SSB, PDCCH and CSI-RS configured both in Pcell and Scell, in addition of the MAC processes associated with CA) due to single baseband carrier usage:

| Spectrum block [MHz] | Number of PRBs (15kHz SCS without 100kHz raster alignment) | Spectrum utilization (without 100kHz raster alignment) [%] | Number of PRBs (15kHz SCS with 100kHz raster alignment) | Spectrum utilization (with 100kHz raster alignment) [%] |
| --- | --- | --- | --- | --- |
| 6 | 30 | 90 | 30 | 90 |
| 7 | 36 | 92.6 | 35 | 90 |
| 11 | 58 | 94.9 | 57 | 93.3 |
| 12 | 63 | 94.5 | 62 | 93 |
| 13 | 69 | 95.5 | 67 | 92.8 |

#### 6.3.2	Overlapping CA (two cells)

From network perspective:

For gNB, no new filter for RF channel is needed. The CA implementation can be reused. The only update is to allow the configuration that the two carriers can be partially overlapped by adjusting the channel spacing. The network can prevent collisions between the two component carriers.

For UE, there is no impact and fully backwards compatible.

From UE perspective:

For gNB, there is no further requirements compared to the above description for overlapping CA from network perspective.

For UE, it is optional support in DL only. For the UE supports DL intra-band non-contiguous CA with corresponding channel bandwidth(s), overlapping CA can be considered by support the configuration with partially overlapping carriers.  And in the case no new channel filter for UE is needed.

For overlapping CA solution, the legacy channel bandwidth should be supported, hence the minimum guardband should not be less than the minimum guardband of lower UE channel bandwidth than operator licensed bandwidth. To support legacy UEs, channel raster should be applied for each UE channels. And in order to simplify the resource schedule and make the use of single SSB for CA operation, RB alignment is required. Table 6.3.x-1 show the channel spacing and spectrum utilization for the example channel bandwidths.

Table 6.3.2-1: channel spacing and spectrum utilization

| Channel Bandwidth(MHz) | SCS(kHz) | CA combos | Nominal channel spacing (MHz) | Transmission bandwidth configuration NRB | Spectrum utilization (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 15 | 5+5 | 0.9 | 30 | 90 |
| 7 | 15 | 5+5 | 1.8 | 35 | 90 |
| 11 | 15 | 10+10 | 0.9 | 57 | 93.3 |
| 12 | 15 | 10+10 | 1.8 | 62 | 93 |
| 13 | 15 | 10+10 | 2.7 | 67 | 92.8 |

## 6.4	Generic solutions guidance

NOTE: The 6th objective is not an analysis/study but a guidance on solutions. A comparison of the proposed solutions with respect to the criteria in the 6th objective should be included in this clause.

#### 6.4.1	Overlapping UE CBW

This approach can be also be applied to all irregular channel bandwidths with only consideration of different spectral utilization considerations. However the single CORESET#0 and SSB is only applicable for bandwidths greater than 10 MHz.  All irregular channel bandwidths two SSBs can be used (irregular channel bandwidths  < 10 MHz SSB can be TDM).  .

#### 6.4.2	Combined UE CBW (one cell)

-	Generic and future proof solution, the channel bandwidth can be tailored with the resolution of 1PRB

-	Ensured co-existence with very limited specification impact since both the “main RF carrier” and the “additional RF carrier” would conform to existing 3GPP requirements

-	Does not require new channel filters for UE and gNB to be designed and tested

## 6.5	RAN1 and RAN2 impact

Placeholder for 7th objective study

## 6.6	Legacy UE impact

NOTE: The 8th objective is not an analysis/study but a guidance on solutions

#### 6.6.1	Overlapping UE CBW

This approach works with all the legacy UEs and thus an operator can use this solution with the whole ecosystem of available devices.

#### 6.6.2	Combined UE CBW (one cell)

-	No impact to legacy UEs. UEs which support this solution would be reconfigured in RRC_CONNECTED, there is no change to UE behaviour in IDLE mode which could create potential issue with legacy UE compatibility

## 6.7	RAN4 standard impact identification

#### 6.7.1	Combined UE CBW (one cell)

-	Very limited since both the “main RF carrier” and the “additional RF carrier” would conform to existing 3GPP requirements, to guarantee co-existence

#### 6.7.2	Overlapping CA (two cells)

The CA framework defines the transmitter emission and receiver blocking at the edge of carriers. Limited impacted is expected to enable coexistence with neighbouring channels for this scenario. Overlapping CA channel spacing need to be updated to consider channel raster, minimum guard band and RB alignment.

For 15 KHz SCS and 100 KHz channel raster,

Channel spacing=2*BWChannel-(BWChannel1+BWChannel2)1.80.9 (MHz)

For NR operating bands with 15 kHz channel raster,

Channel spacing=2*BWChannel-(BWChannel1+BWChannel2)0.18*2n+10.18*2n (MHz)

with

n=μ0

#### 6.7.y	Overlapping UE CBW

If the irregular channel bandwidths are explicitly defined in the specification, it will create huge technical specification work. For each channel bandwidth, the guard band size and the transmission bandwidth configuration need to be specified and used as a basis for defining transmitter and receiver requirements. The identified impact to BS core specification for a new channel BW is shown in following Table 6.7.y-1.

Table 6.7.y-1 Analysis on the impact to BS core specification for a new channel BW

| Subject | Clause in 36.104/38.104 | Requirement | Assessment for new channel BW |
| --- | --- | --- | --- |
| General | 5.3.2 | Transmission bandwidth configuration | the Transmission bandwidth configuration NRB for the new CBW need to be defined |
|  | 5.3.3 | Minimum guardband and transmission bandwidth configuration | The minimum guardband for the new CBW need to be defined |
|  | 5.3.5 | BS channel bandwidth per operating band | new CBW need defined per band |
| Transmitter | 6.3.3 | Total power dynamic range | it is a NRB related requirement |
|  | 6.6.2 | Occupied bandwidth | BS channel bandwidth should be defined |
|  | 6.6.3 | ACLR | The filter are set using transmission bandwidth configuration (BWConfig). It need to be defined for testing |
|  | 6.7 | Transmitter intermodulation | the interfering is defined according to BS channel bandwidth |
| Receiver | 7.2 | Reference sensitivity level | for 15 KHz SCS, 25 RB and 106 RB FRC which can be reused |
|  | 7.3 | Dynamic range | interfering signal level is according to BS channel bandwidth |
|  | 7.4 | In-band selectivity and blocking | the position of interfering signal is defined according to BS channel bandwidth/transmission bandwidth configuration |
|  | 7.7 | Receiver intermodulation | the position of interfering signal is defined according to BS channel bandwidth/transmission bandwidth configuration |
|  | 7.8 | In-channel selectivity | it is defined according to BS channel bandwidth |

It is concluded that new dedicated BS channel bandwidths for irregular channel bandwidths are not defined explicitly in the specification.

# 7	Conclusion

TBD

# Page setup parameters

This clause defines the margin parameters and the header to be used (implemented in the macros).

Title page (= title section)

A4 portrait, Top: 4 cm, Bottom: 19 cm, Left: 1,5 cm, Right: 1,5 cm, Gutter: 0 cm, Header: 0 cm, Footer: 0 cm.

Portrait sections

A4 portrait, Top: 2.5 cm, Bottom: 2 cm, Left: 2 cm, Right: 2 cm, Gutter: 0 cm, Header: 1,5 cm, Footer: 0,6 cm.

Landscape sections

A4 landscape, Top: 2 cm, Bottom: 2 cm, Left: 2 cm, Right: 2,5 cm, Gutter: 0 cm, Header: 1,5 cm, Footer: 0,6 cm.

Headers and footers

Header

The following contains the master location for all headers (except for the title section). These paragraphs contain framed fields which will result in one header line and are bookmarked "header".

The left-most entry contains a possible additional document reference, e.g. " Release 17", identified on the title page by the use of the ZGSM character style.

Release | 17 | 16 | 15

The centre entry is the page number.

11

The right-most entry repeats the title page information, identified by the use of the ZA paragraph style.

3GPP TS ab.cde Vx.y.z (yyyy-mm)

NOTE:	For documents which are split into more than one file, the possible additional document reference and the title page information need to be hardcoded in all files except the one containing the title section.

NOTE: It has been found that opening very long documents with MS Word 2016 onwards (including versions of Word packaged in MS Office 365) can take a very long time, as can navigating around the document. This applies both in draft view and in print layout view. To solve this problem, the page header for each section of the document may be hard-coded, replicating the text which would otherwise have been automated via the use of ZGSM and ZA styles.

Footer

The footer contains always "3GPP" (except for the title page).

3GPP

Annex <A> (normative):
<Normative annex for a Technical Specification>

Annex <B> (informative):
<Informative annex for a Technical Specification>

Annex <X> (informative):
Change history

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-11 | RAN4#97 | R4-2015722 |  |  |  | TR Skeleton | 0.0.1 |
| 2020-11 | RAN4#97 | R4-2016930 |  |  |  | TR Skeleton (revised) | 0.0.1 |
| 2021-01 | RAN4#98 | R4-2101555 |  |  |  | Updated TR 38.844 | 0.0.2 |
| 2021-01 | RAN4#98 | R4-2103262 |  |  |  | Updated TR 38.844 (revised) | 0.0.2 |
| 2021-04 | RAN4#98bis | R4-2106690 |  |  |  | TP to the TR 38.844: Terminology | 0.0.2 |
| 2021-04 | RAN4#98bis | R4-2105420 |  |  |  | TP to the TR 38.844: Terminology (revised) | 0.0.2 |
| 2021-05 | RAN4#99-e | R4-2110487 |  |  |  | Update TR 38.844 with TP | 0.0.3 |
| 2021-05 | RAN4#99-e | R4-2108021 |  |  |  | TP on the use of overlapping channel bandwidths from UE perspective | 0.0.3 |
| 2021-05 | RAN4#99-e | R4-2107885 |  |  |  | TP to TR38.844 on wider channel BW method | 0.0.3 |
| 2021-05 | RAN4#99-e | R4-2108022 |  |  |  | TP to TR38.844 on wider channel BW method (revised) | 0.0.3 |
| 2021-05 | RAN4#99-e | R4-2109436 |  |  |  | TP on using overlapping channels from the network perspective solution | 0.0.3 |
| 2021-05 | RAN4#99-e | R4-2107887 |  |  |  | TP on using overlapping channels from the network perspective solution (revised) | 0.0.3 |
| 2021-05 | RAN4#99-e | R4-2108112 |  |  |  | TP on using overlapping channels from the network perspective solution (revised) | 0.0.3 |
| 2021-08 | RAN4#100-e | R4-2113948 |  |  |  | Updated TR 38.844 | 0.0.4 |
| 2021-08 | RAN4#100-e | R4-2114239 |  |  |  | TP for TR 38.844: 6 MHz for n85 with overlapping CHBW |  |
| 2021-08 | RAN4#100-e | R4-2112365 |  |  |  | TP on using next larger channel bandwidth solution |  |
| 2021-08 | RAN4#100-e | R4-2114999 |  |  |  | TP on using next larger channel bandwidth solution (revised) |  |
| 2021-08 | RAN4#100-e | R4-2115114 |  |  |  | TP on using next larger channel bandwidth solution (revised) |  |
| 2021-08 | RAN4#100-e | R4-2113046 |  |  |  | TP on overlapping UE channel bandwidths (Overlapping CA) |  |
| 2021-08 | RAN4#100-e | R4-2115000 |  |  |  | TP on overlapping UE channel bandwidths (Overlapping CA) (revised) |  |
| 2021-08 | RAN4#100-e | R4-2113949 |  |  |  | TP to TR 38.844: Section 6.2 Overlapping UE Channel BWs |  |
| 2021-08 | RAN4#100-e | R4-2115050 |  |  |  | TP to TR 38.844: Section 6.2 Overlapping UE Channel BWs (revised) |  |