TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2118779

Electronic Meeting, 1st – 12th November 2021

Source:	Ericsson

Title:	On gNB DL Filter Analysis for widerCBW Approach

Agenda item:	9.2.2.1

Document for:	Discussion

# 1	Introduction

- For further discussion on widerCBW placement is optimized to be contained in the band or meeting interference level in adjacent channels

During RAN4#101-e discussion relating to all irregular bandwidth approaches.  In particular for wider channel bandwidth approach a WF [1] was agreed to continue study on aspects relating to the details of the approach.  Several questions, totaling 6, were documented.  In this contribution the focus on the DL/UL filters specifically for the wider channel bandwidth approach is further considered.

- Confirm DL and UL filter assumptions (for both gNB and UE) and RB placement e.g should have (or have not) same center.  WiderCBW (DL), smallerCBW (UL) and irregular bandwidths all contain same center point.

  - Consider raster point may not align, shift to (right or left) next point relative to center.

# 2	Discussion

The study of the placement of blanked RBs has been discussed as a possible solution for the wider channel bandwidth approach in order to fulfil out of band requirements.  Although this has been hypothesized the placement of the RF front end filter and BB analog filter on different edges to help with the highest interference suppression it has never been shown whether this would be sufficient in meeting transmitter spurious emissions mask on the gNB side or the ACS/blocking on the UE side.  In the following analysis the focus will be on gNB requirements.

## 2.1 Irregular bandwidth centered to next larger channel bandwidth

The first analysis is looking at equal number of blanked RBs on either side of the irregular bandwidth.  Based upon the agreed number of RBs in each irregular bandwidth shown in Table 6.1.1-1 and Table 6.1.1-2 below:

Table 6.1.1-1: Exemplary number of RBs based on the next larger channel guard bands (15kHz SCS).

| Channel (MHz) | Next larger channel (MHz) | Next larger channel guard band (kHz) | Next larger channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 10 | 312,5 | 52 | 29 30 | 87 90 |
| 7 | 10 | 312,5 | 52 | 35 36 | 90 92,57 |
| 11 | 15 | 382,5 | 79 | 56 57 | 91,6 93,27 |
| 12 | 15 | 382,5 | 79 | 62 63 | 93 94,5 |
| 13 | 15 | 382,5 | 79 | 67 | 92,8 |

Table 6.1.1-2: Exemplary number of RBs based on the next larger channel guard bands (30kHz SCS).

| Channel (MHz) | Next larger channel (MHz) | Next larger channel guard band (kHz) | Next larger channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 10 | 665 | 24 | 12 | 72 |
| 7 | 10 | 665 | 24 | 15 16 | 77,1 82,29 |
| 11 | 15 | 645 | 38 | 26 | 85,1 |
| 12 | 15 | 645 | 38 | 29 30 | 87 90 |
| 13 | 15 | 645 | 38 | 32 | 88,6 |

With the following changes marked in red to show the changes in order to adopt to symmetric guard allocation.  As these were initial exemplary numbers the additional changes would not alter any initial considerations.  PRB alignment between the next larger and irregular bandwidth is also assumed.

Figure 1: Irregular bandwidth PRB allocation centered

Filter analysis show that blanking PRBs does provide a reduction in PSD outside the irregular bandwidth (blue arrow in Figure 2) but it also indicates that the blanked RPBs does not provide enough suppression needed to meet all out of band emissions requirements.

Figure 2: Wider channel bandwidth RB blanking approach for irregular bandwidth of 6 MHz centered and aligned to NR nominal 10 MHz channel.

As the results show simply blanking RBs in the next wider channel bandwidths provides some protection near the irregular bandwidth edges however not sufficient.  The new adoption of transmit filter with the same number filter size compared to the next larger channel bandwidth can be used to provide the needed attenuation to meet all out of band RF requirements.  In order to comply with any out of band requirements a larger channel bandwidth filter may be adopted to meet the necessary RF requirements which can be demonstrated below.  Due to the reduction of number of allocated RBs is reduced compared to the larger channel bandwidth it can facilitate the needed change without difficulty.

Figure 3: Wider channel bandwidth RB blanking approach with filter for irregular bandwidth of 6 MHz centered and aligned to NR nominal 10 MHz channel.

## 2.2 Irregular bandwidth aligned left/right to next larger channel bandwidth

The initial thinking behind a left or right shifted RB allocation was to be able to utilize the filter suppression on one edge which would then be able to meet the out of band requirements.  As point (3) stated in the WF [1] the placement of the irregular bandwidth RBs and filter are optimized to minimize the interference level; however, it’s the current emissions requirements would need to be realized in order to comply with regulatory testing.  In this following section it examines the different placements of the filter and effects to mitigate interference in adjacent channels.  PRB alignment between the next larger and irregular bandwidth is also assumed.

Figure 4: Irregular bandwidth PRB allocation left aligned (left) and right aligned (right)

Similar to Section 2.1 the results show that there is a reduction in PSD when PRBs are blanked in the allocation outside of the irregular bandwidth within the next larger channel bandwidth.  However, it is clear that although there is a reduction in PSD the suppression needed outside of the irregular bandwidth is not enough to meet emissions requirements.

In the results shown below the red line is the spectrum emissions mask overlayed of the 6 MHz irregular bandwidth utilizing a 10 MHz channel filter.  The reduction of PSD is between 25- 30 dB from the peak PSD.  In some deployment scenarios this approach can still be used without new channel filters.  In the example Figure 5, if Operator B licensed spectrum allocation contains the blanked PRBs the approach to shift the wider channel bandwidth filter may prove as an adequate implementation solution.

Operator B

Operator A

Figure 5: Right aligned wider channel bandwidth filter

Some coordination in Operator B’s network such as a fall-back mode can be adopted when needed, where the fall-back to the smaller nominal channel bandwidth in this example that would be 5 MHz.

# 3	Conclusion

In this contribution the following key observations were highlighted:

- For the blanked RB approach irregular bandwidth gNB transmit filters are required in order to meet emissions requirements

- In the scenarios of only single adjacent neighbouring operator the right or left filter shift may be applied along with possibly a fall back mode.

# 4	References

- R4-2114998, “WF on widerCBW Approach”, Ericsson

- R4-2107253, “Support of irregular channel bandwidth with larger or overlapped BW”, Skyworks Solutions Inc.