3GPP TSG-RAN WG4 Meeting #101-e	R4-2118817

Electronic Meeting, November 01-12, 2021

Title: 	Simulation assumption on hypothetical PDCCH performance for RLM and BFD for RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	8.20.3.1.4

Document for:	Approval

Introduction

This simulation assumption on hypothetical PDCCH parameters for RedCap UE is to evaluate:

- The PDCCH performance impact due to reducing the BW for CSI-RS based RLM and CSI-RS based BFD to 24PRBs @60kHz;

- The PDCCH performance impact in RLM and BFD due to reducing 2RX to 1RX.

Simulation Assumption

The proposed simulation assumptions for PDCCH for RLM and BFD are listed in Table 1 and Table 2.

Table 1: Simulation assumptions for PDCCH performance for RLM

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | FR1, 4 GHz | FR2, 30 GHz |
| DCI format / payload size | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs) | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs) |
| Number of control OFDM symbols | SSB based RLM {2 symbols, 15 kHz, 24 PRB} {2 symbols, 30 kHz, 24 PRB} CSI-RS based RLM {2 symbols, 15 kHz, 48 PRB} {2 symbols, 30 kHz, 48 PRB} {2 symbols, 60 kHz, 48 PRB} {2 symbols, 60 kHz, 24 PRB} | SSB based RLM {2 symbols, 60 kHz, 24 PRB} {2 symbols, 120 kHz, 24 PRB} CSI-RS based RLM {2 symbols, 60 kHz, 24 PRB} {2 symbols, 60 kHz, 48 PRB} {2 symbols, 120 kHz, 48 PRB} |
| Bandwidth (MHz) |  |  |
| Sub-carrier spacing (kHz) |  |  |
| Aggregation level (CCE) | OOS:8 and 16 IS: 4 and 8 | OOS:8 and 16 IS: 4 and 8 |
| Ratio of hypothetical PDCCH RE energy to average RLM-RS RE energy | OOS:4 IS: 0 dB and 3dB | OOS:4 IS: 0 dB and 3dB |
| Ratio of hypothetical PDCCH DMRS energy to average RLM-RS RE energy | OOS:4 IS: 0 dB and 3dB | OOS:4 IS: 0 dB and 3dB |
| Precoder granularity in frequency domain | REG bundle size | REG bundle size |
| REG bundle size | 6 | 6 |
| CP length | Normal | Normal |
| Mapping from REG to CCE | Distributed | Distributed |
| SNR | [-14, -13, ., 1, 0] | [-14, -13, ., 1, 0] |
| Propagation condition | AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns | AWGN TDL-A 30ns |
| Antenna configuration | 2x2 2x1 | 2x2 2x1 |
| UE speed | 3km/h, 42km/h | 3km/h, 30km/h |
| Note: Companies are encouraged to clarify whether the 2Tx simulation results are based Tx diversity or not. |  |  |

The outcome metric is the SNR levels corresponding to 10% and 2% PDCCH BLER.

Table 2: Simulation assumptions for PDCCH performance for BFD

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | FR1, 4 GHz | FR2, 30 GHz |
| DCI format / payload size | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs) | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs) |
| Number of control OFDM symbols | SSB based BFD {2 symbols, 15 kHz, 24 PRB} {2 symbols, 30 kHz, 24 PRB} CSI-RS based BFD {2 symbols, 15 kHz, 48 PRB} {2 symbols, 30 kHz, 48 PRB} {2 symbols, 60 kHz, 48 PRB} {2 symbols, 60 kHz, 24 PRB} | SSB based BFD {2 symbols, 60 kHz, 24 PRB} {2 symbols, 120 kHz, 24 PRB} CSI-RS based BFD {2 symbols, 60 kHz, 24 PRB} {2 symbols, 60 kHz, 48 PRB} {2 symbols, 120 kHz, 48 PRB} |
| Bandwidth (MHz) |  |  |
| Sub-carrier spacing (kHz) |  |  |
| Aggregation level (CCE) | 8 and 16 | 8 and 16 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB and 3dB | 0 dB and 3dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB and 3dB | 0 dB and 3dB |
| Precoder granularity in frequency domain | REG bundle size | REG bundle size |
| REG bundle size | 6 | 6 |
| CP length | Normal | Normal |
| Mapping from REG to CCE | Distributed | Distributed |
| SNR | [-14, -13, ., 1, 0] | [-14, -13, ., 1, 0] |
| Propagation condition | AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns | AWGN TDL-A 30ns |
| Antenna configuration | 2x2 2x1 | 2x2 2x1 |
| UE speed | 3km/h, 42km/h | 3km/h, 30km/h |
| Note: Companies are encouraged to clarify whether the 2Tx simulation results are based Tx diversity or not. |  |  |

The outcome metric is the SNR levels corresponding to 10% PDCCH BLER.

Conclusion

In this paper, we provided simulation assumptions for PDCCH performance for RLM and BFD. We suggest RAN4 to use the assumptions in Table 1 and Table 2 for the simulation evaluation.