3GPP TSG-RAN WG4 Meeting #101-e	R4-2118841

Electronic Meeting, November 1 – 12, 2021

Title: 	Discussion on RRM impacts for R17 NR sidelink relay

Source: 	Huawei, HiSilicon

Agenda item:	8.24.2

Document for:	Discussion

Introduction

In RAN4#100-e meeting, the RRM impacts for NR sidelink relay have been initially discussed and at least the following requirements need to be specified for NR sidelink relay in Rel-17.

- Selection/reselection of NR sidelink relay UE

- Intra-frequency measurement accuracy requirements

Whether to specify the following requirements for NR sidelink relay is still FFS.

- Interruptions to serving cells at discovery configuration

- Interruptions to serving cells during discovery

- Cell reselection for discovery on non-serving frequency

In this contribution, we will provide the further discussion on the RRM impacts on the above aspects.

Discussion

Since the mechanisms for LTE relay discovery and (re)selection is used as baseline, RRM requirements for ProSe relay in LTE can be used as start point for studying the RRM requirements for NR sidelink relay.

- Selection/reselection of NR sidelink relay UE

For LTE ProSe relay selection/reselection procedure, the remote UE is required to perform SD-RSRP measurements on the selected relay UE and the detected candidate relay UEs. The measurement period and the evaluation period of SD-RSRP measurements used for ProSe relay selection or reselection criterion are defined and depend upon the configured discovery period. For LTE ProSe relay, the UE performs SD-RSRP measurements based on PSDCH DMRS. However, there is no PSDCH and PSDCH-DMRS for NR sidelink relay in Rel-17. For NR sidelink relay, the sidelink discovery messages could be possibly carried by PSSCH. Then, the UE could consider to use L1 SL-RSRP measured on PSSCH-DMRS for relay reselection trigger evaluation. Besides, there would be no discovery period configured for PDSCH. Hence, the existing measurement period and evaluation period of SD-RSRP measurements used for LTE ProSe relay selection or reselection criterion cannot be reused for NR sidelink relay selection/reselection.

Proposal 1: If sidelink discovery messages are carried on PSSCH, RAN4 could consider to define the measurement period and evaluation period of L1 SL-RSRP measurements without discovery period for NR sidelink relay selection/reselection.

- Cell reselection for discovery on non-serving frequency

In LTE, the ProSe direct discovery transmissions can be configured on a non-serving carrier and a non-serving cell on the non-serving carrier can be selected as reference cell used for ProSe Direct Discovery synchronization. The UE is required to perform cell reselection procedure on the non-serving carrier used for ProSe direct discovery transmissions. UE needs to perform cell detection and RSRP/RSRQ measurements on the non-serving carrier, and the intra-frequency measurements requirements are introduced.

In RAN2, the following agreement has been achieved:

Agreements: Proposal 1b: RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency - If there is Uu coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency. - If there is no Uu coverage at the concerned frequency, UE shall rely on pre-configuration. … Proposal 3a [easy]: RAN2 agree that RRC_CONNECTED L3 relay/remote UE or layer 2 remote UE which are in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency, - If there is Uu coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency. - If there is no Uu coverage at the concerned frequency, UE shall rely on pre-configuration.

It can be observed that the concerned SL frequency could be a non-serving frequency. If there is Uu coverage on concerned SL frequency, UE needs to perform cell selection/reselection on the non-serving frequency (i.e. concerned SL frequency) for NR sidelink discovery. Therefore, RAN4 needs to investigate the cell selection/reselection requirements on the non-serving frequency.

Proposal 2: It is suggested to specify cell reselection requirements for NR sidelink discovery on non-serving carrier.

For LTE ProSe relay, the cell reselection requirements on non-serving frequency are defined by using LTE cell reselection requirements on serving frequency in idle mode as baseline, where DRX cycle is replaced by discovery period. Similarly, the existing NR cell reselection on serving frequency in idle mode can be used as baseline for deriving the cell reselection requirements on non-serving frequency for NR sidelink relay.

Proposal 3: The existing NR cell reselection requirements on serving frequency in RRC idle mode can be used as baseline for investigating the cell reselection requirements for NR sidelink discovery on non-serving carrier.

- Interruptions for NR sidelink discovery

In NR, the UE capable of V2X sidelink communication is allowed an interruption on the PCell/serving cell during the RRC reconfiguration procedure that indicates its interest (initiation or termination) in V2X sidelink communication to the connected gNodeB. Similarly, the UE capable of NR sidelink discovery can be allowed an interruption on the PCell/serving cell during the RRC reconfiguration procedure that indicates its interest (initiation or termination) in V2X sidelink discovery to the connected gNodeB and the existing R16 interruption requirements on WAN due to V2X sidelink communication can be reused.

Proposal 4: It is suggested to specify the interruption requirements on WAN due to NR sidelink discovery and the existing R16 interruption requirements on WAN due to V2X sidelink communication can be used as baseline.

Conclusions

This contribution provides the discussion on the RRM impacts of NR sidelink relay in Rel-17. The following are provided:

Proposal 1: If sidelink discovery messages are carried on PSSCH, RAN4 could consider to define the measurement period and evaluation period of L1 SL-RSRP measurements without discovery period for NR sidelink relay selection/reselection.

Proposal 2: It is suggested to specify cell reselection requirements for NR sidelink discovery on non-serving carrier.

Proposal 3: The existing NR cell reselection requirements on serving frequency in RRC idle mode can be used as baseline for investigating the cell reselection requirements for NR sidelink discovery on non-serving carrier.

Proposal 4: It is suggested to specify the interruption requirements on WAN due to NR sidelink discovery and the existing R16 interruption requirements on WAN due to V2X sidelink communication can be used as baseline.

Reference

- R4-2115373, “WF on R17 NR SL Relay RRM”, OPPO