3GPP TSG-RAN WG4 Meeting #101-e	R4-2118873

Electronic Meeting, 1-12 Nov, 2021

Title:	R17 FR1 SCC drop

Source:	OPPO

Agenda item:	8.3.2.6.1

Document for:	Approval

- Introduction

The issue of low priority serving cell power drop in CA has been brought up in RAN4 with WF [1] as below to capture the status and inputs. Then in the September RAN plenary this issue was added to the R17 FR1 enhancement WID. This paper continue discuss on this topic.

- The observed field issue applies for both intra-band and inter band UL CA. - FFS for FR2 inter band UL CA - To solve the issue, it is agreed to be handled from Rel-17: - RAN1 and RAN2 shall be involved in the discussion and their spec impact should be considered - LS should be sent to RAN1 to check their understanding of this low priority CC dropping issue and the possible spec impact - The specific solution is FFS - The study on Scell dropping solution should cover both FR1 and FR2 CA.

- Discussion

In RAN4, several solutions or questions have been brought up in last meeting, for example the NW scheduling based solution or NW configuration based solution, etc. And the solution of RRC configuration based hard power limit on PCC and MAC CE based enabling/disabling power limit method was proposed in [2]. The main idea is to limit the PCC (where it considers the PCC is the high priority CC) and leave power for the other CC. Generally it is understandable of this method, however, several enhancements might still be needed.

1. Power limit configuration issue

According to 38.213, the high priority CC is not fixed in CA where the priority is a comparison of channels (PRACH, PUCCH, PUSCH, SRS), and the priority can also be indicated by NW with phy-PriorityIndex signaling. So there is no fixed priority in CA, and put max power limit always on the PCC is not always proper. The priority conditions need to be further considered at least, for example activate the power limit only on the higher priority CC by comparison of channels and NW priority indication.

Observation 1:    There is no fixed CC priority in CA, and the priority is determined by channels (PRACH, PUCCH, PUSCH, SRS), and NW indication with phy-PriorityIndex signaling.

Proposal 1:         It is proposed to put max power limit on the high priority CC instead of always on the PCC.

2. Power limit enabling issue

In the reply of why NW scheduling cannot avoid the SCC dropping in the field, following answers are got in moderator summary [3]:

- gNB cannot get UE MPR in advance, it means gNB does not know Pcmax,CA is lower than Pcmax,c1+Pcmax,c2 in advance - In the field the prioritization is highly dynamic for the determination of the required power transmissions for each serving cell (independent power control) - Only the UE is aware of its actual configured power setting and balance between its transmissions on the UL carriers. This is why the solution should be implemented in the UE but configured and controlled by the gNB.

As has been commented by companies, putting hard limit to one CC will cause the coverage issue, this is not desired thus the power limit should only be enabled when it is necessary. However, from above table at least it can be seen that the Pcmax,CA is unknown to the NW since different MPRs are applied in CA/non-CA, as is shown in figure 1, that makes NW never know how much total power (PHR total) is left for a CA combination since current PHR is reported per CC rather than per band combination. Then the problem might be how NW to decide when to enable/disable the max power limit?

Figure 1 PHR and Pcmax,CA ambiguity in CA

Observation 2:    Pcmax,CA and PHR for CA is unknown to the NW which makes NW doesn’t know when to enable/disable the max power limit.

To make NW clear of the total UE power status in CA, further reporting the PHR total and Pcmax,CA for a band combination via reusing current PHR reporting mechanism will be helpful as has been done in the MPE PMPR reporting.

One example is as below figure (red part), extending current PHR reporting and adding the PHR and Pcmax for total band combination.

Figure 2 Example of PHR total and Pcmax,CA reporting

Observation 3:    Minor changes are needed in RAN2 to report Pcmax,CA and PHR for CA by reusing current PHR reporting mechanism.

Proposal 2:         It is proposed to consider reporting Pcmax,CA and total PHR for band combination via reusing current PHR reporting mechanism.

- Conclusion

This paper discussed the power limit configuration based solution for SCC drop and the issues related, mainly on the power limit configured on lower priority CC rather than always on PCC, and the necessary assist information needed to facilitate NW enable/disable the power limit.

1. Power limit configuration issue

Observation 1:    There is no fixed CC priority in CA, and the priority is determined by channels (PRACH, PUCCH, PUSCH, SRS), and NW indication with phy-PriorityIndex signaling.

Proposal 1:         It is proposed to put max power limit on the high priority CC instead of always on the PCC.

2. Power limit enabling issue

Observation 2:    Pcmax,CA and PHR for CA is unknown to the NW which makes NW doesn’t know when to enable/disable the max power limit.

Observation 3:    Minor changes are needed in RAN2 to report Pcmax,CA and PHR for CA by reusing current PHR reporting mechanism.

Proposal 2:         It is proposed to consider reporting Pcmax,CA and total PHR for band combination via reusing current PHR reporting mechanism.

- References

[1] R4-2114949, WF on solution for Scell dropping issue, HW

[2] R4-2112826, Resolution of the Scell dropping (power reduction) problem, Ericsson

[3] R4-2115027, [100-e][127] NR_RF_FR1_enh_Part_1_HPUE_ULMIMO