3GPP TSG-RAN WG4 Meeting #101-e	R4-2118885

Electronic Meeting, 1-12 Nov, 2021

Title:	R17 FR2 CBM inter-band DL CA

Source:	OPPO

Agenda item:	8.4.2.1.1

Document for:	Approval

- Introduction

FR2 Inter-band DL CA within same frequency group based on CBM is one of the target to be specified for Rel-17 FR2. And in last meeting the WF [1] was agreed. This paper continues discussing these issues.

WF – UE capabilities - GTW Agreement: Add a new enumerated value to beam management type in [Rel-17] so that a UE can support both IBM and CBM, i.e., ENUMERATED {ibm, cbm, both}. - FFS on the applicability of requirements for UE supporting both capabilities. - FFS whether to introduce it from early release. - GTW Agreement: The capability to indicate whether the UE support CBM under multi-chain architecture is not introduced. - Fs_inter_CBM: FFS WF - Beam management reference signal - Beam management reference signal (BMRS): The DL signal designated by the network for the UE to make measurements on, for the purpose of selecting its DL Rx beam(s). - GTW Agreement: For core requirements applicability in relation to BMRS location: - CBM inter-band CA requirements apply per-band with the BMRS configured in any one of the participating bands. - Introduce side condition for core requirement that BMRS can only be placed on PCC for the DL CA case with a single uplink. - FFS whether to set side condition only for the worst case - GTW Agreement: For test cases, further discussion on setup for testing to reduce the test burden and send LS to RAN5. WF – Sensitivity Requirements - GTW Agreement: RAN4 agree to introduce REFSENS and EIS spherical coverage requirements based on IBM inter-band CA framework. - FFS on the values for the requirements - FFS whether there is PSD difference and what is the difference - FFS the impact of frequency separation

- Discussion

2.1 Beam management capability

The capability of UE supporting both CBM and IBM is agreed to be added to the signaling. And this is for UE with independent chain for each CCs. And in theory, UE supporting IBM can also support CBM since the hardware is supported. It can be supported from the early releases.

Proposal 1:               It is proposed to support UE with both CBM and IBM capability from Rel-16.

Regarding the applicability of requirements for UE supporting both CBM and IBM capabilities, most straight forward approach is that UE shall meet both requirements. From testing perspective, the costs would be high, especially for CBM tests which might have BMRS (SSB/CSI-RS) in different locations (tested band or the other band). And it needs to further analyze whether UE has to be double tested.

Observation 1:          UE with both CBM and IBM capabilities needs to meet both requirements, but testing complexity and costs need to be well considered.

Proposal 2:               It is proposed to agree that UE with both CBM and IBM capabilities needs to meet both requirements, and further discuss how to reduce the test complexity.

2.2 CBM testing

It has been agreed that CBM inter-band DL CA requirements apply per-band with the BMRS configured in any one of the participating bands. And for the side condition, it was also common understanding that test reduction is needed to keep simplicity.

CBM requirements should focus on beamforming matrix mapping from one band to the other band. The only difference of configure “BMRS” in CC1 comparing to in CC2, or with different “BMRS” is the RSRP measurement to do the beam selection. This is quite similar to beam correspondence where the RSRP is also measured and then select the beam.

- BMRS type

In principle BMRS type could be SSB only, CSI-RS only, SSB+ CSI RS as has been discussed in beam correspondence. However, defining core requirements with three different “BMRS” side condition is redundant. And it is the common understanding that only one of them is needed, and majority seems the SSB only condition from last meeting feedbacks.

Proposal 3:               It is proposed to only define one BMRS type for CBM testing, that is SSB only.

- BMRS location

It is not justified the difference of swapping the “BMRS” configurations in band under test or the other band, and not clear of the expected outcome.

It is also common view from many companies that the BMRS is putting on the band not under test is more difficult situation for a UE comparing to BMRS on the band under test. Therefore, to define the worst case, BMRS only configured on the band not under test is a good approach.

Table 1 CBM test configuration

| CBM requirements applicability for a UE supporting Band A + Band B CA based on CBM |  | BMRS (SSB only) Location |  |
| --- | --- | --- | --- |
|  |  | In band ‘A’ | in band ‘B’ |
| Tested Band | ‘A’ |  | Yes |
|  | ‘B’ | Yes |  |
| Note: BMRS is provided in the CC with a configured UL BWP |  |  |  |

Proposal 4:               It is proposed to only define one BMRS location for each band under test, i.e. BMRS is located in the band not under test.

2.2 REFSENS requirements

For the IBM based inter-band DL CA, REFSENS is defined for one band with the other band setting its DL power at a relatively high power level (at spherical coverage), so the other band is working as an interferer to the band under testing. And a relative larger relaxation comapring was defined.

With this approach, the interference effect will be bigger than the inter band IBM since they have much larger freq separation than the same freq group bands.

Observation 2:         CBM within same freq group may face larger interference from the other CC than above CA_n260-n261 case.

In Rel-16, when the reference sensitivity relaxation for inter-band CA was defined, it takes multi-band relaxation, common coverage loss, extra hardware loss, and also PSD difference degradation aspect into account. The additional loss caused for inter band CA within same freq group depends on the exact band combinations.

Currently in the WID it only mentions about same freq group or different freq groups instead of exact band combinations.

From the freq range in below figure, it can be seen that for the case of CBM between different freq groups, the freq range of 28GHz + 37GHz is similar as Rel-16 n260+n261 IBM case. However, from UE performance perspective, the interference for CBM might be severer than IBM since in CBM especially shared chain architecture no isolation can be done for the two CCs. In this case, additional 0.5dB relaxation is proposed in addition to Rel-16 IBM relaxation.

For the 28GHz+28GHz case, the freq separation is much smaller than the 28GHz+37GHz case. And PSD difference caused interference would be much larger. From this point of view, around 1dB additional relaxation is proposed in addition to Rel-16 IBM relaxation.

Proposal 5:               It is proposed to define additional 0.5dB relaxation for 28GHz+37GHz CBM comparing to Rel-16 IBM relaxation, i.e. in total 4dB relaxation.

Proposal 6:               It is proposed to define additional 1dB relaxation for 28GHz+28GHz CBM comparing to Rel-16 IBM relaxation, i.e. in total 4.5dB relaxation.

Regarding the EIS spherical coverage, CBM UE is not able to adjust the 2nd beam freely this makes the spherical coverage be a challenge. And large relaxation is expected.

Proposal 7:               For common spherical coverage, same relaxation as peak EIS can be defined for CBM inter-band case.

2.3 Fs_inter

The shared RF chain and antenna panel architecture UE might have the limitation of supporting inter-band combination due to the max receive BW. Therefore, this capability shall be known to the NW. And the Fs concept in intra-band non-contiguous can be reused here. Whether to reuse the exact table as below or introduce new tables can be further discussed.

Proposal 8:               Introduce frequency separation class for inter-band combination within same freq group CBM UE similar as the Fs in intra-band non-contiguous CA.

Conclusion

2.1 Beam management capability

Proposal 1:               It is proposed to support UE with both CBM and IBM capability from Rel-16.

Observation 1:          UE with both CBM and IBM capabilities needs to meet both requirements, but testing complexity and costs need to be well considered.

Proposal 2:               It is proposed to agree that UE with both CBM and IBM capabilities needs to meet both requirements, and further discuss how to reduce the test complexity.

2.2 CBM testing

Proposal 3:               It is proposed to only define one BMRS type for CBM testing, that is SSB only.

Proposal 4:               It is proposed to only define one BMRS location for each band under test, i.e. BMRS is located in the band not under test.

2.2 REFSENS requirements

Observation 2:         CBM within same freq group may face larger interference from the other CC than above CA_n260-n261 case.

Proposal 5:               It is proposed to define additional 0.5dB relaxation for 28GHz+37GHz CBM comparing to Rel-16 IBM relaxation, i.e. in total 4dB relaxation.

Proposal 6:               It is proposed to define additional 1dB relaxation for 28GHz+28GHz CBM comparing to Rel-16 IBM relaxation, i.e. in total 4.5dB relaxation.

Proposal 7:               For common spherical coverage, same relaxation as peak EIS can be defined for CBM inter-band case.

2.3 Fs_inter

Proposal 8:               Introduce frequency separation class for inter-band combination within same freq group CBM UE similar as the Fs in intra-band non-contiguous CA.

- Reference

- R4-2114960, WF on FR2 DL CA based on CBM, QC