3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118916

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	Requirements under RRC Connected mode for RedCap UEs

Agenda item:	8.20.3.1.2

Document for:	Discussion

# 1	Introduction

In RAN4 100-e, there were discussions on requirements under RRC Connected mode for RedCap UEs. Some options were captured in the WF [1] which are copy pasted below.

Impact on interruption requirements in CONNECTED state - Proposal 1 (ZTE, Ericsson, Nokia): RRM requirements related to interruptions shall be modified for redCap UEs. - Proposal 2 (Vivo, Oppo, Vivo): The interruption requirements defined at 8.2 are not applicable (no impact) for Redcap. - Proposal 2a (Apple, CMCC): RedCap features would not cause impact to the existing interruption requirements defined at 8.2. Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’ - Proposal 1 (Ericsson, Apple, Vivo, Nokia): RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. - Proposal 2 (Huawei, CMCC): Depends on network deployment. - Proposal 3 (Xiaomi, Oppo, MTK): FFS

- This paper discusses the issues listed above and provide our views.

2	Discussions

Gapless measurement capabilities for RedCap UE: ‘inter-frequency without gap’

We support the first proposal, which is that RedCap UEs don’t support inter-f measurements without gap. We should note that all RedCap UEs are now assumed not to support CA or DC operations, which reduces the value of such measurements. We also don’t fully understand the logic behind option 2, which says that such capability is to depend on network deployment. The RedCap UEs are to be manufactured and used in different regions of the world, so how can the manufacturers pre-determine on network configuration? The spirit behind RedCap UEs is to have a category of UEs having less capability as well as reduced costs, reduced power consumption and lighter weight as well as longer duration. Thus, we propose not to support this capability for RedCap UEs following this spirit.

RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

Impact on interruption requirements in CONNECTED state

The other aspect of RedCap UE in CONNECTED mode is the interruption caused by RF retuning. The current interruption requirements defined in TS 38.133 [2] cannot be directly re-used for RedCap UEs due to the difference in minimum UE capability. This, it’s proposed to also re-visit interruption requirements for RedCap UEs.

RRM requirements related to interruptions shall be modified for redCap UEs.

3	Conclusion

- Proposal 1: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

Proposal 2: RRM requirements related to interruptions shall be modified for redCap UEs.

References

- R4-2115358

- TS 38.133