3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118918

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	On Signalling characteristics of RedCap UEs

Agenda item:	8.20.3.1.4

Document for:	Discussion

# 1	Introduction

In RAN4 100-e, there were discussions on RLM / BFD capabilities for reduced capability UEs. Some options were captured in the WF [1] which are copy pasted below.

Impact on BW used in RLM Impact on bandwidth of CSI-RS for RLM with 60 kHz SCS in FR1 is identified. How to address that impact is FFS. Following options are discussed: - Option 1: Set BW of CSI-RS to 24 PRBs for 60 kHz SCS in FR1 - Option 2: Exclude 60 kHz SCS in FR1 - Option 3: other options are not precluded. Impact on BW used in BFD Impact on bandwidth of CSI-RS for BFD with 60 kHz SCS in FR1 is identified. How to address that impact is FFS. Following options are discussed: - Option 1: Set BW of CSI-RS to 24 PRBs for 60 kHz SCS in FR1 - Option 2: Exclude 60 kHz SCS in FR1 - Option 3: other options are not precluded.

- This paper discusses the issues listed above and provide our views.

2	Discussions

Impact on BW used in RLM

The reason this topic was brought up was that the maximum bandwidth supported by a RedCap UE in FR1 is 20 MHz after initial access, and 200 MHz in FR2 after initial access. However, the bandwidth of CSI-RS for RLM is 48 RBs, which means for 60 kHz SCS, the bandwidth of the CSI-RS is 48 RBs * 12 SCS/RB * 60 kHz = 34.56 MHz, which exceeds 20 MHz. This means that the RedCap UE will not be able to use this CSI-RS as a RLM resource.

Observation 1: The RedCap UEs cannot use CSI-RS with 60 kHz SCS as a RLM-RS.

Looking at the current specification and the situation, we think that the most effective way is to remove 60 kHz CSI-RS from supported RLM-RS by RedCap UEs. The fact that the RedCap UEs cannot monitor these RSs anyways is the main reason. Looking at the other option, which is to set the BW of CSI-RS to 24 RBs when SCS = 60 kHz, we think that this is not necessary. First of all the network can configure suitable RLM-RS, and in this case, SSB can be configured as the RLM-RS. Reducing the bandwidth of CSI-RS would also reduce the accuracy of RLM, and might affect accuracy requirements. Thus, this option is not desirable.

Exclude 60 kHz SCS in FR1 for CSI-RS as the RLM-RS.

Impact on BW used in BFD

Based on the same reasons and concerns, we support here the option to exclude 60 kHz.

Exclude 60 kHz SCS in FR1 for CSI-RS as the BFD-RS.

3	Conclusion

Observation 1: The RedCap UEs cannot use CSI-RS with 60 kHz SCS as a RLM-RS.

- Proposal 1: Exclude 60 kHz SCS in FR1 for CSI-RS as the RLM-RS.

Proposal 2: Exclude 60 kHz SCS in FR1 for CSI-RS as the BFD-RS.

References

- R4-2115358