3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118938

Electronic Meeting, November 01-12, 2021

Source:	ZTE Corporation

Title:	Impact on existing UE positioning and RRM requirements

Agenda item:	8.21.2.4

Document for:	Discussion

# 1	Introduction

In RAN4 100-e, there were discussions on latency reduction for positioning enhancement. Some of the WF [1] are copied below.

Issue 3-2-1: SRS antenna port switching has impact on positioning measurement and its accuracy FFS: the impact of SRS antenna port switching impact on UE Rx-Tx measurement - FFS: solution depends on the impact Issue 3-2-3: Avoidance of SRS antenna port switching impact on positioning measurement and its accuracy for gNB Rx-Tx and UL RTOA measurement accuracy FFS: the impact of SRS antenna port switching impact on gNB Rx-Tx measurement and ULRTOA - FFS: solution depends on the impact

- This paper discusses the issues listed above and also provide our view on the other related issues.

2	Discussions

Issue 3-2-1: SRS antenna port switching has impact on positioning measurement and its accuracy

As discussed during the last meeting, R15 SRS as well as R16 positioning-dedicated SRS can both be used as the SRS for UE Rx-Tx and gNB Rx-Tx measurements.

- Observation 1: R15 SRS as well as R16 positioning-dedicated SRS can both be used as the SRS for UE Rx-Tx and gNB Rx-Tx measurements.

Then, R15 SRS might be switched to another antenna port and thus, positioning accuracy might be affected. In general, we can agree on the statement that if SRS antenna port switching happens during UE/gNB Rx-Tx or UL RTOA, there can be impact on positioning measurement and its accuracy.

If SRS antenna port switching happens during UE/gNB Rx-Tx or UL RTOA, there can be impact on positioning measurement and its accuracy.

Issue 3-2-3: Avoidance of SRS antenna port switching impact on positioning measurement and its accuracy for gNB Rx-Tx and UL RTOA measurement accuracy

Regarding whether SRS AP switching can be avoided, we think there is no need to specify whether this can be avoided by the network or not in specifications. This issue is dependent on the previous issue, and corresponding requirements / UE behaviors can be defined. If under some certain cases, such SRS AP switching can be avoided, then it will be done up to the network. If under some cases it cannot be avoided, then the corresponding requirements apply.

There is no need to specify whether this can be avoided by the network or not in specifications.

3	Conclusion

- Observation 1: R15 SRS as well as R16 positioning-dedicated SRS can both be used as the SRS for UE Rx-Tx and gNB Rx-Tx measurements.

Proposal 1: If SRS antenna port switching happens during UE/gNB Rx-Tx or UL RTOA, there can be impact on positioning measurement and its accuracy.

Proposal 2: There is no need to specify whether this can be avoided by the network or not in specifications.

References

- R4-2115365