3GPP TSG-RAN WG4 Meeting #101-e	R4-2119016

# Electronic Meeting, 1st - 12th Nov, 2021

Title: 	Simulation results on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.8.3.2

Document for:	Information

Background

During RAN4#100-e meeting, the way forward [1] for NR UE HST FR1 demodulation requirements was approved.

• Since huge performance gap for HST-SFN is still observed, companies are encouraged to check the summary on simulation results (R4-2113455) and come back in the next meeting if necessary

In this contribution, we provide our updated simulation results about the demodulation requirements for NR UE HST FR1 CA. The simulation assumption can be derived from [2] as below Table 1-1 shows.

Table 1-1 Simulation assumption for NR UE HST FR1 CA

| Parameter | Value |
| --- | --- |
| DMRS type | type 1 |
| Number of DMRS symbols | DMRS 1+1+1 |
| TDD pattern | DDDSU, S: 10D+2G+2U for 15 kHz SCS 7DS2U, S: 6D+4G+4U for 30 kHz SCS |
| MCS | For HST-SFN JT: MCS 13 based on 64QAM table For DPS: MCS 17 based on 64QAM table |
| TRS periodicity | 10 ms, 2 slot pattern |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Ds and Dmin | Ds = 700m, Dmin = 150m |
| Rank | Rank = 2 |
| Testing metric | SNR @ 70% of maximum throughput |

Simulation results

HST-SFN

FDD 15 kHz

Table 2.1.1-1 Ideal simulation results on FDD 15 kHz under HST-SFN channel model

| Case | CBW(MHz)/SCS(KHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- |
| 1 | 5 | 2x2 | 9.00 |
| 2 | 5 | 2x4 | 6.33 |
| 3 | 10 | 2x2 | 9.35 |
| 4 | 10 | 2x4 | 6.69 |
| 5 | 15 | 2x2 | 10.11 |
| 6 | 15 | 2x4 | 7.37 |
| 7 | 20 | 2x2 | 10.65 |
| 8 | 20 | 2x4 | 7.89 |
| 9 | 25 | 2x2 | 11.03 |
| 10 | 25 | 2x4 | 8.13 |
| 11 | 30 | 2x2 | 11.52 |
| 12 | 30 | 2x4 | 8.79 |
| 13 | 40 | 2x2 | 11.44 |
| 14 | 40 | 2x4 | 8.70 |
| 15 | 50 | 2x2 | 11.52 |
| 16 | 50 | 2x4 | 8.81 |

Figure 2.1.1-1 Ideal simulation results on FDD 15 kHz under HST-SFN channel model

TDD 30 kHz

Table 2.1.2-1 Ideal simulation results on TDD 30 kHz under HST-SFN channel model

| Case | CBW(MHz)/SCS(KHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- |
| 17 | 5 | 2x2 | 9.16 |
| 18 | 5 | 2x4 | 6.55 |
| 19 | 10 | 2x2 | 9.35 |
| 20 | 10 | 2x4 | 6.68 |
| 21 | 15 | 2x2 | 9.86 |
| 22 | 15 | 2x4 | 7.09 |
| 23 | 20 | 2x2 | 9.76 |
| 24 | 20 | 2x4 | 6.96 |
| 25 | 25 | 2x2 | 10.15 |
| 26 | 25 | 2x4 | 7.41 |
| 27 | 30 | 2x2 | 10.66 |
| 28 | 30 | 2x4 | 7.84 |
| 29 | 40 | 2x2 | 11.06 |
| 30 | 40 | 2x4 | 8.11 |
| 31 | 50 | 2x2 | 11.33 |
| 32 | 50 | 2x4 | 8.43 |
| 33 | 60 | 2x2 | 11.64 |
| 34 | 60 | 2x4 | 8.80 |
| 35 | 80 | 2x2 | 11.65 |
| 36 | 80 | 2x4 | 8.89 |
| 37 | 90 | 2x2 | 11.86 |
| 38 | 90 | 2x4 | 9.03 |
| 39 | 100 | 2x2 | 11.65 |
| 40 | 100 | 2x4 | 8.85 |

Figure 2.1.2-1 Ideal simulation results on TDD 30 kHz under HST-SFN channel model

HST-DPS

FDD 15 kHz

Table 2.2.1-1 Ideal simulation results on FDD 15 kHz under HST-DPS channel model

| Case | CBW(MHz)/SCS(KHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- |
| 41 | 5 | 2x2 | 9.77 |
| 42 | 5 | 2x4 | 7.06 |
| 43 | 10 | 2x2 | 9.85 |
| 44 | 10 | 2x4 | 7.04 |
| 45 | 15 | 2x2 | 9.82 |
| 46 | 15 | 2x4 | 7.12 |
| 47 | 20 | 2x2 | 9.50 |
| 48 | 20 | 2x4 | 6.61 |
| 49 | 25 | 2x2 | 9.65 |
| 50 | 25 | 2x4 | 7.09 |
| 51 | 30 | 2x2 | 9.96 |
| 52 | 30 | 2x4 | 7.35 |
| 53 | 40 | 2x2 | 9.54 |
| 54 | 40 | 2x4 | 6.69 |
| 55 | 50 | 2x2 | 9.67 |
| 56 | 50 | 2x4 | 7.21 |

Figure 2.2.1-1 Ideal simulation results on FDD 15 kHz under HST-DPS channel model

TDD 30 kHz

Table 2.2.2-1 Ideal simulation results on TDD 30 kHz under HST-DPS channel model

| Case | CBW(MHz)/SCS(KHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- |
| 57 | 5 | 2x2 | 10.45 |
| 58 | 5 | 2x4 | 7.49 |
| 59 | 10 | 2x2 | 9.80 |
| 60 | 10 | 2x4 | 7.09 |
| 61 | 15 | 2x2 | 9.59 |
| 62 | 15 | 2x4 | 6.64 |
| 63 | 20 | 2x2 | 9.55 |
| 64 | 20 | 2x4 | 6.63 |
| 65 | 25 | 2x2 | 9.56 |
| 66 | 25 | 2x4 | 6.67 |
| 67 | 30 | 2x2 | 9.50 |
| 68 | 30 | 2x4 | 6.63 |
| 69 | 40 | 2x2 | 9.45 |
| 70 | 40 | 2x4 | 6.51 |
| 71 | 50 | 2x2 | 9.53 |
| 72 | 50 | 2x4 | 6.86 |
| 73 | 60 | 2x2 | 9.50 |
| 74 | 60 | 2x4 | 6.73 |
| 75 | 80 | 2x2 | 9.45 |
| 76 | 80 | 2x4 | 6.52 |
| 77 | 90 | 2x2 | 9.54 |
| 78 | 90 | 2x4 | 6.98 |
| 79 | 100 | 2x2 | 9.47 |
| 80 | 100 | 2x4 | 6.58 |

Figure 2.1.2-1 Ideal simulation results on TDD 30 kHz under HST-DPS channel model

Conclusion

In this contribution, we provide our updated simulation results about NR UE HST FR1 CA demodulation performance under propagation condition of HST-SFN and HST-DPS.

Reference

R4-2115724, WF on FR1 HST demodulation, RAN4#100-e, CMCC

R4-2106098, WF on UE demodulation for FR1 HST, RAN4#98-bis-e, CMCC