3GPP TSG-RAN WG4 Meeting #101-e	R4-2119022

# Electronic Meeting, 1st - 12th Nov, 2021

Title: 	Discussion on NR FR2 HST deployment Scenario-B

Source: 	Huawei, HiSilicon

Agenda item:	8.9.2.2

Document for:	Discussion

Background

During RAN4#100-e meeting, WF [1] on remaining issues on FR2 HST deployment scenario and channel modeming was approved. In this contribution, we share our views about NR FR2 HST deployment Scenario-B as per agreements reached in RAN4#100-e.

- Introducing performance requirements for both uni-directional and bi-directional deployment in scenario B which pending on further discussion on following aspect: - The test applicable rules can be further discussed and introduced if needed - FFS whether single test case cover both uni-directional and bi-directional deployment - BS declaration for applicable test cases can be further discussed - Test feasibility for bi-directional deployment under performance test cases - Performance comparision among uni-directional and bi-directional deployment

Discussion

Bi-directional

There are four schemes for Bi-directional deployment are for further analysis.

a) Scheme-1								b) Scheme-2

c) Scheme-3

Figure 2.1-1 Candidate schemes for Bi-directional deployment

For Scheme-1, 1 beam per BS panel and 6 beam per UE panel is selected. The BS panel boresight is pointed to the railway at the distance of Ds, the beam is pointed to the railway at the distance of Ds. When the UE is at the distance of Ds, the UE panel boresight is point to BS panel boresight rightly. The link budget analysis is shown as Figure 2.1-2 below.

Figure 2.1-2 Link budget for Scheme-1

For Scheme-2, 1 beam for one BS panel and 2 beams for another BS panel and 6 beam per UE panel is selected. The BS panel boresight is pointed to the railway at the distance of Ds, the beam is pointed to 0 and 10 degrees. When the UE is at the distance of Ds, the UE panel boresight is point to BS panel boresight rightly. The link budget analysis is shown as Figure 2.1-3 below.

Figure 2.1-3 Link budget for Scheme-2

For Scheme-3, 2 beam per BS panel and 6 beam per UE panel is selected. The BS panel boresight is pointed to the railway at the distance of Ds, the beam is pointed to 0 and 10 degrees. When the UE is at the distance of Ds, the UE panel boresight is point to BS panel boresight rightly. The link budget analysis is shown as Figure 2.1-4 below.

Figure 2.1-4 Link budget for Scheme-3

The link budget remaining and the minimum beam dwelling time for three schemes are shown as Table 2.1-1 below.

Table 2.1-1 Link budget remaining and minimum beam dwelling time

|  | link budget remaining[dB] | Minimum beam dwelling time[s] | Beam switching point[m] |
| --- | --- | --- | --- |
| Scheme-1 | 14.3 | 3.60 | [0,350] |
| Scheme-2 | 17.7 | 2.06 | [150, 350, 500] |
| Scheme-3 | 19.8 | 1.54 | [0, 200, 350, 500] |

For Bi-directional Scenario-B deployment for FR2 HST, Scheme-3 provide the largest link budget remaining while it requires more switching which leads to smaller minimum beam dwelling time; link budget remaining may be un-sufficient for Scheme-1 considering the uplink.

Uni-directional

For Uni-directional deployment, 2 beam per BS panel and 6 beam per UE panel is selected. The BS panel boresight is pointed to the railway at the distance of Ds, the beam is pointed to 0 and 10 degrees. When the UE is at the distance of Ds, the UE panel boresight is point to BS panel boresight rightly. The link budget analysis is shown as Figure 2.2-1 below.

Figure 2.2-1 Link budget for Uni-directional deployment

The link budget remaining and the minimum beam dwelling time for Uni-directional deployment is shown as Table 2.2-1 below.

Table 2.2-1 Link budget remaining and minimum beam dwelling time

| link budget remaining[dB] | minimum beam dwelling time[s] | Beam switching point[m] |
| --- | --- | --- |
| 15.8 | 2.57 | [200, 450] |

For Uni-directional Scenario-B deployment for FR2 HST, it provides lower link budget remaining while it requires only two switching which leads to larger minimum beam dwelling time.

Proposals

In this contribution, we discuss on NR FR2 HST deployment Scenario-A. Our observations and proposals are:

- For Bi-directional Scenario-B deployment for FR2 HST, Scheme-3 provide the largest link budget remaining while it requires more switching which leads to smaller minimum beam dwelling time; link budget remaining may be un-sufficient for Scheme-1 considering the uplink.

For Uni-directional Scenario-B deployment for FR2 HST, it provides lower link budget remaining while it requires only two switching which leads to larger minimum beam dwelling time.

Reference

R4-2115725, WF on remaining issues on FR2 HST deployment scenario and channel modeling, RAN4#100-e, Samsung

R4-2108660, WF on FR2 HST Deployment Scenario Analysis, RAN4#99-e, Samsung

- Annex Simulation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Ds | 700m |
| Dmin | 150m |
| RRH Tx power | 47dBm |
| RRH height | 15m |
| RRH antenna array | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] |
| Path Loss | RMa LoS |
| UE antenna height | 5m |
| UE antenna array | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] |
| UE noise figure | 10dB |
| ILs | 13 dB |
| SNR | 18.6dB (i.e. FR2 Test 2-6, 64QAM CR=0.43 and Rank2 in TS 38.101-4) |
| Others | In WF [2] |