3GPP TSG-RAN WG4 Meeting #101e	 R4-2119027

# Electronic Meeting, Nov 1st – 12th, 2021

Title: 	Simulation results for PUSCH demodulation requirements for FR1 256QAM

Source: 	Huawei, HiSilicon

Agenda item:	8.12.3.1

Document for:	Discussion

Simulation assumptions

In RAN4 #99-e meeting, the WF [1] for PUSCH 256QAM was agreed. Following simulation results were agreed:

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz; 30kHz: 10MHz, 40MHz, 100MHz |
| SRS, PT-RS | Not configured |
| Phase noise modelling | No explicit PN modelling (Note 1) |
| Tx EVM | No explicit 3.5% Tx EVM modelling (Note 2) |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| Note 1: The phase noise impact can be included in the impairment results, but it is left up to companies Note 2: Consider 3.5% Tx EVM impact in the impaired results, but it is left up to companies. |  |

In this paper, we share our views on those left open issues.

Simulation results

1T2R

| PUSCH mapping type | Propagation condition | CBW/SCS | MCS | Ideal Results (dB) | Impairment Results (dB) |
| --- | --- | --- | --- | --- | --- |
| PUSCH mapping type A | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 18.34 |  |
|  |  | 15kHz, 10MHz | MCS20 | 19.09 |  |
|  |  | 30kHz, 10MHz | MCS20 | 18.59 |  |
|  |  | 30kHz, 40MHz | MCS20 | 18.51 |  |
|  |  | 30kHz, 100MHz | MCS 20 | 18.40 |  |
| PUSCH mapping type B | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 18.75 |  |
|  |  | 15kHz, 10MHz | MCS20 | 18.61 |  |
|  |  | 30kHz, 10MHz | MCS20 | 18.41 |  |
|  |  | 30kHz, 40MHz | MCS20 | 18.40 |  |
|  |  | 30kHz, 100MHz | MCS 20 | 18.40 |  |

1T4R

| PUSCH mapping type | Propagation condition | CBW/SCS | MCS | Ideal Results (dB) | Impairment Results (dB) |
| --- | --- | --- | --- | --- | --- |
| PUSCH mapping type A | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 13.80 |  |
|  |  | 15kHz, 10MHz | MCS20 | 13.47 |  |
|  |  | 30kHz, 10MHz | MCS20 | 13.80 |  |
|  |  | 30kHz, 40MHz | MCS20 | 13.47 |  |
|  |  | 30kHz, 100MHz | MCS20 | 13.43 |  |
| PUSCH mapping type B | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 13.80 |  |
|  |  | 15kHz, 10MHz | MCS20 | 13.42 |  |
|  |  | 30kHz, 10MHz | MCS20 | 13.66 |  |
|  |  | 30kHz, 40MHz | MCS20 | 13.40 |  |
|  |  | 30kHz, 100MHz | MCS20 | 13.43 |  |

1T8R

| PUSCH mapping type | Propagation condition | CBW/SCS | MCS | Ideal Results (dB) | Impairment Results (dB) |
| --- | --- | --- | --- | --- | --- |
| PUSCH mapping type A | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 10.81 |  |
|  |  | 15kHz, 10MHz | MCS20 | 10.63 |  |
|  |  | 30kHz, 10MHz | MCS20 | 10.80 |  |
|  |  | 30kHz, 40MHz | MCS20 | 10.42 |  |
|  |  | 30kHz, 100MHz | MCS20 | 11.18 |  |
| PUSCH mapping type B | TDLA30-10 Low | 15kHz, 5MHz | MCS20 | 11.07 |  |
|  |  | 15kHz, 10MHz | MCS20 | 10.50 |  |
|  |  | 30kHz, 10MHz | MCS20 | 10.80 |  |
|  |  | 30kHz, 40MHz | MCS20 | 10.40 |  |
|  |  | 30kHz, 100MHz | MCS20 | 11.18 |  |

Summary

In this paper, we provide our simulation results for PUSCH performance requirements for FR1 UL 256QAM as per the approved simulation assumptions [1].

Reference

[1] R4-2115748, Way forward for FR1 PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#100-e, August, 2021

[2] R4-2108667, Way forward for FR1 PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#99-e, May, 2021