3GPP TSG-RAN WG4 Meeting # 101-e	R4-2119047

# Electronic Meeting, Nov. 1-12, 2021

Title: 	Simulation results on intra cell inter-user MMSE-IRC receiver

Source: 	Huawei, HiSilicon

Agenda item:	8.12.2.2

Document for:	Discussion

Background

In RAN4 #99-e meeting, WF R4-2108666 [1] was approved and corresponding initial simulation assumptions are determined. In this paper, we provide our initial simulation results.

Simulation results

The simulation assumptions are derived from WF [1] and for power allocation and SNR definition, we used the following assumptions:

- Power allocation:  Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- SNR definition: SNR = (STargetUE+ SInterfUE)/N

Rank 1+1 with same CDM group

Simulation results for case with rank 1(Target UE) + 1(interference UE) with same CDM group are shown in Figure 2-1.

| - MCS4,TDLA30-10,2T2R | - MCS4, TDLA30-10,2T4R |
| --- | --- |
| - MCS4,TDLC300-100,2T2R | - MCS4,TDLC300-100,2T4R |
| - MCS13,TDLA30-10,2T2R | - MCS13, TDLA30-10,2T4R |
| - MCS13,TDLC300-100,2T2R | - MCS13,TDLC300-100,2T4R |

Figure 2-1: Simulation results for case with rank 1(Target UE) + 1(interference UE) with same CDM group

Rank 1+1 with different CDM group

Simulation results for case with rank 1(Target UE) + 1(interference UE) with different CDM group are shown in Figure 2-2

| - MCS4,TDLA30-10,2T2R | - MCS4, TDLA30-10,2T4R |
| --- | --- |
| - MCS4,TDLC300-100,2T2R | - MCS4,TDLC300-100,2T4R |
| - MCS13,TDLA30-10,2T2R | - MCS13, TDLA30-10,2T4R |
| - MCS13,TDLC300-100,2T2R | - MCS13,TDLC300-100,2T4R |

Figure 2-2: Simulation results for case with rank 1(Target UE) + 1(interference UE) with different CDM group

The summary of simulation results for rank 1+1 is captured in Table 2-2.

Table 2-1: Summary of simulation results for case with Rank 1+1

| Propagation conditions |  | TDLA30-10 |  |  |  |  |  |  |  | TDLC300-100 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS |  | 4 |  |  |  | 13 |  |  |  | 4 |  |  |  | 13 |  |  |  |
| Precoding Matrix selection |  | Op1 |  | Op2 |  | Op1 |  | Op2 |  | Op1 |  | Op2 |  | Op1 |  | Op2 |  |
| Antenna configuration |  | 2x2 | 2x4 | 2x2 | 2x4 | 2x2 | 2x4 | 2x2 | 2x4 | 2x2 | 2x4 | 2x2 | 2x4 | 2x2 | 2x4 | 2x2 | 2x4 |
| Same CDM group (SNR@70% of max TP) | MRC | 1.7 | -2.6 | 2.3 | -2.2 | Inf | Inf | Inf | Inf | 2.2 | -2.1 | 2.7 | -1.6 | Inf | Inf | Inf | Inf |
|  | MMSE-IRC | 1.2 | -2.4 | 1.7 | -1.9 | 10.6 | 5.6 | 11.6 | 6.4 | 1.6 | -1.9 | 2.2 | -1.5 | 12.2 | 6.3 | 13.5 | 7.4 |
| Different CDM group (SNR@70% of max TP) | MRC | 1.4 | -3.1 | 1.9 | -2.5 | Inf | Inf | Inf | Inf | 1.8 | -2.6 | 2.4 | -2.0 | Inf | Inf | Inf | Inf |
|  | MMSE-IRC | 0.8 | -3.2 | 1.3 | -2.7 | 10.6 | 5.2 | 11.6 | 6.1 | 1.2 | -2.6 | 1.7 | -2.2 | 11.9 | 6.0 | 13.3 | 6.9 |

We have following observations for case with rank 1+1 according to the simulation results:

Observation 1: There is no obvious much gain for MMSE-IRC receiver over MMSE-MRC receiver for MCS4, but much gain can be achieved for MCS 13.

Observation 2: The performance for case with different CDM groups is better than that with same CDM groups.

Rank 2+1 with different CDM group

Simulation results for case with rank 2(Target UE) + 1(interference UE) with different CDM group are shown in Figure 2-3

| - MCS13,TDLA30-10 | - MCS13,TDLC300-100 |
| --- | --- |
| - MCS19,TDLA30-10 | - MCS19,TDLC300-100 |

Figure 2-3: Simulation results for case with rank 2(Target UE) + 1(interference UE) with different CDM group

Summary of simulation results for rank 2+1 is captured in Table 2-3:

Table 2-3: Summary of simulation results for case with Rank 2+1

| Propagation conditions |  | TDLA30-10 |  |  |  | TDLC300-100 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS |  | 13 |  | 19 |  | 13 |  | 19 |  |
| Precoding Matrix selection |  | Opt1 | Opt2 | Opt1 | Opt2 | Opt1 | Opt2 | Opt1 | Opt2 |
| Receiver Type (SNR@70% of max TP) | MRC | Inf | Inf | Inf | Inf | Inf | Inf | Inf | Inf |
|  | MMSE-IRC | 8.5 | 10.2 | 13.7 | 16.5 | 9.5 | 11.6 | 16.4 | 20.9 |

According to the simulation results, we have following observations for rank 2+1:

Observation 3: The performance gain is always obvious for MMSE-IRC receiver over MMSE-MRC receiver.

Rank 2+2 with different CDM group

Simulation results for case with rank 2(Target UE) + 2(interference UE) with different CDM group are shown in Figure 2-4

| - MCS13,TDLA30-10 | - MCS13,TDLC300-100 |
| --- | --- |
| - MCS19,TDLA30-10 | - MCS19,TDLC300-100 |

Figure 2-4: Simulation results for case with rank 2(Target UE) + 2(interference UE) with different CDM group

Summary of simulation results for rank 2+2 is captured in Table 2-4:

Table 2-4: Summary of simulation results for case with Rank 2+2

| Propagation conditions |  | TDLA30-10 |  |  |  | TDLC300-100 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS |  | 13 |  | 19 |  | 13 |  | 19 |  |
| Precoding Matrix selection |  | Opt1 | Opt2 | Opt1 | Opt2 | Opt1 | Opt2 | Opt1 | Opt2 |
| Receiver Type (SNR@70% of max TP) | MRC | Inf | Inf | Inf | Inf | Inf | Inf | Inf | Inf |
|  | MMSE-IRC | 12.2 | 15.8 | 18.6 | 22.9 | 14.5 | 21.1 | Inf | Inf |

According to the simulation results, we have following observations for rank 2+2:

Observation 4: MMSE-IRC can’t work for case with MCS 19 under TDLC300-100.

Observation 5: MMSE-IRC always has obvious performance gain over MMSE-MRC.

Conclusion

In this paper, we provide our simulation results for intra cell inter-user MMSE-IRC receiver. The observations are:

- For rank 1+1：

Observation 1: There is no obvious much gain for MMSE-IRC receiver over MMSE-MRC receiver for MCS4, but much gain can be achieved for MCS 13.

Observation 2: The performance for case with different CDM groups is better than that with same CDM groups.

- For rank 2+1:

Observation 3: The performance gain is always obvious for MMSE-IRC receiver over MMSE-MRC receiver.

- For rank 2+2:

Observation 4: MMSE-IRC can’t work for case with MCS 19 under TDLC300-100.

Observation 5: MMSE-IRC always has obvious performance gain over MMSE-MRC.

Reference

- R4-2108666, Way Forward on MMSE-IRC for intra-cell inter-user interference. Huawei, HiSilicon