3GPP TSG-RAN WG4 Meeting #101-e		R4-2119077

Electronic meeting, Nov 1 – 12, 2021

Agenda item:	8.4.5.1

Source:	ZTE Corporation

Title:	Discussion on new FR2 CA BW class

Document for:	Approval

1	Introduction

In previous RAN4 meetings, how to extend FR2 CA BW class to support 1600MHz has been intensively discussed and a WF on new CA BW class was approved in [1] in last RAN4 meeting. The agreements are as follows.

- New FR2 fallback group FBG3+2 is defined to allow mix of FBG3 and FBG2

- This new FBG3+2 contains 4 new CA BW classes as MA, MD, ME, and MF

- New FBG3+2 CA BW classes as MA, MD, ME and MF are captured into Table 5.3A.4-1 and applicable CA configurations using FBG3+2 are captured into Clause 5.5A.1.

  - Alternative  to Table 5.3A.4-1 is to have a new one, see below

Table 5.3A.4-x: Intra-band contiguous CA bandwidth classes for mixing FBGs

| Intra-band contiguous CA bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | FBG3 | FBG2 |
| MA | 8 | 1 |
| MD | 8 | 2 |
| ME | 8 | 3 |
| MF | 8 | 4 |

- Fallback behaviour

  - Update NOTE 2 of “Table 5.3A.4-1: CA bandwidth classes” as below

    - ALT 1: NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. The UE shall be configured with a carrier from FBG2 only when it is already configured with the highest supported order CA bandwidth class from FBG3. The aggregated channel bandwidth shall be not larger than 1600 MHz

    - ALT2: NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. The UE shall not be configured with a combination that simultaneously consists of a lower-order CA bandwidth class from each fallback group. The aggregated channel bandwidth shall be not larger than 1600 MHz

  - Above note means that no FBG3 carriers are released when there are still FBG2 CC(s) i.e. valid fallback sequence is 8x100+4x200   8x100+3x200 8x100+2x200 8x100+1x200 8x100  7/6/… x100

  - Further study the fallback behavior applicability to signaling.

In this paper, we further provide our opinion on the new CA BW class for FR2.

- Discussion

In [2], it is agreed that a new fallback group “3+2” for FR2 CA BW class is introduced to support channel BW of 1600MHz. Two alternatives are supposed to define the new CA BW classes. One alternative (1) is to reuse and extend the new BW classes into Table 5.3A.4-1. The other alternative (2) is to define a new table parallel to Table 5.3A.4-1 as follows.

Table 5.3A.4-x: Intra-band contiguous CA bandwidth classes for mixing FBGs

| Intra-band contiguous CA bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | FBG3 | FBG2 |
| MA | 8 | 1 |
| MD | 8 | 2 |
| ME | 8 | 3 |
| MF | 8 | 4 |

However for the alternative (2) in the above Table 5.3A.4-x, the column of “Aggregated channel bandwidth” is missing in the table. If we add the column of “Aggregated channel bandwidth” into Table 5.3A.4-x, there is no difference between the template of Table 5.3A.4-x and Table 5.3A.4-1. So from our perspective of view, we prefer the template in Table 5.3A.4-1to reuse and extend the new BW classes.

Regarding to the fallback group, two options are supposed to update the Note 2 in Table 5.3A.4-1. The notes in Alt 1 is more directly than Alt 2. We think Alt 1 is more suitable

    - ALT 1: NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. The UE shall be configured with a carrier from FBG2 only when it is already configured with the highest supported order CA bandwidth class from FBG3. The aggregated channel bandwidth shall be not larger than 1600 MHz

    - ALT2: NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. The UE shall not be configured with a combination that simultaneously consists of a lower-order CA bandwidth class from each fallback group. The aggregated channel bandwidth shall be not larger than 1600 MHz

The new CA bandwidth class for FR2 is suggested to be as Fig 1.

Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| MA |  | 800 MHz < BWChannel_CA ≤ 1000 MHz |  | 8 + 1 |  | 3 + 2 |  |
| MD |  | 1000 MHz < BWChannel_CA ≤ 1200 MHz |  | 8 + 2 |  |  |  |
| ME |  | 1200 MHz < BWChannel_CA ≤ 1400 MHz |  | 8 + 3 |  |  |  |
| MP |  | 1400 MHz < BWChannel_CA ≤ 1600 MHz |  | 8 + 4 |  |  |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A. NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. For fallback group “3+2”, the UE shall be configured with a carrier from FBG2 only when it is already configured with the highest supported order CA bandwidth class from FBG3. The aggregated channel bandwidth shall be not larger than 1600MHz. NOTE 3: For fallback group “3+2”, it means that no FBG3 carriers are released when there are still FBG2 CC(s) i.e. valid fallback sequence is 8x100+4x200 8 x100+3x200 8x100+2x200 8x100+1x200 8x100 7/6/ x100. |  |  |  |

Fig 1  Updated CA bandwidth classes

3	Conclusions

In this paper we discussed the issues of new CA bandwidth class for FR2. Based on the discussion, we have the following proposal.

Proposal 1:	 In order to extend FR2 CA BW class to support maximum channel BW of 1600MHz, the CA bandwidth classes table in Table 5.3A.4-1 for fallback group “3+2” is suggested.

Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| R | 800 MHz < BWChannel_CA ≤ 1000 MHz | 5 |  |
| S | 1000 MHz < BWChannel_CA ≤ 1200 MHz | 6 |  |
| T | 1200 MHz < BWChannel_CA ≤ 1400 MHz | 7 |  |
| U | 1400 MHz < BWChannel_CA ≤ 1600 MHz | 8 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| MA |  | 800 MHz < BWChannel_CA ≤ 1000 MHz |  | 8 + 1 |  | 3 + 2 |  |
| MD |  | 1000 MHz < BWChannel_CA ≤ 1200 MHz |  | 8 + 2 |  |  |  |
| ME |  | 1200 MHz < BWChannel_CA ≤ 1400 MHz |  | 8 + 3 |  |  |  |
| MP |  | 1400 MHz < BWChannel_CA ≤ 1600 MHz |  | 8 + 4 |  |  |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively except for CA bandwidth class A. NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. For fallback group “3+2”, the UE shall be configured with a carrier from FBG2 only when it is already configured with the highest supported order CA bandwidth class from FBG3. The aggregated channel bandwidth shall be not larger than 1600MHz. NOTE 3: For fallback group “3+2”, it means that no FBG3 carriers are released when there are still FBG2 CC(s) i.e. valid fallback sequence is 8x100+4x200 8 x100+3x200 8x100+2x200 8x100+1x200 8x100 7/6/ x100. |  |  |  |

4	References

R4-2114963, WF on FR2 new CA BW class, Nokia