3GPP TSG-RAN WG4 #101-e	                                                        R4-2119105

e-Meeting, November 1 – 12, 2021

Agenda item:			8.16.3.2

Source:	Qualcomm Inc

Title:	60 GHz UE RX and General requirements

Document for:	Approval

# Introduction

In this paper we discuss changes to 38.101-2 UE RX and general requirements. General requirements are combined with the UE RX requirements to limit the number of submissions per meeting improvement guidance.

# Spec structure for 60 GHz

We propose the general approach of combining FR2-1 and FR2-2 requirements in common tables. There may be exceptions where we can use separate tables for FR2-1 and FR2-2. Examples are included in the text below.

Proposal: Typically, combine FR2-1 and FR2-2 requirements in common tables. In some cases, they can be separated for FR2-1 and FR2-2.

# General clauses

In the general requirements sections, we propose to do the following:

- Introduce the agreed FR2-1 and FR2-2 definition

- Define the agreed unlicensed band as n263, the next unused band number in FR2

- Introduce n263 as a contiguous CA band. This is in preparation for defining requirements.

- Provide tables for TBD values of maximum transmission bandwidth and minimum guard band. Values will come from discussion in RAN4.

- Channel bandwidth table for FR2-2 including the agreed values.

The proposed changes are shown for convenience here. We have also submitted a draft CR with these proposals.

**** for convenience change in draft CR shown here 38.101-2 v 17.3.0 band definition*****************

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2-1 | 24250 MHz – 52600 MHz |
| FR2-2 | 52600 – 71000 MHz |

The present specification covers FR2 operating bands.

## 5.2	Operating bands

NR is designed to operate in the FR2 operating bands defined in Table 5.2-1.

Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n259 | 39500 MHz | – | 43500 MHz | 39500 MHz | – | 43500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| n262 | 47200 MHz | – | 48200 MHz | 47200 MHz | – | 48200 MHz | TDD |
| n2631 |  |  |  |  |  |  | TDD |
| Editor’s Note 1: Copy the reference similar to NRU saying this is for unlicensed channel access |  |  |  |  |  |  |  |

## 5.2A	Operating bands for CA

### 5.2A.1	Intra-band CA

NR intra-band contiguous and non-contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR2.

Table 5.2A.1-1: Intra-band contiguous and non-contiguous CA operating bands in FR2-1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257 | n257 |
| CA_n258 | n258 |
| CA_n259 | n259 |
| CA_n260 | n260 |
| CA_n261 | n261 |

Table 5.2A.1-2: Intra-band contiguous CA operating bands in FR2-2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n263 | n263 |

************* end change ****************

************ for convenience change in draft CR shown here 38.101-2 v 17.3.0 ***************

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Tables 5.3.2-1 and 5.3.2-2.

Table 5.3.2-1: Maximum FR2-1 transmission bandwidth configuration NRB

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

Table 5.3.2-2: Maximum FR2-2 transmission bandwidth configuration NRB

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB |
| 120 | TBD | TBD | TBD | n/a | n/a | n/a |
| 480 | n/a | n/a | TBD | TBD | TBD | n/a |
| 960 | n/a | n/a | TBD | TBD | TBD | TBD |

Editor’s note: If SU ends up being the same for FR2-2 we can consider one single table.

### 5.3.3	Minimum guardband and transmission bandwidth configuration

The minimum guardband for each UE channel bandwidth and SCS is specified in Tables 5.3.3-1and  5.3.3-2

Table 5.3.3-1: Minimum FR2-1 guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N. A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Table 5.3.3-1a: Minimum FR2-2 guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB |
| 120 | TBD | TBD | TBD | n/a | n/a | n/a |
| 480 | n/a | n/a | TBD | TBD | TBD | n/a |
| 960 | n/a | n/a | TBD | TBD | TBD | TBD |

Editor’s note: If MGB ends up being the same for FR2-2 we can consider one single table.

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Tables 5.3.2-1 and 5.3.2-1a.

The minimum guardband of receiving BS SCS 240 kHz SS/PBCH block for each UE channel bandwidth is specified in tables 5.3.3-2 and receiving TBD 5.3.3-.

Table: 5.3.3-2: Minimum FR2-1 guardband (kHz) of SCS 240 kHz SS/PBCH block

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- |
| 240 | 3800 | 7720 | 15560 |

Table: 5.3.3-3: Minimum FR2-2 guardband (kHz) of SCS 240 kHz SS/PBCH block

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB |

Editor’s note: TBD what we need in this table. Requires discussion.

NOTE:	The minimum guardband in Table 5.3.3-2 is applicable only when the SCS 240 kHz SS/PBCH block is received adjacent to the edge of the UE channel bandwidth within which the SS/PBCH block is located.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-2 UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB, the minimum guardband on each side of the carrier is the guardband applied at the configured channel bandwidth for the numerology that is transmitted immediately adjacent to the guard band.

If multiple numerologies are multiplexed in the same symbol and the UE channel bandwidth is > 200 MHz, the minimum guardband applied adjacent to 60 kHz SCS shall be the same as the minimum guardband defined for 120 kHz SCS for the same UE channel bandwidth.

Figure 5.3.3-3 Guard band definition when transmitting multiple numerologies

Note:	Figure 5.3.3-3 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

### 5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to "Reference point A" in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth [13] and will fulfil the minimum UE guardband requirement specified in Clause 5.3.3.

### 5.3.5	Channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: Channel bandwidths for each NR band

| Operating band | SCS (kHz) | UE channel bandwidth (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 |  | 100 |  | 200 |  | 4001 |  | 800 |  | 1200 |  | 1600 |  | 2000 |  |
| n257 |  | 60 |  | 50 |  | 100 |  | 200 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  | 50 |  | 100 |  | 200 |  | 400 |  |  |  |  |  |  |  |  |  |
| n258 |  | 60 |  | 50 |  | 100 |  | 200 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  | 50 |  | 100 |  | 200 |  | 400 |  |  |  |  |  |  |  |  |  |
| n259 |  | 60 |  | 50 |  | 100 |  | 200 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  | 50 |  | 100 |  | 200 |  | 400 |  |  |  |  |  |  |  |  |  |
| n260 |  | 60 |  | 50 |  | 100 |  | 200 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  | 50 |  | 100 |  | 200 |  | 400 |  |  |  |  |  |  |  |  |  |
| n261 |  | 60 |  | 50 |  | 100 |  | 200 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  | 50 |  | 100 |  | 200 |  | 400 |  |  |  |  |  |  |  |  |  |
| n262 |  | 60 |  | 50 |  | 100 |  | 200 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  | 50 |  | 100 |  | 200 |  | 400 |  |  |  |  |  |  |  |  |  |
| n2632 | 120 |  | 100 | 200 | 400 |  |  |  |  |
|  | 480 |  |  |  | 400 | 800 | 1200 | 1600 |  |
|  | 960 |  |  |  | 400 | 800 | 1200 | 1600 | 2000 |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. EDITOR’S NOTE 2: RAN4 needs to decide which FR2-2 bandwidths are mandatory. Once this is agreed that should be reflected in a note for this table. |  |  |  |  |  |  |  |  |  |

************* end change **********

# Receive requirements

Here we discuss REFSENS and EIS requirements

# Power class 1 Reference sensitivity

Our proposal continues to be based on rel. 15 assumptions for REFSENS. Note that REFSENS estimate is agnostic of misalignment between UE and TE axes. For reference, REFSENS requirement for 400MHz CC and -1 dB target SNR for n262 is -83.5 dBm (47.2-48.2 GHz).  For FR2-2 the array size is 16 elements. Our preliminary budget for FR2-2 is below.

Power class 1 REFSENS budget

| Parameter | Worst Case Value | units | Notes |
| --- | --- | --- | --- |
| # of element per polarization | 64 |  |  |
| Element NF | 14 | dB | PVT corners |
| Gain per element | 4 | dBi |  |
| # of chains | 2 |  | Arranged to receive orthogonal polarizations, MRC |
| Implementation and packaging losses worst case | 9.1 | dB | Rx specific |
| REFSENS | -87.1 | dBm | SNR Target = -1dB, 400MHz channel |

****** for convenience change in draft CR shown here 38.101-2 v 17.3

#### 7.3.2.1	Reference sensitivity power level for power class 1

For FR2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.1-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.1-1: FR2-1 Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5 | -94.5 | -91.5 | -88.5 |
| n258 | -97.5 | -94.5 | -91.5 | -88.5 |
| n260 | -94.5 | -91.5 | -88.5 | -85.5 |
| n261 | -97.5 | -94.5 | -91.5 | -88.5 |
| n262 | -92.5 | -89.5 | -86.5 | -83.5 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

The FR2-1 REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Table 7.3.2.1-2: FR2-1 Uplink configuration for reference sensitivity

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n262 | 32 | 64 | 128 | 256 | 120 kHz | TDD |

For FR2-2, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes TBD with peak reference sensitivity specified in Table 7.3.2.1-3. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.1-3: FR2-2 Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 | 200 | 400 | 800 | 1200 | 1600 | 2000 |
| n263 | [-93.1] | [-90.1] | [-87.1] | [-84.1] | [-82.3] | [-81.1] | [-80.1] |

The FR2-2  REFSENS requirement in Table 7.3.2.1-3 shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-4

Table 7.3.2.1-4: FR2-2 Uplink configuration for reference sensitivity

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 | 200 | 400 | 800 | 1200 | 1600 | 2000 | SCS | Duplex Mode |
| n263 | TBD | TBD | TBD |  |  |  |  | 120 kHz | TDD |
|  |  |  | TBD | TBD | TBD | TBD |  | 480 kHz |  |
|  |  |  | TBD | TBD | TBD | TBD | TBD | 960 kHz |  |

Editor’s note. Table values may be affected by the spectral utilization discussion. TBD for now.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

Editor’s note. If RAN4 concludes that FR2-2 reference measurement channel and max RB config are the same as FR2-1, then it may be possible to combine FR2-1 and FR2-2 in single tables.

Table 7.3.2.1-3: Reserved

| Operating band | Network Signalling value |
| --- | --- |

************ end change **********

Our proposal continues to be based on rel. 15 assumptions for REFSENS. Note that REFSENS estimate is agnostic of misalignment between UE and TE axes. For reference, REFSENS requirement for 400MHz CC and -1 dB target SNR for n262 is -83.5 dBm (47.2-48.2 GHz).  For FR2-2 the array size is 16 elements. Our preliminary budget for FR2-2 is below.

Table TBD FR2-2 Power class 3 REFSENS budget

| Parameter | Worst Case Value | units | Notes |
| --- | --- | --- | --- |
| Signal bandwidth | 380 | MHz | 400 MHz channel assuming 95% SU (SU is FFS) |
| # of elements per polarization | 16 |  |  |
| Element NF | 14 | dB | PVT corners |
| Gain per element | 4 | dBi |  |
| # of chains | 2 |  | Arranged to receive orthogonal polarizations, MRC |
| Implementation and packaging losses | 10.6 | dB | Rx specific |
| REFSENS | -79.6 | dBm | SNR Target = -1dB, 400MHz channel |

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0s 38.101-2 v 17.3.0

#### 7.3.2.3	Reference sensitivity power level for power class 3

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.3-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

For the UEs that support multiple FR2 bands, the minimum requirement for Reference sensitivity in Table 7.3.2.3-1 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∆MBP,n as specified in clause 6.2.1.3.  The requirement for the UE which supports a single FR2 band is specified in Table 7.3.2.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 7.3.2.3-1 and Table 6.2.1.3-4.

Table 7.3.2.3-1: Reference sensitivity for power class 3

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1200 MHz | 1600 MHz | 2000 MHz |
| n257 | -88.3 | -85.3 | -82.3 | -79.3 | n.a. | n.a. | n.a. | n.a. |
| n258 | -88.3 | -85.3 | -82.3 | -79.3 | n.a. | n.a. | n.a. | n.a. |
| n259 | -84.7 | -81.7 | -78.7 | -75.7 | n.a. | n.a. | n.a. | n.a. |
| n260 | -85.7 | -82.7 | -79.7 | -76.7 | n.a. | n.a. | n.a. | n.a. |
| n261 | -88.3 | -85.3 | -82.3 | -79.3 | n.a. | n.a. | n.a. | n.a. |
| n262 | -82.8 | -79.8 | -76.8 | -73.8 | n.a. | n.a. | n.a. | n.a. |
| n263 | [-85.6] | [-82.6] | [-79.6] | [-76.6] | [-74.6] | [-73.6] | [-72.6] | [-85.6] |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

********** end change ***********

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0

## A.2.3	Reference measurement channels for TDD

For UL RMCs defined below, TDD slot pattern defined in Table A.2.3-1 will be used for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, TDD slot patterns defined for reference sensitivity tests in Table A.3.3.1-1 will be used.

Table A.2.3-1: Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) | SCS 480 kHz (µ=TBD) | SCS 960 kHz (µ=TBD) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSUUUU | 7DS8U |  |  |
| Special Slot Configuration (Note 2) |  | S=4D+6G+4U | S=12D+2G |  |  |
| referenceSubcarrierSpacing |  | 60 kHz | 120 kHz |  |  |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 2 ms | 2 ms |  |  |
|  | nrofDownlinkSlots | 3 | 7 |  |  |
|  | nrofDownlinkSymbols | 4 | 12 |  |  |
|  | nrofUplinkSlot | 4 | 8 |  |  |
|  | nrofUplinkSymbols | 4 | 0 |  |  |
| Indexes of active UL slots |  | mod(slot index, 40) = {36,…,39} | mod(slot index, 80) = {72,…,79} |  |  |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |  |  |  |

### A.2.3.1	DFT-s-OFDM Pi/2-BPSK

********** end change ***********

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0

### A.2.3.2	DFT-s-OFDM QPSK

Table A.2.3.2-1: Reference Channels for DFT-s-OFDM QPSK

| Parameter | Allocated resource blocks (LCRB) | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 1 | 11 | QPSK | 2 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 16 | 11 | QPSK | 2 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 20 | 11 | QPSK | 2 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 32 | 11 | QPSK | 2 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 64 | 11 | QPSK | 2 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 128 | 11 | QPSK | 2 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 256 | 11 | QPSK | 2 | 12808 | 24 | 2 | 4 | 67584 | 33792 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. DM-RS symbols are not counted. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in 38.214. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 4: Indexes of active UL slots are given by Table A.2.3-1 with TDD UL-DL configuration specified in A2.3 for the requirements requiring at least one sub frame (1ms) for the measurement period. For other requirements, indexes of active UL slots are given by the slots satisfying mod(slot index+1, 5) = 0 with TDD UL-DL configuration specified in A.3.3.1. NOTE 5: The RMCs apply to all channel bandwidth where LCRB ≤ NRB. |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-2: Void

### A.2.3.3	DFT-s-OFDM 16QAM

********** end change A.2.3.2 ***********

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0 A.3.1

# A.3	DL reference measurement channels

## A.3.1	General

Unless otherwise stated, Tables A.3.3.2-1 and A.3.3.2-2 are applicable for measurements of the Receiver Characteristics (clause 7).

Unless otherwise stated, Tables A.3.3.2-1 and A.3.3.2-2 also apply for the modulated interferer used in Clauses 7.5 and 7.6 with test specific bandwidths.

CSI-RS configuration parameter defined in Table A.3.1-2 and Table A.3.1-3 are used for verifying the beam correspondence requirement, 2 slots of CSI-RS shall be provided at each test grid point. The DL channel shall be configured for zero power on all tones except those used by CSI-RS in slots containing CSI-RS for beam refinement, and the DL and UL channel sizes shall be the same during verification.

Table A.3.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| CORESET frequency domain allocation |  |  | Full BW |
| CORESET time domain allocation |  |  | 2 OFDM symbols at the begin of each slot |
| PDSCH mapping type |  |  | Type A |
| PDSCH start symbol index (S) |  |  | 2 |
| Number of consecutive PDSCH symbols (L) |  |  | 12 |
| PDSCH PRB bundling |  | PRBs | 2 |
| Dynamic PRB bundling |  |  | false |
| MCS table for TBS determination |  |  | 64QAM |
| Overhead value for TBS determination |  |  | 0 |
| First DMRS position for Type A PDSCH mapping |  |  | 2 |
| DMRS type |  |  | Type 1 |
| Number of additional DMRS |  |  | 2 |
| FDM between DMRS and PDSCH |  |  | Disable |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2 |
|  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 12 for CSI-RS resource 2 |
|  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity | Slots | 60 kHz SCS: 80 for CSI-RS resources 1 and 2 120 kHz SCS: 160 for CSI-RS resources 1 and 2 480 kHz SCS: 640 for CSI-RS resources 1 and 2 960 kHz SCS: 1280 for CSI-RS resources 1 and 2 |
|  | CSI-RS offset | Slots | 60 kHz SCS: 40 for CSI-RS resources 1 and 2 120kHz SCS: 80 for CSI-RS resources 1 and 2 480 kHz SCS: 320 for CSI-RS resources 1 and 2 960 kHz SCS: 640 for CSI-RS resources 1 and 2 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  | TCI state #0 |
| PTRS configuration |  |  | PTRS is not configured |

Table A.3.1-2: CSI-RS parameters for beam correspondence based on SSB and CSI-RS

*** end change A.3.1

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0 A.3.3.1

## A.3.3	DL reference measurement channels for TDD

#### A.3.3.1 General

Table A.3.3.1-1. Additional test parameters for TDD

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SCS 60 kHz (µ=2) | SCS 120 kHz (µ=3) | SCS 480 kHz (µ=TBD) | SCS 960 kHz (µ=TBD) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSU | DDDSU |  |  |
| Special Slot Configuration (Note 2) |  | S=4D+6G+4U | S=10D+2G+2U |  |  |
| referenceSubcarrierSpacing |  | 60 kHz | 120 kHz | 480 kHz | 960 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 1.25 ms | 0.625 ms |  |  |
|  | nrofDownlinkSlots | 3 | 3 |  |  |
|  | nrofDownlinkSymbols | 4 | 10 |  |  |
|  | nrofUplinkSlot | 1 | 1 |  |  |
|  | nrofUplinkSymbols | 4 | 2 |  |  |
| Number of HARQ Processes |  | 8 | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,39} | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,79} |  |  |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. NOTE 3: i is the slot index per frame. |  |  |  |  |  |

#### A.3.3.2	FRC for receiver requirements for QPSK

*** end A3.3.1

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0 A.3.3.2

#### A.3.3.2	FRC for receiver requirements for QPSK

Table A.3.3.2-1 Fixed Reference Channel for Receiver Requirements (SCS 60 kHz, TDD)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 23 | 23 | 23 |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | Bits | 4224 | 8456 | 16896 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | CBs | 1 | 2 | 2 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | Bits | 10.138 | 20.294 | 40.550 |
| Max. Throughput averaged over 1 frame | Mbps | 9.715 | 19.449 | 38.861 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {3,4,5,6,7} for i from {0,…,79} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {0,1,2} for i from {0,…,79} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |

Table A.3.3.2-2 Fixed Reference Channel for Receiver Requirements (SCS 120 kHz, TDD)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 47 | 47 | 47 | 47 |
| MCS index |  | 4 | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | Bits | 2088 | 4224 | 8456 | 16896 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | CBs | 1 | 1 | 2 | 2 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | Bits | 6912 | 14256 | 28512 | 57024 |
| Max. Throughput averaged over 1 frame | Mbps | 10.022 | 20.275 | 40.589 | 81.101 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {7,…,15} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {0,…,6} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |  |

Table A.3.3.2-3 Fixed Reference Channel for Receiver Requirements (SCS 480 kHz, TDD)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 400 | 800 | 1200 | 1600 |
| Subcarrier spacing configuration |  |  |  |  |  |
| Allocated resource blocks |  |  |  |  |  |
| Subcarriers per resource block |  |  |  |  |  |
| Allocated slots per Frame |  |  |  |  |  |
| MCS index |  |  |  |  |  |
| Modulation |  |  |  |  |  |
| Target Coding Rate |  |  |  |  |  |
| Maximum number of HARQ transmissions |  |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |
| Transport block CRC | Bits |  |  |  |  |
| LDPC base graph |  |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| Editor’s note: slot indices are TBD | CBs |  |  |  |  |
| Editor’s note: slot indices are TBD | CBs |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |
| Max. Throughput averaged over 1 frame | Mbps |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {7,…,15} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {0,…,6} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |  |

Table A.3.3.2-4 Fixed Reference Channel for Receiver Requirements (SCS 960 kHz, TDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 400 | 800 | 1200 | 1600 | 2000 |
| Subcarrier spacing configuration |  |  |  |  |  |  |
| Allocated resource blocks |  |  |  |  |  |  |
| Subcarriers per resource block |  |  |  |  |  |  |
| Allocated slots per Frame |  |  |  |  |  |  |
| MCS index |  |  |  |  |  |  |
| Modulation |  |  |  |  |  |  |
| Target Coding Rate |  |  |  |  |  |  |
| Maximum number of HARQ transmissions |  |  |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |  |
| Transport block CRC | Bits |  |  |  |  |  |
| LDPC base graph |  |  |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| Editor’s note: slot indices are TBD | CBs |  |  |  |  |  |
| Editor’s note: slot indices are TBD | CBs |  |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |  |
| Editor’s note: slot indices are TBD | Bits |  |  |  |  |  |
| Max. Throughput averaged over 1 frame | Mbps |  |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {7,…,15} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {0,…,6} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. |  |  |  |  |  |  |

#### A.3.3.3	FRC for receiver requirements for 16QAM

********** end change A.3.3.2 ***********

# EIS spherical coverage

For EIS we have added the n263 band to the power class 3 table. Certainly a similar approach should be taken for the other power classes.

It is possible to work out expected spherical coverage performance in FR2-2 based on expected performance in FR2-1 bands. The primary contributor to EIRP drop is the spherical coverage of the antenna gain itself. We therefore focus on the differences in normalized antenna gain between both n262 and n259 vs 60 GHz. n259. In both n259 and n262, the simulated antenna gain has a drop of about 6.5 dB from peak to 50th %ile.

The figure below shows a 2x8 array design simulation. Note the FR2-2 midband drop to 50%ile is 10 dB, significantly higher than FR2-1 band above.

Observation 5:  Peak to 50%ile gain drop is approximately 3.5 dB higher in FR2-2 than n262 due to antenna pattern and other considerations

Antenna gain is but one of the factors involved in establishing spherical coverage metrics. Other considerations that contribute to degradation of EIRP spherical coverage statistics are mutual coupling between PAs, Tx beam forming impairments and uncertainty related to power regulation mechanisms. FR2-2 is significantly high in frequency than n262 and n259, and we expect the degradation from ‘other considerations’ to have a larger effect.

To summarize, we expect more degradation in FR2-2 as compared to n259 and n262,  for both raw antenna spherical coverage performance as well as degradation in spherical coverage due to other considerations (are mutual coupling between PAs, Tx beam forming impairments and uncertainty related to power regulation mechanisms). Recall that n259 has 12.9 dB and n262 13.1 dB gain drop from peak to 50th %ile direction. Since we see n263 has 3.5 dB more drop than n262, we propose the drop for n263 EIS as 16.6 dB,

*** for convenience change in draft CR shown here 38.101-2 v 17.3.0 ****

#### 7.3.4.3	EIS spherical coverage for power class 3

The reference measurement channels and throughput criterion shall be as specified in clause 7.3.2.3

The maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-1 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

For the UEs that support multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 7.3.4.3-1 shall be increased per band, respectively, by the EIS spherical coveragerelaxation parameter ∆MBS,n as specified in clause 6.2.1.3.  The requirement for the UE which supports a single FR2 band is specified in Table 7.3.4.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 7.3.4.3-1 and Table 6.2.1.3-4.

Table 7.3.4.3-1: EIS spherical coverage for power class 3

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1200 MHz | 1600 MHz | 2000 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 | n.a. | n.a. | n.a. | n.a. |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 | n.a. | n.a. | n.a. | n.a. |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 | n.a. | n.a. | n.a. | n.a. |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 | n.a. | n.a. | n.a. | n.a. |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 | n.a. | n.a. | n.a. | n.a. |
| n262 | -69.7 | -66.7 | -63.7 | -60.7 | n.a. | n.a. | n.a. | n.a. |
| n263 | [-69] | [-66] | [-63] | [-60] | [-57] | [-55] | [-54] | [-53] |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

*** end change ****

# Conclusions

Proposal 1: Define n263 as unlicensed band as in Table 5.2-1

Proposal 2: Introduce n263 as a contiguous CA band

Proposal 3: Channel bandwidths as in Table 5.3.5-1

Proposal 4: n263 PC1 REFSENS as in Table 7.3.2.1-3

Proposal 5: n263 PC3 REFSENS as in Table 7.3.2.3-1

Proposal 6: n263 PC3 EIS as in Table 7.3.4.3-1