3GPP TSG-RAN WG4 Meeting #101-e	R4-2119116

Electronic Meeting, 1st - 12thNov, 2021

Source:	ZTE Corporation

Title:	Discussion on Inter-frequency measurements for NR HST FR1

Agenda Item:	8.8.2.2

Document for:	Approval

- Introduction

In the last meeting, a way forward on RRM NR FR1 HST was approved in [1]. For inter-frequency measurement, the following agreements approved.

Whether to define the enhancement for inter-frequency measurement in idle mode for HST Agreements: - Define the enhancement for inter-frequency measurement in IDLE mode for HST - Support of HST Idle mode inter-frequency measurement enhancements is an optional UE feature without capability signalling - UEs not supporting inter-frequency measurement enhancements shall meet the existing inter-frequency measurement requirements Measurement delay requirement for inter-frequency measurement with MG in HST in connected state for HST Agreement: DRX cycle; T SSB_measurement_period_intra for FR1 HST. No DRX; max(200ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(200ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; N4 x DRX cycle x CSSFinter N4 = 4 x M2. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.

In this document, we provide some analysis focus on some remaining open issues.

- Discussion

2.1 The enhancement for inter-frequency measurement in connected state for HST

For the issue of PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST, the following options were discussed during last meeting:

- Option 1 (QC, Nokia): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(6*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 6 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. - Option 2 (CATT, MTK, OPPO, vivo, HW): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 8 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.. - Option 3 (Ericsson, Nokia): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(N Note 4 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; N Note 4 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5.. - Option 4 (Apple): Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, Ceil(8*M2/1.5) * Max(MGRP, SMTC period)) * CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(8*M2 Note 3) * Max(MGRP, SMTC period, DRX cycle)) * CSSFinter. DRX cycle > 320ms; Ceil(8*M2/1.5) * DRX cycle * CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. - Option 5 (CMCC): Condition NOTE1,2; T PSS/SSS_sync_inter. No DRX; Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter. DRX cycle≤ 320ms; Max(600ms, Ceil(6 M Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Y Note 4 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1.5 if SMTC periodicity > 40 ms, otherwise M=1 NOTE 4: Y= 4 when SMTC <= 40ms, Y= 6 when SMTC > 40ms. - Option 6 (QC, Nokia, CMCC, Ericsson, Xiaomi): Condition NOTE1,2; TPSS/SSS_sync_inter for FR1 HST. No DRX; Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(6 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; 6 DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1.

We believe more stringent requirements should be used for HST FR1 so as to decrease the PSS/SSS detection delay, then the performance for CPE can be satisfied. However for non-HST case, the PSS/SSS detection delay of current Rel-16 should remain unchanged. So, we prefer Option 3.

Proposal 1: Option 3 is preferred.

2.2 The enhancement for inter-frequency measurement with MG in idle mode for HST

For the issue of how to perform the enhancement for inter-frequency measurement in idle mode for HST, the following options were proposed during last meeting:

- Option 1 (QC, CATT, CMCC, Ericsson): using the same requirement as for intra-frequency measurement for HST DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2); 0.32 x M3 (1 x M3); 0.96 x M4 (3 x M4). 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. - Option 2 (OPPO, HW, MTK, vivo, Xiaomi): The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). 0.32; 4.16 x M2 (13 x M2)Note 1; 0.64 x M3 (2 x M3)Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 7.68 (12); 1.28 (2); 1.92 (3). 1.28; 12.8(10); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1..

Between the two options, Option 1 is preferred by us, we believe re-use the same requirement as intra-frequency measurement is applicable for inter-frequency measurement in Idle mode.

Proposal 2: We prefer Option 1, reusing the same requirements as intra-frequency measurement in Idle mode.

- Conclusion

In this contribution, we have the following proposals for RX beam sweep number:

Proposal 1: Option 3 is preferred.

Proposal 2: We prefer Option 1, reusing the same requirements as intra-frequency measurement in Idle mode.

- Reference

- R4-2115439,“WF on RRM for FR1 HST”, RAN4#100 e-meeting, CMCC.