3GPP TSG-RAN WG4 Meeting # 100-e	R4-2119139

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	Further discussion on the BS RF requirements for RMR1900

Agenda Item:	7.3.4

Document for:	Discussion

# Introduction

Referring to the WF in [1], the following items were agreed for RMR 1900.

1. Way Forward 1 (Sub topic 1-1: EIRP requirements conversion to conducted requirements): For the next meeting, provided pros and cons analyses for the following options for the EIRP-based ECC requirements conversion to conducted requirements: - Manufacturer declaration based approach, - Fixed antenna gain approach. Other options are not precluded for the analysis next meeting

Way Forward 6 (Sub-topic 4-2: consideration of BS enhanced selectivity for band n1): RAN4 will not specify any requirement to capture the BS enhanced selectivity (band n1) assumption. A note should be added in TS 38.104 and/or in 1900MHz band TR to highlight this.

Way Forward 7 (Sub-topic 5-3: consideration of RMR co-location with MFCN BSs): In principle, RMR BSs are not expected to be co-located with MFCN BSs. Nevertheless, in a coordinated approach co-location is of interest and need to be considered in 3GPP spec.

On top of the above agreements, additional suggestions for further discussion topics were also listed in that WF in [1].

In this contribution, we provide further analysis for the BS RF requirements for RMR1900.

# Discussion

# EIRP requirements conversion to conducted requirements

During last meeting, a WF was agreed to provide pros and cons analyses for the following options for the EIRP-based ECC requirements conversion to conducted requirements:

- Manufacturer declaration based approach,

- Fixed antenna gain approach.

The above issue arises from the fact that the source ECC decision [7] has defined the requirements based on EIRP (i.e. antenna gain and internal (feeder, etc.) losses were accounted), while the RMR WID considers only non-AAS BS architecture, with the conducted requirements defined at the antenna port.

Below we provide the pros and cons analysis for both options:

Option 1: Manufacturer declaration based approach

As the RMR1900 operation is intended for Europe, the related Rel-17 RAN4 conformance testing specification will be transposed by the ETSI MSG TFES group into the updated version of so called ETSI Harmonized Standard. For NR BS, the RMR1900 is expected to be captured in future version of the ETSI EN 301 908-24 (NR BS) specification [2]. This specification is developed under work item in [3] by ETSI MSG TFES group, with the following co-signing companies:

Ericsson LM, Huawei Technologies Sweden AB, Nokia Germany, Qualcomm Wireless GmbH.

During past MSG TFES work, multiple ETSI EN specification reviews were organized by the EC in order to allow an external Consultant to provide an independent technical review. Such reviews were organized by the EC for ETSI specifications from various technical fields. Base Station specifications were also undergoing (multiple rounds) of such reviews. Some of the recent comments to the BS specification can be found in the following references, e.g.:

- NR UE: Review of the ETSI EN 301 908-13 V13.2.1 [4]

- AAS BS: EY Assessment of REN/MSG-TFES-13-23 under the RED (Second Assessment) [5]

- NR BS: EY Second Assessment of REN/MSG-TFES-13-24 under the RED [6]

In multiple cases, there were comments stating that manufacturer’s declarations are not allowed in Harmonized Standard (HS), or that the use of manufacturer declarations may influence the test result (which shall not be allowed). Even if some of the above referred comments could be resolved by appropriate clarifications in ETSI TFES, it is still encouraged to avoid as much as possible any unnecessary manufacturer declaration to be introduced into for the BS products, especially those intended for EU market. In multiple cases, those comments were marked as critical.

With the above feedback on the ETSI TFES discussions, interested companies are encouraged to double-check with their respective TFES delegates, in order to identify an agreeable compromise on the MR requirements definition.

Option 2: Fixed antenna gain approach

It shall be noted, that the concept of converting conducted-to-radiated (or radiated-to-conducted) requirements based on the fixed antenna gain, is not new. Examples of such approach can be found in, e.g.:

- TS 36.104 (EUTRA BS)

  - Home BS output power for adjacent E-UTRA channel protection

  - annex H.1 (Calculation of EIRP based on manufacturer declarations and site specific conditions)

- TS 37.105 (AAS BS)

  - Additional SEM and OBUE requirements: 470-790 MHz, antennal gain assumed: 17 dBi,

- TS 38.104 (NR BS)

  - Additional OBUE requirements: 470-790 MHz, antennal gain assumed: 17 dBi,

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: Manufacturer declaration based approach | Declaration based approach allows high degree of freedom in the NR BS product specification. | High risk of blocking the whole NR BS Harmonized Standard ETSI EN 301 908-24 for NR BS, due to EC comments on the manufacturer self-declared requirements. |
| Option 2: Fixed antenna gain approach | No risk of the EC blocking such specification. “Fixed antenna gain” approach was already used in the past in RAN4 in multiple BS specifications to define RF requirements conversion (conducted radiated). | May be seen as limiting deployments, or products specification flexibility. |

Based on the above analysis, the following is proposed:

Proposal 1: in order to avoid risk of the ETSI Harmonized Standard progress blocking, it is proposed to proceed with the Option 2, i.e. fixed antenna gain approach for the EIRP-to-conducted requirements conversion.

It shall be clarified, the for the purpose of the requirements derivation, internal antenna losses for the conducted/OTA conversion are assumed to be 0 dB. Such assumption shall be captured in the TR.

Proposal 2: It is assumed that the internal losses (feeder, etc.) are assumed to be 0 dB, for the purpose of requirements derivation.

For the RMR deployment flexibility, we may further look into the range of antenna gain values that are envisioned for RMR1900. Last meeting antenna gain values were proposed in [9]:

- For RMR1900: value of [18] dBi may be considered as the starting point

Proposal 3: based on Proposal 1, further discuss on the feasible antenna gain value for RMR 1900, with the [18] dBi being the starting point.

# BS rated output power

During last meeting, a WF in [1] suggested to continue the discussion on the BS rated output power, including consideration of the following items:

- ECC rated power limit translation (based on based on table 9 of ECC decision (20)02 for RMR1900),

- Manufacturer declarations of the rated output power (BS product capability).

Referring to the above table from ECC decision, one can notice that there are in-block requirements defined in terms on the maximum EIRP levels allowed for the uncoordinated deployments. One can envision that in case of coordinated deployments and the use of some interference mitigation techniques, one would be able to allow for higher EIRP values.

As already commented last meeting in [10], it is proposed to proceed with the following way to specify the maximum rated power limit for RMR BS, based on the above extracted ECC limit:

RMR1900: (65 - GantRMR1900) dBm / 10MHz

where GantRMR1900 is the antenna gain of the RMR BS.

Referring to the discussion on section 2.1, the GantRMR1900 could be either of the options:

- manufacturer declared value, or

- fixed value.

The above approach allows to proceed either Option 1, or Option 2.

Proposal 4: agree the following way to specify the maximum rated power limit for RMR BS:

RMR1900: (65 - GantRMR1900) dBm / 10MHz

where GantRMR1900 is the antenna gain of the RMR BS.

Referring to the discussion on section 2.1, the GantRMR1900 could be either of the options:

- manufacturer declared value, or

- fixed value.

# Additional spurious limit requirements

During last meeting, a WF in [1] suggested to continue the discussion on additional spur limits, including considerations of the following items:

- Potential need to consider RMR-specific requirements (based on table 10 of ECC decision (20)02 for RMR1900),

- Investigate the resulting/expected levels and their validity compared to the general spurious emission limits.

During last meeting, a WF in [1] suggested to continue the discussion on additional spur limits, including considerations of the following items:

- Potential need to consider RMR-specific requirements (based on table 10 of ECC decision (20)02 for RMR1900),

- Investigate the resulting/expected levels and their validity compared to the general spurious emission limits.

The above limit is to protect receiver of the Band n1, n65, n84 (which both are NR operating bands for Europe) from potential RMR1900 interference:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |

Referring to the table 10 in ECC 20(02), the emissions limit was specified as -49dBm/5MHz. Now referring to the specification of the additional spurious emission limits in 1920-1980 MHz range in the RAN4 spec, one can find the following:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65 |
|  | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.2.2. |
| E-UTRA Band 65 or NR Band n65 | 2110 – 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65. |
|  | 1920 – 2010 MHz | -49 dBm | 1 MHz | For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.6.5.2.2. This requirement does not apply to BS operating in band n65, since it is already covered by the requirement in clause 6.6.5.2.2. |
| NR Band n84 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1, since it is already covered by the requirement in clause 6.6.5.2.2. |

Based on the above ECC-limit vs. RAN4 limit comparison, the following is observed:

Observation: -49dBm limit seems to be reused by ECC from the already existing requirements for other bands, e.g. band n1.

Observation: as ECC limit was specified as “EIRP limit” and we need to define conducted requirement in RAN4, it requires further clarification whether there is need for the limit conversion at all, or ECC intention was to follow the already existing protection levels.

Observation: the measurement bandwidth in case of -49dBm limits in RAN4 spec is specified as 1MHz. It requires further clarification if ECC intention was to specify RMR limit with 5MHz measurement bandwidth, or the “-49dBm/5MHz” only refers to the channel bandwidth.

With the above observations, more discussion and clarifications are needed before concluding on this requirement.

# Work plan analysis

As per WID in [12], the internal TR was planned to be submitted for Information to December RAN meeting.

As per RAN4 arrangement, the TR/TS drafting requires to arrange the “work-split” for the TPs drafting in order to avoid redundant effort among companies. As there was no such work-split arranged, we would like to check with the Rapporteur on the preferred handling of the WI during the remaining meetings before March 2022 (3 more RAN4 meetings beyond this one).

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.xxx | Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR) | RAN#94 | RAN#95 | Core Part Rapporteur: Ingo Wendler (UIC) Ingo.wendler@sbb.ch; |

# Conclusion

Based on the above, the following proposals are formulated:

Proposal 1: in order to avoid risk of the ETSI Harmonized Standard progress blocking, it is proposed to proceed with the Option 2, i.e. fixed antenna gain approach for the EIRP-to-conducted requirements conversion.

Proposal 2: It is assumed that the internal losses (feeder, etc.) are assumed to be 0 dB, for the purpose of requirements derivation.

Proposal 3: based on Proposal 1, further discuss on the feasible antenna gain value for RMR 1900, with the [18] dBi being the starting point.

Proposal 4: agree the following way to specify the maximum rated power limit for RMR BS:

RMR1900: (65 - GantRMR1900) dBm / 10MHz

where GantRMR1900 is the antenna gain of the RMR BS.

# References

[1]	R4-2115637	WF on BS RF requirements for RMR900 and RMR1900, RAN4#100-e

[2]	ETSI EN 301 908-24

[3]	DEN/MSG-TFES-15-24 work item, MSG TFES, https://portal.etsi.org/webapp/WorkProgram/Report_WorkItem.asp?WKI_ID=54728

[4]	TFES(21)000066, MSGTFES_Resolution Report EN 301 908-13 V13.2.1 for HAS consultants (2nd around)

[5]	TFES(21)000060, EY Assessment of REN/MSG-TFES-13-23 under the RED (Second Assessment)

[6]		TFES(21)000061, EY Second Assessment of REN/MSG-TFES-13-24 under the RED

[7]	ECC 20(02)	Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), ECC

[9]	R4-2113752	RMR 1900 MHz - BS RF, RAN4#100-e

[10]	R4-2115790	Email discussion summary for [100-e][317] RAIL_900_1900MHz_BSRF

[12]	RP‑211496	Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR), WID